3GPP TSG-RAN WG4 Meeting #90	R4-1900601

Athens, GR, 25th  Feb – 1st March, 2019

Agenda item:	6.14.1.2

Source: 		NTT DOCOMO, INC.

Title: 	Views on high-speed train tests

Document for:	Discussion

1. Introduction

In the previous meetings, following agreements were reached regarding HST scenarios for NR performance test [1, 2]. In this contribution, we present our views on high-speed train scenarios.

RAN4 #AH1807

  - Low speed:

    - TDD 10Hz (3km/h)

    - [FDD 5Hz (3km/h)]

  - Medium speed: 100Hz (30km/h)

  - High Speed: 400Hz (120km/h)

  - High speed train scenarios:

    - FFS maximum Doppler shift

    - Need further discussion on the channel model

    - FFS SCS and CP length for high speed performance requirements

RAN4 #88

Agreement: Use HST model as described on section B3 TS36.101 for FR1 for initial simulation purposes (max Doppler shift specified in table)

  - FFS introduction of new requirements

  - This topic will be discussed in agenda for PDSCH demodulation and other related agendas in the next meeting.

RAN #81

The UE and BS demodulation performance requirements under HST scenarios are treated in TEI15 after December 2018.

2. Discussion

In Japan, high speed trains (Shinkansen) have been deployed since 1964. It spreads all across in Japan and is still being expanded in the areas, where it is currently not available. Shinkansen is used by more than 350 million people per year and very common for Japanese long-range transportation system. In addition, maglev is under construction targeting commercial launch in 2027, where the maximum operation speed can reach up to 500 km/h. This kind of high-speed transportation system is currently very common all around the world, e.g., Asian countries and Europe, and it is very important to make sure cellular coverage for the passengers in the high-speed trains, especially for FR1. Table I and Fig. 1 summarize HST tests and requirements in LTE, respectively. In Rel. 8, HST test was introduced. In the scenario, Doppler shift quickly varies when UE passes in front of TRPs that is indicated in blue crosses in the figure. In Rel. 14, additional HST test is introduced that assumes single frequency network deployment (HST-SFN). In this environments, UE is suffered by very wide range of Doppler shifts, in which positive and negative Doppler shifts are observed due to the TRPs at the direction of travel and TRPs on the other side, respectively. For NR, it is very important that we have HST tests in FR1 that has been supported in LTE. Otherwise, high speed support for Rel. 15 NR UE is degraded compared to LTE UEs NR can’t be fully utilized as a coverage support.

Proposal: For FR1, HST tests should be introduced that is at least equivalent to LTE.

Table I. High-speed tests for LTE

Figure 1: High-speed train scenario

- HST test

In LTE, HST test was initially introduced in Rel. 8. DL HST test can guarantee UE demodulation performance up to 750 Hz that was designed to support 300 km/h for the carrier frequency of 2.7 GHz. On the other hand, for the uplink, HST test can guarantee BS demodulation performance up to 1340 Hz that was designed to support 350 km/h for the carrier frequency of 2.1 GHz. In this sense, DL HST test is a bottleneck for HST scenario. Considering the practical deployment of high-speed trains in the world that operates with the maximum speed around or greater than 300 km/h, it is proposed that DL HST should be expanded to support 350 km/h. In addition, Doppler shift value increases according to the carrier frequency. For the operation in 3.6 GHz frequency band, it is proposed that we also have performance test assuming the mobility of 350 km/h that is equivalent to the Doppler shift value of 1167 Hz.

Proposal: Introduce HST test for Rel. 15 NR downlink with the same model as in LTE and following parameters.

| Parameter | Value |
| --- | --- |
| Ds | 300 m |
| Dmin | 2 m |
| v | 350 km/h |
| fd | 875 Hz for 15 kHz SCS test and 1167 Hz for 30 kHz SCS test |

- HST test for SFN scenario

For HST-SFN operation, gNB signalling should be introduced in order to switch channel estimation algorithm that is specific for HST-SFN scenario. Considering ASN. 1 impact, it is better to avoid introducing HST-SFN scenario at this moment.

Observation: Necessity of HST-SFN test can be studied in future releases.

3. Conclusion

In this contribution, we presented our views on channel models for NR performance test.

Proposal: For FR1, HST tests should be introduced that is at least equivalent to LTE.

Proposal: Introduce HST test for Rel. 15 NR downlink with the same model as in LTE and following parameters.

| Parameter | Value |
| --- | --- |
| Ds | 300 m |
| Dmin | 2 m |
| v | 350 km/h |
| fd | 875 Hz for 15 kHz SCS test and 1167 Hz for 30 kHz SCS test |

Observation: Necessity of HST-SFN test can be studied in future releases.

References

[1] 3GPP, “Draft meeting report for RAN4 #88,” Oct. 2018.

[2] 3GPP, “Draft meeting report for RAN4 #AH1808,” Aug. 2018.