3GPP TSG-RAN4 Meeting #90	R4-1900610

# Athens, Greece,  Feb. 25th – Mar. 1st 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | CRNum | rev | - | Current version: | V15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-2: UE maximum output power reduction for power class 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_Core |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PC1 max TRP is 35dBm, with assumed ACLR = 16dB worst case, with current PC1 reference waveform of full 128RB, there is 35-16 = 19dBm leakage at each adjacent channel. With 100MHz BW, then in adjacent channel the average PSD is 19dBm/100MHz = -1dBm/MHz, which is much higher than -13dBm/MHz SEM limit. The worst PSD can be even higher since leakage has roll off shape. Therefore, it is needed to change PC1 reference waveform to meet SEM requirement. Based on offline discussions, there could be a potential to change PC1 reference waveform to be the inner waveform defined by BW = 100MHz, SCS = 60KHz, DFT-s-OFDM QPSK, 45RB44 to meet SEM requirement and increase the efficiency of PC1 device. Based on this new proposed PC1 reference waveform, the PC1 MPR table is updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | FR2 PC1 reference waveform is changed to the waveform defined by BW = 100MHz, SCS = 60KHz, DFT-s-OFDM QPSK, 45RB44. Based on this new PC1 reference waveform, the PC1 MPR table is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC1 device may not meet SEM requirement and may have poor efficiency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

#### 6.2.2.1	UE maximum output power reduction for power class 1

For power class 1, MPR for contiguous allocations is defined as in Table 6.2.2.1-1.

Table 6.2.2.1-1 MPR for power class 1

| Modulation |  | MPR (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Inner RB allocations, ≤ 200M |  |  | Inner RB allocations, 400M | Outer RB allocations, 50M, 100M, 200M, 400M | Outer/Inner RB allocations, 50M, 100M, 200M, 400M |  |
|  |  | BWalloc,RB>10.8MHz NRB3≤RBstart≤2NRB3-LCRB |  | BWalloc,RB>10.8MHz RBstart>2NRB3-LCRB, RBstart < NRB3, | BWalloc,RB>10.8MHz | BWalloc,RB>10.8MHz | 1.44MHz < BWalloc,RB ≤10.8MHz | BWalloc,RB≤1.44MHz |
| DFT-s-OFDM PI/2 BPSK |  | 0.5 |  | 4.5 | 3 | 6 | 10 | 14.4 |
| DFT-s-OFDM QPSK |  | 0.5 |  | 4.5 | 3.5 | 7.5 | 10 | 14.4 |
| DFT-s-OFDM 16 QAM |  | 4.5 |  | 4.5 | 4.5 | 7.5 | 10 | 14.4 |
| DFT-s-OFDM 64 QAM |  | 6.5 |  | 6.5 | 6.5 | 7.5 | 10 | 14.4 |
| CP-OFDM QPSK |  | 5 |  | 5 | 5 | 8.5 | 10 | 14.4 |
| CP-OFDM 16 QAM |  | 5.5 |  | 5.5 | 6.5 | 8.5 | 10 | 14.4 |
| CP-OFDM 64 QAM |  | 8 |  | 8 | 9 | 9 | 10 | 14.4 |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High,

and

LCRB ≤ ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation.

The waveform defined by BW = 100 MHz, SCS = 60 kHz, DFT-S-OFDM QPSK, 45RB44 is the reference waveform with 0 dB MPR and is used for the power class definition.

UE requirements for the waveform defined by BW = 100 MHz, SCS = 60 kHz, DFT-S-OFDM pi/2 BPSK, 45RB44 shall be set to 0 dB MPR.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.4 apply.

< end of changes