3GPP TSG-RAN WG4 Meeting #90	R4-1900759

Athens, GR, 25 Feb - 1 Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | --- | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability rule for BS radidated demodulation test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) The applicability rule for BS radidated demodulation test was not defined. 2) There were some unnumbered hanging paragraphs in cluase 8.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Introduced the applicability rule for BS radidated demodulation test based on RAN4 agreements. 2) Added the subclause number and title “8.1.1 Scope and definitions” to avoid hanging paragraphs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No need to introduce applicability rule for BS radidated demodulation test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Start of first change >

8.1	General

### 8.1.1	Scope and definitions

Radiated performance requirements specify the ability of the BS type 1-O or BS type 2-O to correctly demodulate radiated signals in various conditions and configurations. Radiated performance requirements are specified at the RIB.

Radiated performance requirements for the BS are specified for the fixed reference channels and propagation conditions defined in TS 38.104 [2] annex A and annex F, respectively. The requirements only apply to those FRCs that are supported by the BS.

The radiated performance requirements for BS type 1-O and for the BS type 2-O are limited to two OTA demodulations branches as described in subclause 8.1.1. Conformance requirements can only be tested for 1 or 2 demodulation branches depending on the number of polarizations supported by the BS, with the required SNR applied separately per polarization.

NOTE 1: BS can support more than 2 demodulation branches, however OTA conformance testing can only be performed for 1 or 2 demodulation branches.

Unless stated otherwise, radiated performance requirements apply for a single carrier only. Radiated performance requirements for a BS supporting CA are defined in terms of single carrier requirements.

For BS type 1-O in FDD operation the requirements in clause 8 shall be met with the transmitter units associated with the RIB in the operating band turned ON.

NOTE 2:	BS type 1-O in normal operating conditions in FDD operation is configured to transmit and receive at the same time. The transmitter unit(s) associated with the RIB may be OFF for some of the tests.

In tests performed with signal generators a synchronization signal may be provided from the BS to the signal generator, to enable correct timing of the wanted signal.

Whenever the "RX antennas" term is used for the radiated performance requirements description, it shall refer to the demodulation branches (i.e. not physical antennas of the antenna array).

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S 		is the total signal energy in a slot on a RIB.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot.

8.1.2	OTA demodulation branches

Radiated performance requirements are only specified for up to 2 demodulation branches.

If the BS type 1-O, or the BS type 2-O uses polarization diversity and has the ability to maintain isolation between the signals for each of the demodulation branches, then radiated performance requirements can be tested for up to two demodulation branches (i.e. 1TX-1RX or 1TX-2RX test setups). When tested for two demodulation branches, each demodulation branch maps to one polarization.

If the BS type 1-O, or the BS type 2-O does not use polarization diversity then radiated performance requirements can only be tested for o a single demodulation branch (i.e. 1TX-1RX test setup).

### 8.1.3	Applicability rule

8.1.3.1 	Applicability of PUSCH performance requirements

8.1.3.1.1	Applicability of requirements for different subcarrier spacings

The tests for a specific subcarrier spacing are only applicable if the BS supports it.

For a BS supporting multiple subcarrier spacings, the tests for all the subcarrier spacings supported by the BS are applicable.

8.1.3.1.2	Applicability of requirements for different channel bandwidths

For each subcarrier spacing supported by the BS, the tests for a specific channel bandwidth are only applicable if the BS supports it.

For a BS supporting multiple channel bandwidths for one subcarrier spacing, only the tests for the highest channel bandwidth supported by the BS are applicable. If the performance requirements for the highest channel bandwidth is not defined in this document, the tests for the nearest lower channel bandwidth (i.e., test bandwidth) w.r.t. the highest channel bandwidth are appliabiliable, and the test bandwidth are placed in the middle of the channel bandwidth during the test.

8.1.3.1.3	Applicability of requirements for carrier aggregation

Editor’s note: The applicability of CA requirements will be added.

8.1.3.1.4	Applicability of requirements for different configurations

Editor’s note: For the requirements with different configurations on additional DMRS and mapping type, the applicability will be added.

8.1.3.2 	Applicability of PUCCH performance requirements

8.1.3.2.1	Applicability of requirements for different formats

The tests for a specific PUCCH format are only applicable if the BS supports it.

For a BS supporting multiple PUCCH formats, the tests for all the PUCCH formats supported by the BS are applicable.

8.1.3.2.2	Applicability of requirements for different subcarrier spacings

The tests for a specific subcarrier spacing are only applicable if the BS supports it.

For a BS supporting multiple subcarrier spacings, the tests for all the subcarrier spacings supported by the BS are applicable.

8.1.3.2.3	Applicability of requirements for different channel bandwidths

For each subcarrier spacing supported by the BS, the tests for a specific channel bandwidth are only applicable if the BS supports it.

For a BS supporting multiple channel bandwidths for one subcarrier spacing, only the tests for the highest channel bandwidth supported by the BS are applicable. If the performance requirements for the highest channel bandwidth is not defined in this document, the tests for the nearest lower channel bandwidth (i.e., test bandwidth) w.r.t. the highest channel bandwidth are appliabiliable, and the test bandwidth are placed in the middle of the channel bandwidth during the test.

8.1.3.2.4	Applicability of requirements for different configurations

Editor’s note: For the requirements with different configurations on additional DMRS, the applicability will be added.

8.1.3.3 	Applicability of PRACH performance requirements

8.1.3.3.1	Applicability of requirements for different formats

The tests for a specific PRACH format are only applicable if the BS supports it.

For a BS supporting multiple PRACH formats, the tests for all the PRACH formats supported by the BS are applicable.

8.1.3.3.2	Applicability of requirements for different subcarrier spacings

For PRACH with short sequence, the tests for a specfic subcarrier spacing are only applicable if the BS supports it.

For a BS supporting multiple subcarrier spacings for a PRACH format with short sequence, for each FR, the tests for [smallest] subcarrier spacing supported by the BS are applicable.

8.1.3.3.3	Applicability of requirements for different channel bandwidths

PRACH performance requirements are defined a channel bandwidth agnostic way, and the tests for any one channel bandwidth supported by the BS are applicable.

< End of first change >