3GPP TSG-RAN WG4 Meeting #90                                                                 R4-1900781

Athens, Greece, 25 Feb - 1 Mar 2019

Source:	Huawei, Hisilicon

Title:	TP to TR 37.716.21-21: Addition of CA configuration for DC_3A_n7A-n78A

Agenda item:	9.8.2

Document for:	Approval

Background

This contribution provides text proposal on the EN-DC configuration DC_3A_n7A-n78A as defined in Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) RP-182302 [1].

Text Proposal

---Start of changes---

6.x	DC_3A_n7A-n78A

6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n7A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A_n7A-n78A | DC_3A_n7A DC_3A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.x.3	Co-existence studies

Co-existence studies of Band 3 + Band n7 + Band n78 with UL Band 3 +Band n7 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	2nd order harmonic generated by uplink of Band 3 may also fall into own Rx of band n78.

-	4th order IMD generated by dual uplink of Band 3 + Band n7 may also fall into own Rx of band n7.

-	3rd order IMD generated by dual uplink of Band 3 + Band n7 may also fall into own Rx of band n78.

Co-existence studies of Band 3 + Band n7 + Band n78 with UL Band 3 +Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	2nd order harmonic generated by uplink of Band 3 may also fall into own Rx of band n78.

-	2nd , 4th , 5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band 3.

-	4th , 5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band n78. But there is no issue for TDD band.

6.x.4	∆TIB and ∆RIB values

For DC_3A_n7A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n7-n78 | 3 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n7-n78 | 3 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

6.x.5	MSD

2nd order harmonic problem generated by uplink of Band 3 already have been solved in DC_3A_n78A.

2nd , 4th , 5th order IMD generated by dual uplink of Band 3 + Band n78 already have been solved in DC_3A_n78A.

4th order IMD generated by dual uplink of Band 3 + Band n7 already have been solved in DC_3A_n7A.

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 product produced by Band 3 and n7 that impact the reference sensitivity of Band n78. Following DC_7A-n3A_n78A requirement in table 5.2-5 of TR 37.864-41-21 [3], reference sensitivity exception requirements are defined in Table 6.x.5-1. It is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n7A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A |  | N/A |
|  | n78 | 3390 | 10 | 52 | 3390 | 16.1 | TDD | IMD3 ¦2*fn7-*fB3¦ |

---End of changes---

Reference

[1]	RP-182302, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics

[2]	R4-1814769, “TR 37.716-21-21 v0.2.0 update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16, 37.716-21-21 v0.2.0”, LG Electronics

[3] TR 37.864-41-21, EN-DC of  LTE(xDL/1UL)+ NR(2DL/1UL) DC,  v15.0.0