3GPP TSG RAN WG4 Meeting #89	R4-1900809

Athens, GR, 25th  Feb – 1st March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [draft] Correction CR on UE measurement capability in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-2-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 specified that UE shall be capable of monitoring at least 24 SSB(s) with different SSB index and/or PCI on a single serving carrier which is PCC or PSCC or 1 SCC if PCC/PSCC is in a band different from SCC. However, when there are only SCCs in FR2 band, how UE selects one SCC to measure 24 SSB(s) is up to UE implementation according to the latest agreement. Even in such case, operators would configure RSRP measurement to 1 SCC in FR2 for mobility purpose and of course operators expect that such measurements are performed by using 24 SSB(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify how to select 1 SCC when there are only SCCs in FR2 band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may monitor 24 SSB(s) on 1 SCC which RSRP measurement is not configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---START OF CHANGE 1---

9.2.3	Number of cells and number of SSB

#### 9.2.3.1	Requirements for FR1

For each intra-frequency layer, the UE shall be capable of monitoring at least 8 cells.

For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of monitoring at least [14] SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell (except for the SCell) is no smaller than the number of configured RLM-RS SSB resources.

#### 9.2.3.2	Requirements for FR2

For each intra-frequency layer the UE shall be capable of monitoring at least 6 cells on a single serving carrier (PCC or PSCC or 1 SCC if PCC/PSCC is in a band different from SCC) out of all the serving carriers configured in the same band.

For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of monitoring at least 24 SSB(s) with different SSB index and/or PCI on a single serving carrier (PCC or PSCC or 1 SCC where RSRP measurement is configured if PCC/PSCC is in a band different from SCC) out of all the serving carriers configured in the same band. UE shall be capable of monitoring 2 SSB(s) on serving cell for each of the other serving carrier(s) in the same band. UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR on all above-mentioned SSBs

---END OF CHANGE 1-