3GPP TSG-RAN4 Meeting #90 	R4-1901102

# Athens, Greece, 25th Feb – 1st Mar

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 36.133 | CR | 6246 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on NB random access test cases R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13 |  |  |  |  |  | Date: |  |  | 2019-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Maintenance work is needed for the NB random access test cases for both normal and enhanced coverage. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Maintenance work, - Correct nprach-NumCBRA-StartSubacrriers parameters which are larger than nprach-NumSubacrriers. TS 36.321 specifies in 5.1.2 that the RA preamble for NB should be set to nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers + (ra-PreambleIndex modulo (nprach-NumSubcarriers - nprach-NumCBRA-StartSubcarriers)), where nprach-SubcarrierOffset, nprach-NumCBRA-StartSubcarriers and nprach-NumSubcarriers are parameters in the currently used PRACH resource. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases are not correct to implement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.2.16 and A.6.2.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change 1 >>

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage

A.6.2.16.1	Test Purpose and Environment

The purpose of this test is to verify whether the behavior of the random access procedure of a category NB1 UE in Normal Coverage is according to the requirements, whether the NPRACH power settings and timing are within specified limits, and whether the UE determines properly the enhanced coverage level based on the NRSRP measurement and the configured criterion in NRSRP-ThresholdsPrach [2]. This test will verify the requirements in Clause 6.6.2, Clause 6.6.3 and Clause 7.20.2 in an AWGN model.

For this test a single NB-IoT cell and a single LTE cell are used. The test parameters are given in tables A.6.2.16.1-1, A.6.2.16.1-2 and A.6.2.16.1-4.

Table A.6.2.16.1-1: nCell specific test parameters for HD-FDD contention based random access test for UE category NB1 In-Band mode in Normal Coverage

| Parameter | Unit |  | Value |  | Comments |
| --- | --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  |  | In-band |  |  |
| BWchannel |  | kHz |  | 200 |  |  |
| PRB location within eCell |  | - |  | 30 |  |  |
| NPDSCH parameters Note 2 |  |  |  | R.15 HD-FDD | As defined in A.3.1.5.1 |  |
| NPDCCH parameters Note 2 |  |  |  | R.27 HD-FDD | As defined in A.3.1.6.1 |  |
| NPBCH_RA |  | dB |  | -3 |  |  |
| NPBCH_RB |  | dB |  |  |  |  |
| NPSS_RA |  | dB |  |  |  |  |
| NSSS_RA |  | dB |  |  |  |  |
| NPDCCH_RA |  | dB |  |  |  |  |
| NPDCCH_RB |  | dB |  |  |  |  |
| NPDSCH_RA |  | dB |  |  |  |  |
| NPDSCH_RB |  | dB |  |  |  |  |
| NOCNG_RA Note 1 |  | dB |  |  |  |  |
| NOCNG_RB Note 1 |  | dB |  |  |  |  |
| DRX |  |  |  | OFF |  |  |
|  |  | dBm/15 kHz |  | Specified in Table A.6.2.16.1-2 |  |  |
|  |  | dB |  | 3 |  |  |
| Note 3 |  | dB |  | 3 |  |  |
| NRSRP Note 3 |  | dBm/15 kHz |  | -95 |  |  |
| Io Note 3 |  | dBm/180 kHz |  | -82.45 |  |  |
| Propagation Condition |  | - |  | AWGN |  |  |
| Antenna Configuration |  |  |  | 2x1 |  |  |
| Note 1: NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, NRSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |

Table A.6.2.16.1-2: eCell specific test parameters for HD-FDD contention based random access test for UE category NB1 In-Band mode in Normal Coverage

| Parameter | Unit | Value |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| BWchannel | MHz | 10 |
| NOCNG Pattern Note 1 | - | NOP.1 FDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RA Note 1 | dB |  |
| OCNG_RB Note 1 | dB |  |
|  | dBm/15 kHz | -98 |
|  | dB | 3 |
| Propagation Condition | - | AWGN |
| Antenna Configuration | - | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.6.2.16.1-3: Void

Table A.6.2.16.1-4: NPRACH-Configuration parameters for HD-FDD contention based random access test for UE category NB1 In-Band mode in Normal Coverage

| Field | Value |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH resource |  |  |  |  |  |  |
| RSRP-ThresholdsNPRACH-InfoList | {40, 55} |  |  |  | Corresponding to {-116, -101} dBm as defined in Section 9.1.22.9 |  |
| nprach-CP-Length | us66dot7 |  |  |  |  |  |
| nrs-Power | -5 dBm/15 kHz |  |  |  | As defined in clause 6.7.3 in TS 36.331. |  |
| Backoff Parameter Index | 1 |  |  |  | As defined in table 7.2-2 in TS 36.321 |  |
| Configured UE transmitted power () | 23 dBm for power class 3, 20 dBm for power class 5 |  |  |  | As defined in clause 6.2.5F in TS 36.101 |  |
| powerRampingStep | dB2 |  |  |  |  |  |
| preambleInitialReceivedTargetPower | dBm-112 |  |  |  |  |  |
| preambleTransMax-CE | n6 |  |  |  |  |  |
| Parameters per NPRACH Resource |  |  |  |  |  |  |
| NPRACH Resource | Level 0 | Level 1 | Level 2 |  |  |
| nprach-Periodicity | ms40 | ms240 | ms1280 |  |  |
| nprach-StartTime | ms8 | ms64 | ms512 |  |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart | zero | zero | zero |  |  |
| maxNumPreambleAttemptCE | n3 | n6 | n10 |  |  |
| numRepetitionsPerPreambleAttempt | n2 | n8 | n64 |  |  |
| npdcch-NumRepetitions-RA | r4 | r16 | r128 |  |  |
| npdcch-StartSF-CSS-RA | v1dot5 | v1dot5 | v1dot5 |  |  |
| npdcch-Offset-RA | zero | zero | Zero |  |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  |  |
| ra-ResponseWindowSize (per NPRACH Resource) | pp2 | pp2 | pp2 |  |  |
| mac-ContentionResolutionTimer (per NPRACH Resource) | pp2 | pp2 | pp2 |  |  |
| Note 1: See Clause 6.7. 3 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |  |  |

A.6.2.16.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.6.2.16.2.1	Random Access Response Reception

To test the UE behavior specified in Subclause 6.6.2, the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 3 preamble transmission attempts (the preamble may be transmitted multiple times in each attempt) have been received by the System Simulator. In response to the first 2 preamble transmission attempts, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.6.2. The power of the first preamble shall be -25 dBm with an accuracy specified in clause 6.3.5F.1.1 of TS 36.101 [5]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.5F.2 of TS 36.101 [5].

The transmit timing of all NPRACH transmissions shall be within the accuracy specified in Subclause 7.20.2.

A.6.2.16.2.2	No Random Access Response Reception

To test the UE behavior specified in subclause 6.6.2.2, the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 3 preamble transmission attempts have been received by the System Simulator. The System Simulator shall not respond to the first 2 preamble transmission attempts.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.6.2. The power of the first preamble shall be -25 dBm with an accuracy specified in clause 6.3.5F.1.1 of TS 36.101 [5]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.5F.2 of TS 36.101 [5].

The transmit timing of all NPRACH transmissions shall be within the accuracy specified in Subclause 7.20.2.

A.6.2.16.2.3	Receiving a NACK on msg3

To test the UE behavior specified in subclause 6.6.2.3, the System Simulator shall NACK all UE msg3 following a successful Random Access Response.

The UE shall re-transmit the msg3 upon the reception of a NACK on msg3 until the maximum number of re-transmissions defined by maxNumPreambleAttemptCE in the table A.6.2.16.1-4 is reached.

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI

To test the UE behavior specified in Subclause 6.6.2.4, the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI

To test the UE behavior specified in Subclause 6.6.2.4, the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element matching the CCCH SDU transmitted in the msg3 uplink message.

The UE shall send ACK if the Contention Resolution is successful.

A.6.2.16.2.6	Contention Resolution Timer expiry

To test the UE behavior specified in Subclause 6.6.2.5, the System Simulator shall not send a response to a msg3.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

A.6.2.16.2.7	NPRACH Resource Selection

The UE shall select NPRACH resources and transmits or re- transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 0. The rate of correct coverage enhancement level selection during repeated tests shall be at least 90%.

Note: Correct coverage enhancement level selection is a prerequisite for testing the other NPRACH requirements.

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage

A.6.2.17.1	Test Purpose and Environment

The purpose of this test is to verify whether the behavior of the random access procedure of a category NB1 UE in Enhanced Coverage is according to the requirements, whether the NPRACH power settings and timing are within specified limits, and whether the UE determines properly the enhanced coverage level based on the NRSRP measurement and the configured criterion in NRSRP-ThresholdsPrach [2]. This test will verify the requirements in Clause 6.6.2, Clause 6.6.3 and Clause 7.20.2 in an AWGN model.

For this test a single NB-IoT cell and a single LTE cell are used. The test parameters are given in tables A.6.2.17.1-1, A.6.2.17.1-2 and A.6.2.17.1-4.

Table A.6.2.17.1-1: nCell specific test parameters for HD-FDD contention based random access test for UE category NB1 In-Band mode in Enhanced Coverage

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| NB-IOT operational mode |  | In-band |  |  |
| BWchannel | kHz | 200 |  |  |
| PRB location within eCell | - | 30 |  |  |
| NPDSCH parameters Note 2 |  | R.15 HD-FDD | As defined in A.3.1.5.1 |  |
| NPDCCH parameters Note 2 |  | R.27 HD-FDD | As defined in A.3.1.6.1 |  |
| NPBCH_RA | dB | -3 |  |  |
| NPBCH_RB | dB |  |  |  |
| NPSS_RA | dB |  |  |  |
| NSSS_RA | dB |  |  |  |
| NPDCCH_RA | dB |  |  |  |
| NPDCCH_RB | dB |  |  |  |
| NPDSCH_RA | dB |  |  |  |
| NPDSCH_RB | dB |  |  |  |
| NOCNG_RA Note 1 | dB |  |  |  |
| NOCNG_RB Note 1 | dB |  |  |  |
| DRX |  | OFF |  |  |
|  | dBm/15 kHz | Specified in Table A.6.2.17.1-2 |  |  |
|  | dB | -12.5 |  |  |
| Note 3 | dB | -12.5 |  |  |
| NRSRP Note 3 | dBm/15 kHz | -110.5 |  |  |
| Io Note 3 | dBm/180 kHz | -86.97 |  |  |
| Propagation Condition | - | AWGN |  |  |
| Antenna Configuration |  | 2x1 |  |  |
| Note 1: NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, NRSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |

Table A.6.2.17.1-2: eCell specific test parameters for HD-FDD contention based random access test for UE category NB1 In-Band mode in Enhanced Coverage

| Parameter | Unit | Value |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| BWchannel | MHz | 10 |
| NOCNG Pattern Note 1 | - | NOP.1 FDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RA Note 1 | dB |  |
| OCNG_RB Note 1 | dB |  |
|  | dBm/15 kHz | -98 |
|  | dB | 3 |
| Propagation Condition | - | AWGN |
| Antenna Configuration | - | 2x1 |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.6.2.17.1-3: Void

Table A.6.2.17.1-4: NPRACH-Configuration parameters for HD-FDD contention based random access test for UE category NB1 In-Band mode in Enhanced Coverage

| Field | Value |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH resource |  |  |  |  |  |  |
| RSRP-ThresholdsNPRACH-InfoList | {35, 56} |  |  |  | Corresponding to {-121, -100} dBm as defined in Section 9.1.22. 9 |  |
| nprach-CP-Length | us266dot7 |  |  |  |  |  |
| nrs-Power | -5 dBm/15 kHz |  |  |  | As defined in clause 6.7.3 in TS 36.331. |  |
| Backoff Parameter Index | 1 |  |  |  | As defined in table 7.2-2 in TS 36.321 |  |
| Configured UE transmitted power () | 23 dBm for power class 3, 20 dBm for power class 5 |  |  |  | As defined in clause 6.2.5F in TS 36.101 |  |
| powerRampingStep | dB2 |  |  |  |  |  |
| preambleInitialReceivedTargetPower | dBm-120 |  |  |  |  |  |
| preambleTransMax-CE | n6 |  |  |  |  |  |
| Parameters per NPRACH Resource |  |  |  |  |  |  |
| NPRACH Resource | Level 0 | Level 1 | Level 2 |  |  |
| nprach-Periodicity | ms40 | ms240 | ms1280 |  |  |
| nprach-StartTime | ms8 | ms64 | ms512 |  |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart | zero | zero | zero |  |  |
| maxNumPreambleAttemptCE | n3 | n6 | n10 |  |  |
| numRepetitionsPerPreambleAttempt | n2 | n8 | n64 |  |  |
| npdcch-NumRepetitions-RA | r4 | r16 | r128 |  |  |
| npdcch-StartSF-CSS-RA | v1dot5 | v1dot5 | v1dot5 |  |  |
| npdcch-Offset-RA | zero | zero | Zero |  |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  |  |
| ra-ResponseWindowSize (per NPRACH Resource) | pp2 | pp2 | pp2 |  |  |
| mac-ContentionResolutionTimer (per NPRACH Resource) | pp2 | pp2 | pp2 |  |  |
| Note 1: See Clause 6.7.3 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |  |  |

A.6.2.17.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.6.2.17.2.1	Random Access Response Reception

To test the UE behavior specified in Subclause 6.6.2, the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preamble transmission attempts (the preamble may be transmitted multiple times in each attempt) have been received by the System Simulator. In response to the first 4 preamble transmission attempts, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.6.2. The power of the first preamble shall be 23 dBm for power class 3 and 20 dBm for power class 5 with an accuracy specified in clause 6.3.5F.1.1 of TS 36.101 [5].

The transmit timing of all NPRACH transmissions shall be within the accuracy specified in Subclause 7.20.2.

A.6.2.17.2.2	No Random Access Response Reception

To test the UE behavior specified in subclause 6.6.2.2, the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preamble transmission attempts have been received by the System Simulator. The System Simulator shall not respond to the first 4 preamble transmission attempts.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.6.2. The power of the first preamble shall be 23 dBm for power class 3 and 20 dBm for power class 5 with an accuracy specified in clause 6.3.5F.1.1 of TS 36.101 [5].

The transmit timing of all NPRACH transmissions shall be within the accuracy specified in Subclause 7.20.2.

A.6.2.17.2.3	Receiving a NACK on msg3

To test the UE behavior specified in subclause 6.6.2.3, the System Simulator shall NACK all UE msg3 following a successful Random Access Response.

The UE shall re-transmit the msg3 upon the reception of a NACK on msg3 until the maximum number of re-transmissions defined by maxNumPreambleAttemptCE in the table A.6.2.17.1-4 is reached.

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI

To test the UE behavior specified in Subclause 6.6.2.4, the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI

To test the UE behavior specified in Subclause 6.6.2.4, the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element matching the CCCH SDU transmitted in the msg3 uplink message.

The UE shall send ACK if the Contention Resolution is successful.

A.6.2.17.2.6	Contention Resolution Timer expiry

To test the UE behavior specified in Subclause 6.6.2.5, the System Simulator shall not send a response to a msg3.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

A.6.2.17.2.7	NPRACH Resource Selection

The UE shall select NPRACH resources and transmits or re- transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1. The rate of correct coverage enhancement level selection during repeated tests shall be at least 90%.

Note: Correct coverage enhancement level selection is a prerequisite for testing the other NPRACH requirements.

<< End of Change 1 >>