3GPP TSG-RAN WG4> Meeting #90	R4-1901545

# Athens, Greece,  25 Feb - 1 Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | <CR#> | rev | <Rev#> | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.817-02 capture FR2 TX ON/OFF transient background |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA-Perf |  |  |  |  |  | Date: |  |  | 25-01-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The FR2 TX ON/OFF transient test requirememt is still not described, the background for the measurement procedure needs to be documented |  |  |  |  |  |  |  |  |
| Summary of change: |  | The background of the TRP to EIRP trasnslation is captured in the TR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the TX OFF EIRP measurement was derived. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<START OF  CHANGE>

#### 9.5.1.3	OTA Transmitter OFF power for NR BS Type 2-O

For BS Type 1-O three cases are analyzed:

-	Own receiver – this is possible but, it is unlikely a TDD system will have separate Tx and Rx antennas so will be identified by the RX sensitivity requirement even for a conducted system.

-	Co-located receivers – the requirement is measured for the Tx channel – however the same noise can be expected across the whole operational band. Other co-located receivers using other channels in the band are hence protected.

-	Same geographical area receivers – most co-location requirements in the same geographical area assume a reasonable separation between BS’s, hence the isolation is assumed to be greater.

In the case of inter-site interference, the TDD system must be dimensioned with a Guard Period which corresponds to the cell size and its corresponding propagation delay. If we assume that the cell size corresponds to an ISD of d as illustrated in Figure 9.5.1.3-1, we can then calculate the OFF power interference at the neighboring site that can be tolerated as follows.

d

Figure 9.5.1.3-1: The inter site interference propagation case

Assume the Total Radiated Power Off Power, TRPOff , from an interfering site traverse a path loss LP to reach the victim receiver. The power at the victim receiver array is then:

TRPOff - LP

The off power into each victim transceiver equipped with an element receive gain of  GERX ,becomes:

TRPOff - LP+ GERX

The total off power fed into the whole array of N transceivers, then becomes:

TRPOff - LP+ GERX+10logN   (1)

If we assume the margin for a given noise increase in the system is M dB and the applicable bandwidth of B Hz, we get the total noise in the system as:

-174 + 10log(B) + NF - M    (2)

From equations  (1) and (2) we get

TRPOff=-174 + 10log(B) + NF - M - 10logN+LP - GERX

The path loss LP can be estimated using the 3GPP model for NLOS case:

Where:

-	d is in km, and f in GHz

LP=31.3 - (15.07 - 1480hb2)log(hb)+(43.42-3.1log(hb))log(d)+ 20log(f)- 0.6(hm-1.5)

By assuming the following parameters for the path loss LP:

-	B = 1 MHz

-	NF = 10 dB

-	M = 6 dB, for 1 dB noise rise

-	N = 128 transceivers

-	GETX= GERX= 6 dB

-	f = 28 GHz

-	d = 100 m

-	hb = 25 m

-	hm = 1.5 m

We get a value for LP= 117 dB. However, when considering interference, the NLOS case is not the limiting factor but rather the LOS case. In 3GPP we have 2 LOS models LOS1 and LOS2. The path loss LP for each of these LOS models can be expressed as follows:

LP = 28 + 22log(d) + 20log(f), at LOS1

LP = 28 + 22log(d) + 20log(f)– 9log(dbrk2+(hb-hm)2), at LOS2

LOS1 model is used for the Urban environment which is characterized by shorter distance.

We get LP= 101 dB and the corresponding off power TRPOff = - 36 dBm/MHz.

This value of -36dBm/MHz can then be a candidate to be adopted as the Transmitter off power for BS Type 2-O if test equipment can measure such low power at FR2.

This value can be measured either as TRP or at the output of the colocation reference antenna.

In case of transmit power off measurement as a TRP, the transient time can also be measured as a relative EIRP measurement.

For the colocation interference scenario corresponds to when the base stations of two operators mutually interfere with each other simultaneously. A typical configuration is shown in Figure 9.5.1.3-2 where both antennas are mounted on the same vertical plane. The most difficult case is when both operators coexist in the same band.

The TDD network is synchronized to within a TSync, Cell Phase Sync requirement and the Guard Period TGUARD is dimensioned to cope with the cell size and the transient times:

TGUARD ≥ 2* TSync + 2*Tprop_cell edge +max ((TBS on off), (TUE off on)) + max ((TBS off on), (TUE on off))

The synchronization assumption, the guard period and the assumptions that UL and DL TDD configurations are coordinated so that only the TDD OFF power needs to be considered.

d

Figure 9.5.1.3-2: Colocation interference scenario

This colocation scenario is governed by the OTA reference sensitivity. Equipment not conforming to reference sensitivity requirements would self desense their own transceiver array and its neighboring sectors, even in its own channel. In this colocation case we would have some additional protection, since this is a neighbor carrier, even if it is in the same band.

For the scenario of sector site example, where different sectors are installed within one site. This scenario is also governed by the OTA reference sensitivity. Equipment not conforming to reference sensitivity requirements would self desense their own transceiver array and its neighboring.

The TX OFF level core requirement is expressed in terms of TRP, however it is not possible to measure the transient nature of the requirement as TRP. The TX OFF level and the transient is hence measured as an EIRP parameter with an appropriate assumption of the antenna gain to translate TRP to and EIRP value.

The TX transient TX OFF level and the transient applied to it are at a higher level than the level required to provided good sensitivity for the BS own receiver, however the TX OFF level is required after 3usec whereas the sensitivity requirement has a guard band before the measurement is taken. The OFF level for the transient test therefore is likely to be on the transient slope between ON and the off level in the receiver timeslot.

Practical implementations of FR2 transmitter along with methods of turning the power off fat enough to meet the 3usec transient requirement result in the TX noise being non-correlated and hence the antenna gain will be similar to that of the element pattern.

An approximation of the element pattern gain to translate the TRP requirement to a measurable EIRP value can be obtained using the declared values of OTA REFSENS RoAoA and Elliot’s formula

Note. The same approach is used to calculate the FR1 OTA REFSENS value in sub-clause 10.3.2

Resulting in an EIRP requirement of:

-36dBm – (44.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS))

For example if the element spacing is 0.5λ x 0.5λ and the corresponding element beam pattern is approx. 90º x 90º then the EIRP requirement is:

-36dBm – (44.1 - 10*log10(90*90)) = -36-5 = -39dBm

<END OF CHANGE>