3GPP TSG-RAN WG4> Meeting #90	R4-1901550

# Athens, Greece,  25 Feb - 1 Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.105 | CR | 0126 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.105 Spurious emissions scaling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA-Core |  |  |  |  |  | Date: |  |  | 25-01-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The ECC recommendation 74-01 has been released for public consultation so the spurious emissions scaling for ECC is updated to be aligned with this document. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is made clear that the calues in some cases include the scaling factor and that regional regulations may differ. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specifiation does not reflect the current ECC regulation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.6.2, 6.6.6.3, 6.6.6.4, 9.7.6.3, 9.7.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<START OF  CHANGE>

#### 6.6.6.2	Minimum requirement for MSR operation

The MSR spurious emission basic limits are the same as those specified in 3GPP TS 37.104 [9], subclauses 6.6.1.1, 6.6.1.2, 6.6.1.3 and 6.6.1.4.

The spurious emission requirements for an MSR AAS BS are that for each TAB connector TX mincell group and each applicable basic limit as specified in 3GPP TS 37.104 [5], the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed an AAS limit specified as the basic limit + 10log10(NTXU,countedpercell), unless stated differently in regional regulation.

NOTE:	Conformance to the AAS BS spurious emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the emissions power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the AAS BS limit as defined in this subclause for the respective frequency span.

Or

2)	The unwanted emissions power at each TAB connector shall be less than or equal to the AAS BS limit as defined in this subclause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

#### 6.6.6.3	Minimum requirement for single RAT UTRA operation

The single RAT UTRA FDD spurious emission basic limits are the same as those specified in 3GPP TS 25.104 [6], subclauses 6.6.3.1-6.6.3.8.

The single RAT UTRA TDD spurious emission basic limits are the same as those specified in 3GPP TS 25.105 [7], subclauses 6.6.3.1-6.6.3.5.

The spurious emission requirements for a UTRA single RAT AAS BS are that for each TAB connector TX min cell group and each applicable basic limit as specified in 3GPP TS 25.104 [6] or 3GPP TS 25.105 [7], the power sum of the emissions at the TAB connectors associated with the TAB connector TX min cell group shall not exceed an AAS limit specified as the basic limit + 10log10(NTXU,countedpercell), unless stated differently in regional regulation.

NOTE:	Conformance to the AAS BS spurious emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the emissions power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the AAS BS limit as defined in this subclause for the respective frequency span.

Or

2)	The unwanted emissions power at each TAB connector shall be less than or equal to the AAS BS limit as defined in this subclause for the respective frequency span,  scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

#### 6.6.6.4	Minimum requirement for single RAT E-UTRA operation

Single RAT E-UTRA operation spurious emission basic limits are the same as those specified in 3GPP TS 36.104 [8], subclauses 6.6.4.1-6.6.4.4.

The spurious emission requirements for AAS BS in single RAT E-UTRA operation are that for each TAB connector TX min cell group and for each applicable basic limit as specified in 3GPP TS 36.104 [4], the total emissions at the TAB connectors associated with the TAB connector TX min cell group shall not exceed an AAS limit specified as the basic limit + 10log10(NTXU,countedpercell), unless stated differently in regional regulation.

NOTE:	Conformance to the AAS BS spurious emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the emissions power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the AAS BS limit as defined in this subclause for the respective frequency span.

Or

2)	The unwanted emissions power at each TAB connector shall be less than or equal to the AAS BS limit as defined in this subclause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

<NEXT CHANGE>

#### 9.7.6.3	Minimum requirement for single RAT UTRA operation

##### 9.7.6.3.1	Mandatory Requirements

###### 9.7.6.3.1.1	Minimum requirement (Category A)

The minimum requirement for single RAT UTRA BS is the same as that defined for an MSR BS in subclause 9.7.6.2.1.1.

###### 9.7.6.3.1.2	Minimum requirement (Category B)

The following limits shall be met in cases where Category B limits for spurious emissions, as defined in ITU-R Recommendation SM.329 [17], are applied.

The TRP of any spurious emission shall not exceed the limits in table 9.7.6.2.1.1-2

Table 9.7.6.3.1.2-1: OTA AAS BS Mandatory spurious emissions limits, operating band I, II, III, IV, VII, X, XXII, XXV, XXXII (Category B)

| Band | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz 1 GHz | -30 dBm | 100 kHz | NOTE 1，NOTE 5， NOTE 6 |
| 1 GHz Flow - 10 MHz | -24 dBm | 1 MHz | NOTE 1，NOTE 5， NOTE 6 |
| Flow - 10 MHz Fhigh + 10 MHz | -9 dBm | 1 MHz | NOTE 2，NOTE 5， NOTE 6 |
| Fhigh + 10 MHz 12.75 GHz | -24 dBm | 1 MHz | NOTE 3，NOTE 5， NOTE 6 |
| 12.75 GHz - 5th harmonic of the upper frequency edge of the DL operating band in GHz | -24 dBm | 1 MHz | NOTE 3, NOTE 4，NOTE 5， NOTE 6 |
| NOTE 1: Bandwidth as in ITU-R Recommendation SM.329 [14], s4.1 NOTE 2: Limit based on ITU-R Recommendation SM.329 [14], s4.3 and Annex 7 NOTE 3: Bandwidth as in ITU-R Recommendation SM.329 [14], s4.1. Upper frequency as in ITU-R SM.329 [17], s2.5 table 1 NOTE 4: Applies only for Band XXII NOTE 5: The requirement is derived from the basic limit a scaling factor of 9 dB NOTE 6: The requirements may be subject to additional regional regulation. |  |  |  |
| Key: Flow: The lowest downlink frequency of the operating band as defined in subclause 9.7.1 Fhigh: The highest downlink frequency of the operating band as defined in subclause 9.7.1 |  |  |  |

Table 9.7.6.3.1.2-2: BS Mandatory spurious emissions limits, operating band V, VIII, XII, XIII, XIV, XX, XXVI (Category B)

| Band | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz Flow - 10 MHz | -30 dBm | 100 kHz | NOTE 1，NOTE 4， NOTE5 |
| Flow - 10 MHz Fhigh + 10 MHz | -10 dBm | 100 kHz | NOTE 2，NOTE 4， NOTE5 |
| Fhigh + 10 MHz 1 GHz | -30 dBm | 100 kHz | NOTE 1，NOTE 4， NOTE5 |
| 1GHz 12.75GHz | -24 dBm | 1 MHz | NOTE 3，NOTE 4， NOTE5 |
| NOTE 1: Bandwidth as in ITU-R Recommendation SM.329 [17], s4.1 NOTE 2: Limit based on ITU-R Recommendation SM.329 [17], s4.3 and Annex 7 NOTE 3: Bandwidth as in ITU-R Recommendation SM.329 [17], s4.1. Upper frequency as in ITU-R SM.329 [17], s2.5 table 1 NOTE 4: The requirement is derived from the basic limit a scaling factor of 9 dB NOTE 5: The requirements may be subject to additional regional regulation. |  |  |  |
| Key: Flow: The lowest downlink frequency of the operating band as defined in subclause 9.7.1 Fhigh: The highest downlink frequency of the operating band as defined in subclause 9.7.1 |  |  |  |

<NEXT CHANGE>

#### 9.7.6.4	Minimum requirement for single RAT E-UTRA operation

##### 9.7.6.4.1	Mandatory Requirements

###### 9.7.6.4.1.1	Minimum requirement (Category A)

The minimum requirement for single RAT E-UTRA BS is the same as that defined for an MSR BS in subclause 9.7.6.2.1.1.

###### 9.7.6.4.1.2	Minimum Requirement (Category B)

The TRP of any spurious emission shall not exceed the limits in table 9.7.6.4.1.2-1

Table 9.7.6.4.1.2-1: AAS BS OTA Spurious emissions limits, Category B

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz 1 GHz | -27 dBm | 100 kHz | NOTE 1，NOTE 4， NOTE 5 |
| 1 GHz 12.75 GHz | -21 dBm | 1 MHz | NOTE 2，NOTE 4， NOTE 5 |
| 12.75 GHz 5th harmonic of the upper frequency edge of the DL operating band in GHz | -21 dBm | 1 MHz | NOTE 2, NOTE 3, NOTE 4， NOTE 5 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [14] , s4.1 NOTE 2: Bandwidth as in ITU-R SM.329 [14] , s4.1. Upper frequency as in ITU-R SM.329 [14] , s2.5 table 1 NOTE 3: Applies only for Bands 22, 42 and 43. NOTE 4: The requirement is derived from the basic limit a scaling factor of 9 dB. NOTE 5: The requirements may be subject to additional regional regulation. |  |  |  |

<END OF CHANGE>