3GPP TSG-RAN WG4 Meeting #90	R4-1901615

Athens, Greece, February 25th- March 1st

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0031 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change lower limit of band n259 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Many administrations globally have shown interest in allocation of the whole or different part of the 37-43.5 frequency range to mobile services. Therefore, it is needed that 3GPP also provides technical requirements for the whole range. To do so, it is proposed to define limits of band n259 as 39.8 -43.5 GHz (a new lower limit of 39.8 GHz). Accordingly, combination of band n260 (37-40 GHz) and band n259 (39.8-43.5 GHz) will cover the whole range of 37-43.5 GHz. The proposed limit results in 200 MHz overlap between n259 and n260 which in turn will allow a 200MHz carrier to be used everywhere in the band for efficient use of spectrum. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 9.2.1: Tables 9.2.1-1 and 9.2.1-2 are updated with introduction of band n259 with the Directivity variation value in square bracket |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This will hinder administrations interested in allocation of the whole band for mobile services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

## 9.2	Radiated transmit power

### 9.2.1	General

The minimum requirements for radiated transmit power, are placed on one or more manufacturer declared beam(s) over a declared OTA peak direction set. OTA requirements for NR BS output power are defined for directional EIRP requirements as radiated transmit power requirements (sub-clauses 9.2.2 and 9.2.3) and for TRP requirements as OTA base station output power (sub-clauses 9.3.2 and 9.3.3).

Some NR bands have a significantly larger fractional band width than existing bands as shown in Table 9.2.1-1.

Table 9.2.1-1: NR bands

| Band | Band Definition fl to fh (MHz) | Absolute Bandwidth fh – fl (MHz) | Relative Bandwidth fh/fl (dB) | Fractional Bandwidth 100.(fh-fl)/fc (%) |
| --- | --- | --- | --- | --- |
| n41 | 2496 to 2690 | 194 | 0.3 | 7.5 |
| n77 | 3300 to 4200 | 900 | 1.0 | 24.0 |
| n78 | 3300 to 3800 | 500 | 0.6 | 14.1 |
| n79 | 4400 to 5000 | 600 | 0.6 | 12.8 |
| n257 | 26500 to 29500 | 3000 | 0.5 | 10.7 |
| n258 | 24250 to 27500 | 3250 | 0.5 | 12.6 |
| n259 | 39800 to 43500 | 3700 | 0.4 | 8.8 |
| n260 | 37000 to 40000 | 2500 | 0.3 | 6.5 |
| n261 | 27500 to 28350 | 850 | 0.13 | 3.0 |

The fractional bandwidth FBW is given in percent as:

The radiated transmit power in a specific direction in terms of EIRP can be expressed as: EIRP = TRP + D in dBm, where TRP is the total radiated power in dBm and D is the directivity in dBi. For an array antenna the composite directivity is determined by the element directivity and the array factor directivity. From TR 38.803 [24], Annex C [2], the element directivity can be expressed as:

where Aeff is the antenna aperture in m2 and  is the wave length in m.

For a uniform rectangular array antenna, where elements as separated d m along both y-axis and the z-axis, the maximum area for the antenna aperture is limited to an area of (d)2 m2. Typically, the d is in the range of 0.5 to 0.7 and  is derived from the highest supported frequency. Since the directivity is depending on frequency, it is interesting to analyze the directivity characteristics as function of very wide operation bands.

In Table 9.2.1-2, some wide NR bands have been analyzed with respect to directivity variations (due to fixed antenna aperture.

Table 9.2.1-2: Directivity variation over supported frequency range

| Band |  (dB) |
| --- | --- |
| n41 | -0.6 |
| n77 | -2.1 |
| n78 | -1.2 |
| n79 | -1.1 |
| n257 | -0.9 |
| n258 | -1.1 |
| n259 | [-0.7] |
| n260 | -0.7 |

It’s clear that the directivity is lower at lowest supported frequency compared with the highest supported frequency as expected. For a NR base station supporting wide bands the directivity variation is manganged by declaring EIRP at lowest supported frequency and highest supported frequency within a specific band.

< end of changes >