3GPP TSG RAN WG4 #90	R4-1902425

# Athens, Greece, 25 Feb 2019 - 1 Mar, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | CRNum | rev | 0 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on RI Tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-02-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some parameters and requirements are TBD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some of the TBD parameters and requirements were defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for RI reporting will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-4 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

## 6.4	Reporting of Rank Indicator (RI)

The purpose of this test is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission.

### 6.4.1	1RX requirements

(Void)

### 6.4.2	2RX requirements

#### 6.4.2.1	FDD

The minimum performance requirement in Table 6.4.2.1-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 6.4.2.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.4.2.1-2.

Table 6.4.2.1-1: RI Test (FDD)

| Parameter |  |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  | Test 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  |  |  | MHz | 10 |  |  | 10 |  | 10 |
| Duplex Mode |  |  |  |  |  |  | FDD |  |  | FDD |  | FDD |
| DL BWP configuration #1 |  |  | First PRB |  |  |  | 0 |  |  | 0 |  | 0 |
|  |  |  | Number of contiguous PRB |  |  |  | 52 |  |  | 52 |  | 52 |
|  |  |  | Subcarrier spacing |  |  | kHz | 15 |  |  | 15 |  | 15 |
| SNR |  |  |  |  |  | dB | [0] |  |  | [20] |  | [20] |
| Propagation channel |  |  |  |  |  |  | TDLA30-5 |  |  | TDLA30-5 |  | TDLA30-5 |
| Antenna configuration |  |  |  |  |  |  | ULA Low 2x2 |  |  | ULA Low 2x2 |  | ULA High 2x2 |
| Beamforming Model |  |  |  |  |  |  | As defined in Annex B.4.1 |  |  | As defined in Annex B.4.1 |  | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  |  |  | Periodic |  |  | Periodic |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  |  |  | 4 |  |  | 4 |  | 4 |
|  | CDM Type |  |  |  |  |  | FD-CDM2 |  |  | FD-CDM2 |  | FD-CDM2 |
|  | Density (ρ) |  |  |  |  |  | 1 |  |  | 1 |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  |  |  | Row 5, (4,-) |  |  | Row 5, (4,-) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  |  |  | (9,-) |  |  | (9,-) |  | (9,-) |
|  | CSI-RS periodicity and offset |  |  |  |  | slot | 5/1 |  |  | 5/1 |  | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  |  |  | Periodic |  |  | Periodic |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  |  |  | 2 |  |  | 2 |  | 2 |
|  | CDM Type |  |  |  |  |  | FD-CDM2 |  |  | FD-CDM2 |  | FD-CDM2 |
|  | Density (ρ) |  |  |  |  |  | 1 |  |  | 1 |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  |  |  | Row 3 (6,-) |  |  | Row 3 (6,-) |  | Row 3 (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  |  |  | (13,-) |  |  | (13,-) |  | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  |  |  | slot | 5/1 |  |  | 5/1 |  | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  |  |  | Pattern 0 |  |  | Pattern 0 |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  |  |  | (4,9) |  |  | (4,9) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  |  |  |  | slot | 5/1 |  |  | 5/1 |  | 5/1 |
| ReportConfigType |  |  |  |  |  |  | Periodic |  |  | Periodic |  | Periodic |
| CQI-table |  |  |  |  |  |  | Table 2 |  |  | Table 2 |  | Table 2 |
| reportQuantity |  |  |  |  |  |  | cri-RI-PMI-CQI |  |  | cri-RI-PMI-CQI |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  |  |  | not configured |  |  | not configured |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  |  | not configured |  |  | not configured |  | not configured |
| cqi-FormatIndicator |  |  |  |  |  |  | Wideband |  |  | Wideband |  | Wideband |
| pmi-FormatIndicator |  |  |  |  |  |  | Wideband |  |  | Wideband |  | Wideband |
| Sub-band Size |  |  |  | RB | [8] | [8] | [8] |
| csi-ReportingBand |  |  |  |  | [1111111] | [1111111] | [1111111] |
| CSI-Report periodicity and offset |  |  |  |  |  | slot | 5/1 |  |  | 5/1 |  | 5/1 |
| Codebook configuration |  | Codebook Type |  |  |  |  | typeI-SinglePanel |  |  | typeI-SinglePanel |  | typeI-SinglePanel |
|  |  | Codebook Mode |  |  |  |  | 1 |  |  | 1 |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  |  | N/A |  |  | N/A |  | N/A |
|  |  | CodebookSubsetRestriction |  |  |  |  | [010000 for fixed rank 2, 010011 for following rank] |  |  | [000011 for fixed rank 1, 010011 for following rank] |  | [000011 for fixed rank 1, 010011 for following rank] |
|  |  | RI Restriction |  |  |  |  | N/A |  |  | N/A |  | N/A |
| Physical channel for CSI report |  |  |  |  |  |  | PUCCH |  |  | PUCCH |  | PUCCH |
| CQI/RI/PMI delay |  |  |  |  |  | ms | 8 |  |  | 8 |  | 8 |
| Maximum number of HARQ transmission |  |  |  |  |  |  | 1 |  |  | 1 |  | 1 |
| RI Configuration |  |  |  |  |  |  | Fixed RI = 2 and follow RI |  |  | Fixed RI = 1 and follow RI |  | Fixed RI = 1 and follow RI |

Table 6.4.2.1-2: Minimum requirement (FDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | [1.05] | [0.9] |
| 2 | [1.0] | N/A | N/A |

#### 6.4.2.2	TDD

The minimum performance requirement in Table 6.4.2.2-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 6.4.2.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.4.2.2-2.

Table 6.4.2.2-1: RI Test (TDD)

| Parameter |  |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  | Test 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  |  |  | MHz | 40 |  |  | 40 |  | 40 |
| Duplex Mode |  |  |  |  |  |  | TDD |  |  | TDD |  | TDD |
| TDD Slot Configuration |  |  |  |  |  |  | FR1.30-1 |  |  | FR1.30-1 |  | FR1.30-1 |
| DL BWP configuration #1 |  |  | First PRB |  |  |  | 0 |  |  | 0 |  | 0 |
|  |  |  | Number of contiguous PRB |  |  |  | 106 |  |  | 106 |  | 106 |
|  |  |  | Subcarrier spacing |  |  | kHz | 30 |  |  | 30 |  | 30 |
| SNR |  |  |  |  |  | dB | [0] |  |  | [20] |  | [20] |
| Propagation channel |  |  |  |  |  |  | TDLA30-5 |  |  | TDLA30-5 |  | TDLA30-5 |
| Antenna configuration |  |  |  |  |  |  | ULA Low 2x2 |  |  | ULA Low 2x2 |  | ULA High 2x2 |
| Beamforming Model |  |  |  |  |  |  | As defined in Annex B.4.1 |  |  | As defined in Annex B.4.1 |  | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  |  |  | Periodic |  |  | Periodic |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  |  |  | 4 |  |  | 4 |  | 4 |
|  | CDM Type |  |  |  |  |  | FD-CDM2 |  |  | FD-CDM2 |  | FD-CDM2 |
|  | Density (ρ) |  |  |  |  |  | 1 |  |  | 1 |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  |  |  | Row 5, (4,-) |  |  | Row 5, (4,-) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  |  |  | (9,-) |  |  | (9,-) |  | (9,-) |
|  | CSI-RS periodicity and offset |  |  |  |  | slot | 10/1 |  |  | 10/1 |  | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  |  |  | Periodic |  |  | Periodic |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  |  |  | 2 |  |  | 2 |  | 2 |
|  | CDM Type |  |  |  |  |  | FD-CDM2 |  |  | FD-CDM2 |  | FD-CDM2 |
|  | Density (ρ) |  |  |  |  |  | 1 |  |  | 1 |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  |  |  | Row 3 (6,-) |  |  | Row 3 (6,-) |  | Row 3 (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  |  |  | (13,-) |  |  | (13,-) |  | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  |  |  | slot | 10/1 |  |  | 10/1 |  | 10/1 |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  |  |  | Pattern 0 |  |  | Pattern 0 |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  |  |  | (4,9) |  |  | (4,9) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  |  |  |  | slot | 10/1 |  |  | 10/1 |  | 10/1 |
| ReportConfigType |  |  |  |  |  |  | Periodic |  |  | Periodic |  | Periodic |
| CQI-table |  |  |  |  |  |  | Table 2 |  |  | Table 2 |  | Table 2 |
| reportQuantity |  |  |  |  |  |  | cri-RI-PMI-CQI |  |  | cri-RI-PMI-CQI |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  |  |  | not configured |  |  | not configured |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  |  | not configured |  |  | not configured |  | not configured |
| cqi-FormatIndicator |  |  |  |  |  |  | Wideband |  |  | Wideband |  | Wideband |
| pmi-FormatIndicator |  |  |  |  |  |  | Wideband |  |  | Wideband |  | Wideband |
| Sub-band Size |  |  |  | RB | [16] | [16] | [16] |
| csi-ReportingBand |  |  |  |  | [1111111] | [1111111] | [1111111] |
| CSI-Report periodicity and offset |  |  |  |  |  | slot | 10/1 |  |  | 10/1 |  | 10/1 |
| Codebook configuration |  | Codebook Type |  |  |  |  | typeI-SinglePanel |  |  | typeI-SinglePanel |  | typeI-SinglePanel |
|  |  | Codebook Mode |  |  |  |  | 1 |  |  | 1 |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  |  | N/A |  |  | N/A |  | N/A |
|  |  | CodebookSubsetRestriction |  |  |  |  | [010000 for fixed rank 2, 010011 for following rank] |  |  | [000011 for fixed rank 1, 010011 for following rank] |  | [000011 for fixed rank 1, 010011 for following rank] |
|  |  | RI Restriction |  |  |  |  | N/A |  |  | N/A |  | N/A |
| Physical channel for CSI report |  |  |  |  |  |  | PUCCH |  |  | PUCCH |  | PUCCH |
| CQI/RI/PMI delay |  |  |  |  |  | ms | 9.5 |  |  | 9.5 |  | 9.5 |
| Maximum number of HARQ transmission |  |  |  |  |  |  | 1 |  |  | 1 |  | 1 |
| RI Configuration |  |  |  |  |  |  | Fixed RI = 2 and follow RI |  |  | Fixed RI = 1 and follow RI |  | Fixed RI = 1 and follow RI |

Table 6.4.2.2-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | [1.05] | [0.9] |
| 2 | [1.0] | N/A | N/A |

### 6.4.3	4RX requirements

#### 6.4.3.1	FDD

The minimum performance requirement in Table 6.4.3.1-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 6.4.3.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.4.3.1-2.

Table 6.4.3.1-1: RI Test (FDD)

| Parameter |  |  |  | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 | 10 | 10 | 10 |
| Duplex Mode |  |  |  |  | FDD | FDD | FDD | FDD |
| DL BWP configuration #1 |  |  | First PRB |  | 0 | 0 | 0 | 0 |
|  |  |  | Number of contiguous PRB |  | 52 | 52 | 52 | 52 |
|  |  |  | Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| SNR |  |  |  | dB | TBD | [16] | [16] | TBD |
| Propagation channel |  |  |  |  | TDLA30-5 | TDLA30-5 | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  |  |  | ULA Low 2x4 | ULA Low 2x4 | ULA High 2x4 | ULA Low 4x4 |
| Beamforming Model |  |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 4 | 4 | 4 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (9,-) | (9,-) | (9,-) | (9,-) |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 | 5/1 | 5/1 | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 2 | 2 | 2 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) | Row 4 (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (13,-) | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 | 5/1 | 5/1 | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | Pattern 0 | Pattern 0 | Pattern 0 | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) | (4,9) | (4,9) | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 | 5/1 | 5/1 | 5/1 |
| ReportConfigType |  |  |  |  | Periodic | Periodic | Periodic | Periodic |
| CQI-table |  |  |  |  | Table 2 | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  | not configured | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | not configured | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband | Wideband |
| Sub-band Size |  |  |  | RB | [8] | [8] | [8] | [8] |
| csi-ReportingBand |  |  |  |  | [1111111] | [1111111] | [1111111] | [1111111] |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 | 5/1 | 5/1 | 5/1 |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  |  | 1 | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | N/A | N/A | N/A | (2,1) |
|  |  | CodebookSubsetRestriction |  |  | [010000 for fixed rank 2, 010011 for following rank] | [000011 for fixed rank 1, 010011 for following rank] | [000011 for fixed rank 1, 010011 for following rank] | 11111111 |
|  |  | RI Restriction |  |  | N/A | N/A | N/A | 00000010 for fixed Rank 2 and 00001111 for follow RI |
| Physical channel for CSI report |  |  |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  |  | ms | 8 | 8 | 8 | 8 |
| Maximum number of HARQ transmission |  |  |  |  | 1 | 1 | 1 | 1 |
| RI Configuration |  |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 2 and follow RI |

Table 6.4.3.1-2: Minimum requirement (FDD)

|  | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| 1 | N/A | [1.05] | [0.9] | N/A |
| 2 | TBD | N/A | N/A | TBD |

#### 6.4.3.2	TDD

The minimum performance requirement in Table 6.4.3.2-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 6.4.3.2-1, and using the downlink physical channels specified in Annex TBD, the minimum requirements are specified in Table 6.4.3.2-2.

Table 6.4.3.2-1: RI Test (TDD)

| Parameter |  |  |  | Unit | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 | 40 | 40 | 40 |
| Duplex Mode |  |  |  |  | TDD | TDD | TDD | TDD |
| TDD Slot Configuration |  |  |  |  | FR1.30-1 | FR1.30-1 | FR1.30-1 | FR1.30-1 |
| DL BWP configuration #1 |  |  | First PRB |  | 0 | 0 | 0 | 0 |
|  |  |  | Number of contiguous PRB |  | 106 | 106 | 106 | 106 |
|  |  |  | Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| SNR |  |  |  | dB | TBD | [16] | [16] | TBD |
| Propagation channel |  |  |  |  | TDLA30-5 | TDLA30-5 | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  |  |  | ULA Low 2x4 | ULA Low 2x4 | ULA High 2x4 | ULA Low 4x4 |
| Beamforming Model |  |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 4 | 4 | 4 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (9,-) | (9,-) | (9,-) | (9,-) |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 | 10/1 | 10/1 | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  |  | 2 | 2 | 2 | 2 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) | Row 4 (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (13,-) | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 | 10/1 | 10/1 | 10/1 |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | Pattern 0 | Pattern 0 | Pattern 0 | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) | (4,9) | (4,9) | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 | 10/1 | 10/1 | 10/1 |
| ReportConfigType |  |  |  |  | Periodic | Periodic | Periodic | Periodic |
| CQI-table |  |  |  |  | Table 2 | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  | not configured | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | not configured | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband | Wideband |
| Sub-band Size |  |  |  | RB | [16] | [16] | [16] | [16] |
| csi-ReportingBand |  |  |  |  | [1111111] | [1111111] | [1111111] | [1111111] |
| CSI-Report periodicity and offset |  |  |  | slot | 10/1 | 10/1 | 10/1 | 10/1 |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  |  | 1 | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | N/A | N/A | N/A | (2,1) |
|  |  | CodebookSubsetRestriction |  |  | [010000 for fixed rank 2, 010011 for following rank] | [000011 for fixed rank 1, 010011 for following rank] | [000011 for fixed rank 1, 010011 for following rank] | 11111111 |
|  |  | RI Restriction |  |  | N/A | N/A | N/A | 00000010 for fixed Rank 2 and 00001111 for follow RI |
| Physical channel for CSI report |  |  |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 | 9.5 | 9.5 | 9.5 |
| Maximum number of HARQ transmission |  |  |  |  | 1 | 1 | 1 | 1 |
| RI Configuration |  |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 2 and follow RI |

Table 6.4.3.2-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| 1 | N/A | [1.05] | [0.9] | N/A |
| 2 | TBD | N/A | N/A | TBD |

<< End of changes >>

<< Start of changes >>

## 8.4	Reporting of Rank Indicator (RI)

The purpose of this test is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission.

### 8.4.1	1RX requirements

(Void)

### 8.4.2	2RX requirements

#### 8.4.2.1	FDD

(Void)

#### 8.4.2.2	TDD

The minimum performance requirement in Table 8.4.2.2-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 8.4.2.2-1, and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified in Table 8.4.2.2-2.

Table 8.4.2.2-1: RI Test (TDD)

| Parameter |  |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 100 | 100 | 100 |
| Duplex Mode |  |  |  |  | TDD | TDD | TDD |
| TDD Slot Configuration |  |  |  |  | FR1.120-2 | FR1.120-2 | FR1.120-2 |
| DL BWP configuration #1 |  |  | First PRB |  | 0 | 0 | 0 |
|  |  |  | Number of contiguous PRB |  | 66 | 66 | 66 |
|  |  |  | Subcarrier spacing | kHz | 120 | 120 | 120 |
| SNR |  |  |  | dB | [0] | [20] | [20] |
| Propagation channel |  |  |  |  | [TDLA30-35] | [TDLA30-35] | [TDLA30-35] |
| Antenna configuration |  |  |  |  | ULA Low 2x2 | ULA Low 2x2 | XP High 2x2 |
| Beamforming Model |  |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  |  |  | 4 | 4 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 4, (8,-) | Row 4, (8,-) | Row 4, (8,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (13,-) | (13,-) | (13,-) |
|  | CSI-RS interval and offset |  |  | slot | 8/1 | 8/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  |  |  | 2 | 2 | 2 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig interval and offset |  |  | slot | 8/1 | 8/1 | 8/1 |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | Pattern 1 | Pattern 1 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (8,13) | (8,13) | (8,13) |
|  | CSI-IM timeConfig interval and offset |  |  | slot | 8/1 | 8/1 | 8/1 |
| ReportConfigType |  |  |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  |  |  | Table 1 | Table 1 | Table 1 |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  |  |  | RB | [8] | [8] | [8] |
| csi-ReportingBand |  |  |  |  | [111111111] | [111111111] | [111111111] |
| CSI-Report interval and offset |  |  |  | slot | 8/1 | 8/1 | 8/1 |
| aperiodicTriggeringOffset |  |  |  |  | 0 | 0 | 0 |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  |  | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | N/A | N/A | N/A |
|  |  | CodebookSubsetRestriction |  |  | [010000 for fixed rank 2, 010011 for following rank] | [000011 for fixed rank 1, 010011 for following rank] | [000011 for fixed rank 1, 010011 for following rank] |
|  |  | RI Restriction |  |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  |  |  | ms | 1.375 | 1.375 | 1.375 |
| Maximum number of HARQ transmission |  |  |  |  | 1 | 1 | 1 |
| RI Configuration |  |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI |

Table 8.4.2.2-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | [1.05] | [1.05] |
| 2 | [1.0] | N/A | N/A |

<< End of changes >>