3GPP TSG-RAN4 Meeting #90bis 	R4-1902900

Xi’an, China, 8th-12th April, 2019

Title:	Updated reply LS on intra-band combination for NR-CA and MR-DC

Reply To:	R2-1813442

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG 4

To:	RAN WG 1, RAN WG 2

Cc:

Contact Person:

Name:	Jie Cui

E-mail Address:	jie.cui@intel.com

Attachments:	N.A.

1. Overall Description:

During RAN4#88Bis, RAN4 has sent a LS (R4-1813862) on intra-band combination for NR-CA and MR-DC to RAN2. Regarding on the contents related to Question 3 “Handling of TA in Synchronous MR-DC” in the LS, RAN4 has made the following conclusion.

For intra-band EN-DC, only collocated deployment is assumed. The MTTD and MRTD requirements for intra-band synchronous EN-DC are defined in RAN4 as below,

Table 7.5.3-1 Maximum uplink transmission timing difference requirement for intra-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz) Note1 | Maximum transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 5.21Note2 |
| 15 | 30 | 5.21 Note2 |
| 15 | 60 | 5.21 Note2 |
| NOTE 1: UL Sub-carrier spacing is min{SCSSS, SCSDATA}. NOTE 2: This is not applicable for UEs that indicate the capability of only supporting single UL timing (ul-TimingAlignmentEUTRA-NR is signalled). Single UL timing for E-UTRA and NR cell is assumed for UEs that is only capable of handling a single UL timing for E-UTRA and NR cell. |  |  |

Table 7.6.3-1 Maximum receive timing difference requirement for intra-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz) Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 3, 0.26 Note2 |
| 15 | 30 | 3 |
| 15 | 60 | 3 |
| NOTE 1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. NOTE 2: This is applicable for UEs that indicate the capability of only supporting single UL timing (ul-TimingAlignmentEUTRA-NR is signalled). Single UL timing for E-UTRA and NR cell is assumed for UEs that is only capable of handling a single UL timing for E-UTRA and NR cell. |  |  |

2. Actions:

To RAN WG1, WG2 group.

ACTION: 	RAN4 kindly asks RAN1 and RAN2 to take the above information into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 #91			13th – 17th May 2019   Reno, Nevada, US

TSG-RAN4 #92			26th – 30th Aug 2019   Ljubljana, SI