3GPP TSG-RAN WG4 Meeting #90Bis                                                             R4-1902920

# Xi’an, China, April 8th – 12th, 2019

Agenda item:	6.1.1.1

Source: 	Intel Corporation

Title:	Configured output power for inter-band NE-DC

Document for:	Approval

# Introduction

The inter-band EN-DC configured transmitted power was completed in 38.101-3, but inter-band NE-DC configured transmitted power is still not complete. In this contribution, the configured transmitted power for inter-band NE-DC is specified.

# Discussion

7.6.1A	NE-DC

If a UE is configured with a MCG using NR radio access and with a SCG using E-UTRA radio access, the UE is configured a maximum power  for transmissions in FR1 on the MCG by p-NR and a maximum power  for transmissions on the SCG by p-MaxEUTRA. The UE determines transmission power for the MCG in FR1 as described Subclauses 7.1 through 7.5 using  as the maximum transmission power for . The UE determines transmission power for the MCG in FR2 as described Subclauses 7.1 through 7.5.

If the UE is not provided TDD-UL-DL-ConfigurationCommon for the MCG, the UE determines a transmission power for the SCG as described in [13, TS 36.213] using  as the maximum transmission power.

If at least one symbol of slot  of the MCG that is indicated as uplink or flexible by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated overlaps with subframe  of the SCG

-	for subframe , the UE determines a transmission power for the SCG as described in [13, TS 36.213] using  as the maximum transmission power

otherwise

-	the UE determines a transmission power for the SCG as described in [13, TS 36.213] using  as the maximum transmission power

If a UE is configured with , where  is the linear value of ,  is the linear value of , and  is the linear value of a configured maximum transmission power for NE-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the MCG as follows

-	If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 in [13, TS 36.213])

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE does not expect to transmit in a slot on the MCG in FR1 when a corresponding subframe on the SCG is an UL subframe in the reference TDD configuration.

-	If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for NE-DC and

-	if the UE transmission(s) in slot  of the MCG in FR1 overlap in time with UE transmission(s) in subframe  of the SCG, and

-	if  in any portion of slot  of the MCG,

the UE reduces transmission power in any portion of slot  of the MCG so that  in all portions of slot , where  and  are the linear values of the total UE transmission powers in slot  of the MCG in FR1 and in subframe  of the SCG, respectively.

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE expects to be configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 in [13, TS 36.213]).

NE-DC power control requirement was first (and last) introduced in RAN1 spec 38.213 in version of V15.4.0 (2018-12) for the completeness of RAN1 Rel-15 specifications.  The following is the excerpt of NE-DC requirement from 38.213.

At the same time, RAN1 sent an LS [1] to RAN4 regarding NE-DC power control.

There

RAN1 made the following agreements on power control for NR-LTE dual connectivity in RAN1#95, and would like RAN4 and RAN2 to take them into consideration in their future work.

“Agreement:

- A UE configured with NE-DC is configured with P_LTE for LTE and with P_NR for NR

- LTE Power limit is set as follows

  - For an LTE subframe that overlaps with any possible NR UL symbol(s)

    - Set LTE power limit Pcmax<=P_LTE

  - Otherwise

    - Set LTE power limit Pcmax without considering P_LTE

- A possible NR UL symbol is identified as an NR symbol configured as flexible or UL based on cell-specific or UE-specific (if configured) tdd_UL_DL_Configuration_Common/dedicated.

- NR power limit is set as Pcmax <= P_NR in all slots

- When there is simultaneous transmission for LTE and NR with actual power p_lte_power and p_nr_power respectively, such that p_lte_power + p_nr_power > X_total, UE will scale down p_nr_power until the total transmit power does not exceed X_total

  - X_total is computed based on P_LTE, P_NR

- Note:

  - At least from RAN1 perspective, there will be no change to other mechanisms (i.e., Power class, p-UE-FR1, Pemax) specified in RAN4 to limit UE maximum transmit power

  - Specification details  of LTE power limit, NR power limit and X_total will be handled by RAN4”

are two major differences in NE-DC comparing with EN-DC:

- In NE-DC, LTE transmission power may not be limited by P_LTE if LTE transmission is predicted at non-overlapping transmission period with NR specified by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated.

- There is no XSCALE defined.

For the first difference, it is our understanding that it implies in NE-DC, UE knows when it is going to be scheduled the dedicated LTE transmissions since configuration TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated is signaled in advance and if signaled, UEs regardless whether supporting dynamic power sharing or not should have same behavior to transmit LTE without considering P_LTE limit in Pcmax.

For the second difference, since no XSCALE defined, any amount of scaling applied by UE can be valid if is met after scaling. That means NR can be dropped to meet condition. In order to have a way to do the conformance test, we propose to take the same XSCALE currently defined for EN-DC and take same ‘a’ and ‘b’ conditions to specify PCMAX_ NE-DC _L(p,q) as how PCMAX_ EN-DC _L(p,q) is specified

Proposal: Take same approach of specifying PCMAX_ EN-DC _L(p,q) by XSCALE in NE-DC power control.

We have proposed text for configured transmitted power for inter-band NE-DC in the appendix based on inter-band EN-DC spec. The relative changes are tracked in order to indentify the difference.

# Conclusion

In this contribution, we provide proposed text for configured transmitted power for inter-band NE-DC and the following proposal.

Proposal: Take same approach of specifying PCMAX_ EN-DC _L(p,q) by XSCALE in NE-DC power control.

# References

- R1-1814320, “LS on power control for NR-LTE Dual Connectivity”, RAN1#95, Spokane, WA, USA

# Appendix

< start of changes >

### 6.2B.6	Configured output power for NE-DC

#### 6.2B.6.1	Configured output power level

##### 6.2B.6.1.1	Intra-band contiguous NE-DC

##### 6.2B.6.1.2	Intra-band non-contiguous NE-DC

##### 6.2B.6.1.3	Inter-band NE-DC within FR1

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission  power for NE-DC operation, PTotalENNE-DC.

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L_ E-UTRA,c (p) ≤  PCMAX_ E-UTRA,c (p) ≤  PCMAX H _ E-UTRA,c (p)

where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in TS 36.101 [4] sub-clause 6.2.5 modified by PLTE as follows:

PCMAX_L_ E-UTRA,c = MIN { PEMAX, NE-DC , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), MIN(PEMAX,c , PLTE) – tC_ E-UTRA, c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PLTE, PPowerClass – ΔPPowerClass}

except that when uplink E_UTRA transmission is scheduled with non-overlapping with NR uplink transmisions:

PCMAX_L_ E-UTRA,c = MIN { PEMAX, NE-DC , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PEMAX,c  – tC_ E-UTRA, c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

The configured maximum output power PCMAX_ NR,c (q) in physical-channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L_ NR,c and PCMAX H _ NR,c are the limits for a serving cell c as specified in sub-clause 6.2.4 of TS 38.101-1 [2] modified by PNR as follows:

PCMAX_L,f,c,,NR = MIN { PEMAX, NE-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), MIN(PEMAX,c , PNR ) - TC_NR, c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, NE-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PNR , PPowerClass – ΔPPowerClass }

-	PLTE signalled by RRC as p-MaxEUTRA in [36.331]

-	PNR signalled by RRC as p-NR-FR1 defined in [38.331]

-	ΔTc_E-UTRA, c = 1.5dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [4] applies for a serving cell c, otherwise TC_ E-UTRA,c = 0dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0dB;

-	ΔTIB,c specified in sub-clause  6.2B.4.2 for EN-DC;

-	ΔPPowerClass,NE-DC = 3 dB for a power class 2 capable NE-DC UE when  LTE UL/DL configuration is 0 or 6; or LTE UL/DL configuration is 1 and special subframe configuration is 0 or 5; or the IE p-maxUE-FR1 as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower; otherwise ΔPPowerClass,NE-DC = 0 dB;

If the transmissions from NR and E-UTRA do not overlap, then the complete sub-clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, EN-DC or PEMAX, EN-DC shall not be exceeded at any time by UE.

The total configured maximum transmission power for both synchronous and non-synchronous operation is

P_NE-DC_Total = MIN { PEMAX, NE-DC ,PPowerClass, NE-DC – ΔPPowerClass, NE-DC }

P_NE-DC_Total is the dB value of  PTotalENE-DC, which is used in [38.213] and PEMAX, NE-DC is  p-maxUE-FR1-r15 value signaled by RRC and defined in [36.331];

If the UE does not support dynamic power sharing,

P_NE-DC_Total = MIN { PEMAX, NE-DC ,PPowerClass, NE-DC  – ΔPPowerClass, NE-DC } + 0.3 dB

If the NE-DC UE does not support dynamic power sharing, then the complete sub-clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [4] and TS 38.101-1 [2] respectively apply with the modifications specified above and P_NE-DC_Total applies.

When a UE supporting dynamic sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions, the UE can set its configured maximum output power PCMAX_ E-UTRA,c and PCMAX_ NR,c for the configured E-UTRA and NR uplink carriers, respectively, and its configured maximum transmission power for EN-DC operation, PTotalENE-DC, as specified above.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_H) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.6.1.3-2.

When an UL subframe transmission p from E-UTRA overlap with a physical-channel q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.6.1.3-1 when same or different subframe and physical-channel durations are used in aggregated carriers. PPowerClass ,NE-DC shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.6.1.3-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | LTE Subframe | Min(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ NE-DC _H (p,q) , PCMAX_ NE-DC _H (p,q+1), … , PCMAX_ NE-DC _H (p,q+n) }

where PCMAX_ NE-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with LTE subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_ NE-DC _L (p,q) , PCMAX_ NE-DC _L (p,q+1), … , PCMAX_ NE-DC _L (p,q+n)}

where PCMAX_NE-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with LTE subframe p,

With

PCMAX_ NE-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR c(q)], PEMAX, NE-DC ,PPowerClass, NE-DC}

And:

a= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > P_NE-DC_Total

b= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) /X_scale] > P_NE-DC_Total

If a= FALSE

PCMAX_ NE-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,,NR c(q)], PEMAX, NE-DC ,PPowerClass, NE-DC}

ELSE If (a=TRUE) AND (b=FALSE)

PCMAX_ NE-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,,NR c(q) /X_scale ], PEMAX, NE-DC ,PPowerClass, NE-DC}

ELSE If b= TRUE

PCMAX_ NE-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) ], PEMAX, NE-DC ,PPowerClass, NE-DC}

where

-	pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

-	pCMAX H _ NR,c (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

-	pCMAX L _ NR,c(q) is the NR lower limit of the maximum configured power expressed in linear scale;

-	PPowerClass, NE-DC is defined in sub-clause 6.x.x for inter-band NE-DC;

-	X_scale is the linear value of X dB which is configured by RRC and can only take values [0 , 6]

-	pCMAX_ E-UTRA,c (p) is the linear value of PCMAX_ E-UTRA,c (p), the real configured max power for LTE

-	pCMAX,f,c,NR (q) is the linear value of PCMAX,f,c,NR (q), the real configured max power of NR

Table 6.2B.6.1.3-2: PCMAX tolerance for Dual Connectivity LTE-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | [3.0] | [2.0] |
| 22 ≤ PCMAX < 23 | [5.0] | [2.0] |
| 21 ≤ PCMAX< 22 | [5.0] | [3.0] |
| 20 ≤ PCMAX < 21 | [6.0] | [4.0] |
| 16 ≤ PCMAX < 20 | [5.0] |  |
| 11 ≤ PCMAX < 16 | [6.0] |  |
| -40 ≤ PCMAX < 11 | [7.0] |  |
| NOTE 1: For UEs not indicating support of dynamic power sharing, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

When LTE and NR transmissions overlap and the condition (If (a=TRUE) AND (b=FALSE)) is met, MCG shall be transmitted and the following supplementary minimum requirement apply for the measured MCG power, PUMAX,f,c,NR (q), under nominal conditions.

10log(pCMAX L,f,c,,NR c(q)/X_scale)  –  TLOW (10log(pCMAX L,f,c,,NR c(q)/X_scale) )}  ≤  PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,,NR c (q)) + THIGH (10log(pCMAX H, f,c,,NR c (q))).

with the tolerances TLOW and THIGH for applicable values of PCMAX specified in Table 6.2B.6.1.3-2.

< end of changes >