3GPP TSG-RAN WG4 Meeting #90Bis	R4-1902923

# Xi’an, China, Apr. 8th – 12th, 2019

Agenda item:	6.5.11.7.2

Source: 	Intel Corporation

Title: 	On beam Correspondence tolerance requirements

Document for:	Approval

# Introduction

In last RAN4 meeting, the frameworks of beam correspondence requirements for PC3 in 38.101-2 and beam correspondence test procedure 38.810 were largely complete, but the key performance part of beam correspondence tolerance requirements were still open. A WF [1] on simulation assumptions to derive beam correspondence tolerance requirements was agreed in meeting. In this contribution, we present our simulation results based on WF.

# Discussion

Two simulation models were provided in WF. The fundamental difference between these two models is how to apply the impairments. The first model (model 1) applies phase error and amplitude error on each antenna element while second model (model 2) applies phase error (aka beam steering error) and amplitude error on each beam.

Model 1: Phase/Amplitude errors per antenna element

Model 2: Phase/Amplitude errors per beam

## Simulation results

Our performed simulations were based on model 2 since phase error (beam steering error) and amplitude gain error are readily measureable at beam level instead of antenna element level.  Extracting beam level impairment characteristics is more accurate to reflect final measurement results.

Our simulation assumption was based on 2 panels with front and back mounting positions and with glass cover on each side. Each panel has 13 beams, total beams over full sphere is 26.   The effort of the simulations to cover all possible combinations of impairment values with parameters like K and M is significant, we only selected a subset of values under consideration. We expect delta CDF curve is also a function of M, the number of SRS opportunities [2]. We provide the following three plots to compare delta EIRP CDF curves with the different values of M. Within each plot, multiple curves corresponding to different sets of impairments are drawn for comparison.

Simulation 1 – Figure 1

Number of total beam K=26 over two panels (each panel has 13 beams)

UL beam sweeping SRS resource M=4

Measurement Grid: 15o

Figure 1 CDF of delta EIRP with number of SRS resources M=4

Simulation 2 – Figure 2

Number of total beam K=26 over two panels (each panel has 13beams)

UL beam sweeping SRS resource M=8

Measurement Grid: 15o

Figure 2 CDF of delta EIRP with number of SRS resources M=8

Simulation 3 – Figure 3

Number of total beam K=26 over two panels (each panel has 13beams)

UL beam sweeping SRS resource M=13

Measurement Grid: 15o

Figure 3 CDF of delta EIRP with number of SRS resources M=13

The following table is a summary of simulations. σP represents sigma value of beam phase error (beam steering error) in degree, σG represents  sigma value of beam gain error in dB and σRSRP represents RSRP estimation error in dB.

Table 1 Delta EIRP vs. different M values and impairments

| Impairments (σP, σG, σRSRP) | Delta EIRP (Y) = EIRP2 – EIRP1 in dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | M=4 |  | M=8 |  | M=13 |  |
|  | X = 80%-tile | X = 90%-tile | X = 80%-tile | X = 90%-tile | X = 80%-tile | X = 90%-tile |
| (45,3,2) | 8.7 | 11.5 | 10.5 | 13.2 | 11.4 | 14.2 |
| (10,3,2) | 5.5 | 7.7 | 6.4 | 8.5 | 7.4 | 9.4 |
| (10,2,2) | 4.1 | 5.8 | 5.0 | 6.8 | 5.6 | 7.3 |
| (5,2,2) | 3.3 | 4.8 | 3.8 | 5.2 | 4.5 | 6.0 |

In model 2, the curve with Sigma_steering error = 45o is worst among others. It generates the largest Delta EIRP values. This is because with such large beam steering error, the selected corresponding Tx beam actually has quite different steering direction with the best Rx beam (with best RSRP). This leads to very low EIRP1 at given DL test direction. While with UL sweeping, the better EIRP2 can still be achieved. In this case, large Delta EIRP is expected. For practical beam correspondence design, we expect the beam steering error is likely to be controlled to a smaller sigma value.

The EIRP at UE antenna input should be calculated as

Rx_EIRP = EIS50% + SNR - SNRREFENSES

Where

Rx_EIRP - Required EIRP power level at UE antenna

EIS50% - 50%-tile EIS spherical coverage value for PC3 UE

SNR   - Required receive SNR

SNRREFENSES = -1dB, baseband SNR used for REFSENS testing

We also notice that the RSRP measurement errors cannot be assumed to follow the same distributions to all test grid points since Rx beam gains at different test grid points are different. For given fixed EIRP level at UE antenna input, the SNRs are different at different test grid points, in turn, the RSRP estimation error. In our paper [3], we proposed that

to make sure SNR at top of 50%-tile EIS spherical coverage to be maintained high enough to have reasonable RSRP estimation error.

Based on the simulations performed, at given X%-tile, delta EIRP Y values varies within a wide range depending on values of impairments and also values of M and among others, like UE packaging. We propose the following beam correspondence tolerance requirements as a compromised by weighing all factors.

Proposal 1: SRS resource M should be greater than or equal to 8.

Proposal 2: For power class 3 UEs in single FR2 band with beam correspondence capability signaling set to 0, the delta EIRP CDF on selected test points defined in 38.101-2 [4] shall satisfy the following condition with number of SRS resource M≥8:

  - 80-percentile of delta EIRP CDF is no more than [4.5] dB

# Conclusion

In this contribution, we provide our simulation results. Based on simulations and consideration of practical beam correspondence design imperfections, we have following proposals:

Proposal 1: SRS resource M should be greater than or equal to 8.

Proposal 2: For power class 3 UEs in single FR2 band with beam correspondence capability signaling set to 0, the delta EIRP CDF on selected test points defined in 38.101-2 [4] shall satisfy the following condition with number of SRS resource M≥8:

  - 80-percentile of delta EIRP CDF is no more than [4.5] dB

# References

- R4-1902683, “Draft CR to 38.810 on beam correspondence test procedure”, Samsung, Qualcomm, Apple, RAN4#90, Athens, Greece.

- R4-1900134, “on beam correspondence requirements”, Intel Corp., RAN4#90, Athens, Greece.

- R4-1900133, “on beam correspondence test procedure”, Intel Corp., RAN4#90, Athens, Greece.

- 38.101-2, V15.5.0