3GPP TSG RAN WG4 #90-bis	R4-1902964

# Xi’an, China, 8 Apr 2019 - 12 Apr, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | CRNum | rev |  | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on TDD UL-DL Configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-04-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TDD UL-DL configurations for dynamic UL/DL detection are not present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TDD UL-DL configurations for dynamic UL/DL detection were specified. Also, a typo was fixed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for dynamic UL/DL detection will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-4 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

## A.1.2	TDD UL-DL patterns for FR1

TDD UL-DL patterns configurations for performance requirements are provided in Tables A.1.2-1, A.1.2-2, and A.1.2-3.

Table A.1.2-1: TDD UL-DL pattern for SCS 15 kHz

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
|  |  |  | FR1.15-1 |
| TDD Slot Configuration pattern (Note 1) |  |  | DDDSU |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 15 |
|  | dl-UL-TransmissionPeriodicity | ms | 5 |
|  | nrofDownlinkSlots |  | 3 |
|  | nrofDownlinkSymbols |  | 10 |
|  | nrofUplinkSlot |  | 1 |
|  | nrofUplinkSymbols |  | 2 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [4] if mod(I,5) = 0 [3] if mod(i,5) = 1 [2] if mod(i,5) = 2 [6] if mod(i,5) = 3 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,9} |  |  |  |

Table A.1.2-2: TDD UL-DL pattern for SCS 30 kHz

| Parameter |  | Unit | UL-DL pattern |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1.30-1 | FR1.30-2 | FR1.30-3 | FR1.30-4 | FR1.30-5 | FR1.30-6 |
| TDD Slot Configuration pattern (Note 1) |  |  | 7DS2U | DDDSU | DDDSUDDSUU | SU | DDSU | DS1S2U |
| Special Slot Configuration (Note 2) |  |  | 6D+4G+4U | 10D+2G+2U | 10D+2G+2U | 12D+2G+0U | 10D+2G+2U | S1: 10D+2G+2U S2: 12D+2G+0U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 30 | 30 | 30 | 30 | 30 | 30 |
|  | dl-UL-TransmissionPeriodicity | ms | 5 | 2.5 | 2.5 | 1 | 2 | 1 |
|  | nrofDownlinkSlots |  | 7 | 3 | 3 | 0 | 2 | 1 |
|  | nrofDownlinkSymbols |  | 6 | 10 | 10 | 12 | 10 | 10 |
|  | nrofUplinkSlot |  | 2 | 1 | 1 | 1 | 1 | 0 |
|  | nrofUplinkSymbols |  | 4 | 2 | 2 | 0 | 2 | 2 |
| UL-DL configuration2 (tdd-UL-DL-ConfigurationCommon2) | referenceSubcarrierSpacing | kHz | N/A | N/A | 30 | N/A | N/A | 30 |
|  | dl-UL-TransmissionPeriodicity | ms | N/A | N/A | 2.5 | N/A | N/A | 1 |
|  | nrofDownlinkSlots |  | N/A | N/A | 2 | N/A | N/A | 0 |
|  | nrofDownlinkSymbols |  | N/A | N/A | 10 | N/A | N/A | 12 |
|  | nrofUplinkSlot |  | N/A | N/A | 2 | N/A | N/A | 1 |
|  | nrofUplinkSymbols |  | N/A | N/A | 2 | N/A | N/A | 0 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [7] if mod(i,10) = 0 [6] if mod(i,10) = 1 [5] if mod(i,10) = 2 [5] if mod(i,10) = 3 [4] if mod(i,10) = 4 [3] if mod(i,10) = 5 [3] if mod(i,10) = 6 [2] if mod(i,10) = 7 | [4] if mod(i,5) = 0 [3] if mod(i,5) = 1 [2] if mod(i,5) = 2 [6] if mod(i,5) = 3 | [4] if mod(i,10) = 0 [3] if mod(i,10) = 1 [2] if mod(i,10) = 2 [5] if mod(i,10) = 3 [3] if mod(i,10) = 5 [3] if mod(i,10) = 6 [2] if mod(i,10) = 7 | [3] if mod(i,2) = 0 | [3] if mod(i,4) = 0 [2] if mod(i,4) = 1 [5] if mod(i,4) = 3 | [3] if mod(i,4) = 0 [2] if mod(i,4) = 1 [3] if mod(i,4) = 3 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,19} |  |  |  |  |  |  |  |  |

Table A.1.2-2a: TDD UL-DL pattern for SCS 30 kHz for DCI-based dynamic UL/DL detection

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
|  |  |  |  |  |  | FR1.30-1.1 |  |  |
| TDD Slot Configuration pattern (Note 1) |  |  |  |  |  | 7DS2U |  |  |
| Special Slot Configuration (Note 2) |  |  |  |  |  | 6D+4G+4U |  |  |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) (Note 4) | referenceSubcarrierSpacing |  |  | kHz |  | N/A |  |  |
|  | dl-UL-TransmissionPeriodicity |  |  | ms |  | N/A |  |  |
|  | nrofDownlinkSlots |  |  |  |  | N/A |  |  |
|  | nrofDownlinkSymbols |  |  |  |  | N/A |  |  |
|  | nrofUplinkSlot |  |  |  |  | N/A |  |  |
|  | nrofUplinkSymbols |  |  |  |  | N/A |  |  |
| UL-DL configuration2 (tdd-UL-DL-ConfigurationCommon2) (Note 4) | referenceSubcarrierSpacing |  |  | kHz |  | N/A |  |  |
|  | dl-UL-TransmissionPeriodicity |  |  | ms |  | N/A |  |  |
|  | nrofDownlinkSlots |  |  |  |  | N/A |  |  |
|  | nrofDownlinkSymbols |  |  |  |  | N/A |  |  |
|  | nrofUplinkSlot |  |  |  |  | N/A |  |  |
|  | nrofUplinkSymbols |  |  |  |  | N/A |  |  |
| PDCCH DCI Configuration | DCI Format |  |  |  |  | 1-1 for slot indices with mod(i,10) = 0,1,2,3,4,5,6,7 |
|  | Scheduled Grant |  | Symbol 2-13 for slot indices with mod(i,10) = 0,1,2,3,4,5,6 and Symbol 2-5 for slot indices with mod(i,10) = 7 |
| PUCCH Configuration | format |  | PUCCH-format-3 for slot indices with mod(i,10) = 0,1,2,3,4,5 and PUCCH-format-1 for slot indices with mod(i,10) = 6,7 |
|  | nrofSymbols |  | 4 |
|  | startingSymbolIndex |  | 10 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  |  |  | [7] if mod(i,10) = 0 [6] if mod(i,10) = 1 [5] if mod(i,10) = 2 [5] if mod(i,10) = 3 [4] if mod(i,10) = 4 [3] if mod(i,10) = 5 [3] if mod(i,10) = 6 [2] if mod(i,10) = 7 |  |  |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,19} Note 4: Do not configure TDD UL-DL pattern semi-statically using RRC configuration. |  |  |  |  |  |  |  |  |

## A.1.3	TDD UL-DL patterns for FR2

TDD UL-DL patterns configurations for performance requirements are provided in Tables A.1.3-1, A.1.3-2.

Table A.1.3-1: TDD UL-DL pattern for SCS 60 kHz

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
|  |  |  | FR2.60-1 |
| TDD Slot Configuration pattern (Note 1) |  |  | DDSU |
| Special Slot Configuration (Note 2) |  |  | 11D+3G+0U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 60 |
|  | dl-UL-TransmissionPeriodicity | ms | 1 |
|  | nrofDownlinkSlots |  | 2 |
|  | nrofDownlinkSymbols |  | 11 |
|  | nrofUplinkSlot |  | 1 |
|  | nrofUplinkSymbols |  | 0 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | K1 = [3] if mod(i,4) = 0 K1 = [2] if mod(i,4) = 1 K1 = [5] if mod(i,4) = 2 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,39} |  |  |  |

Table A.1.3-2: TDD UL-DL pattern for SCS 120 kHz

| Parameter |  | Unit | UL-DL pattern |  |
| --- | --- | --- | --- | --- |
|  |  |  | FR2.120-1 | FR2.120-2 |
| TDD Slot Configuration pattern (Note 1) |  |  | DDDSU | DDSU |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U | 11D+3G+0U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 120 | 120 |
|  | dl-UL-TransmissionPeriodicity | ms | 0.625 | 0.5 |
|  | nrofDownlinkSlots |  | 3 | 2 |
|  | nrofDownlinkSymbols |  | 10 | 11 |
|  | nrofUplinkSlot |  | 1 | 1 |
|  | nrofUplinkSymbols |  | 2 | 0 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | K1 = [4] if mod(i,5) = 0 K1 = [3] if mod(i,5) = 1 K1 = [2] if mod(i,5) = 2 K1 = [6] if mod(i,5) = 3 | K1 = [3] if mod(i,4) = 0 K1 = [2] if mod(i,4) = 1 K1 = [5] if mod(i,4) = 2 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,79} |  |  |  |  |

Table A.1.3-2a: TDD UL-DL pattern for SCS 120 kHz for DCI-based dynamic UL/DL detection

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
|  |  |  | FR2.120-1.1 |
| TDD Slot Configuration pattern (Note 1) |  |  | DDDSU |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) (Note 4) | referenceSubcarrierSpacing | kHz | N/A |
|  | dl-UL-TransmissionPeriodicity | ms | N/A |
|  | nrofDownlinkSlots |  | N/A |
|  | nrofDownlinkSymbols |  | N/A |
|  | nrofUplinkSlot |  | N/A |
|  | nrofUplinkSymbols |  | N/A |
| UL-DL configuration2 (tdd-UL-DL-ConfigurationCommon2) (Note 4) | referenceSubcarrierSpacing | kHz | N/A |
|  | dl-UL-TransmissionPeriodicity | ms | N/A |
|  | nrofDownlinkSlots |  | N/A |
|  | nrofDownlinkSymbols |  | N/A |
|  | nrofUplinkSlot |  | N/A |
|  | nrofUplinkSymbols |  | N/A |
| PDCCH DCI Configuration | DCI Format |  | 1-1 for slot indices with mod(i,5) = 0,1,2,3 |
|  | Scheduled Grant |  | Symbol 1-13 for slot indices with mod(i,5) = 0,1,2 and Symbol 1-9 for slot indices with mod(i,5) = 3 |
| PUCCH Configuration | format |  | PUCCH-format-3 for slot indices with mod(i,10) = 0,1,2,3 |
|  | nrofSymbols |  | 4 |
|  | startingSymbolIndex |  | 10 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | K1 = [4] if mod(i,5) = 0 K1 = [3] if mod(i,5) = 1 K1 = [2] if mod(i,5) = 2 K1 = [6] if mod(i,5) = 3 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,79} Note 4: Do not configure TDD UL-DL pattern semi-statically using RRC configuration. |  |  |  |

<< End of changes >>