3GPP TSG RAN WG4 Meeting #90bis		R4-1902968

Xi'an, China, 8 - 12 April 2019

Agenda item:	9.19.1

Source:	SoftBank Corp.

Title:	Considerations on LTE/NR spectrum sharing in n41

Document for:	Approval

1	Introduction

Discussion on LTE/NR sharing in n41 was kicked-off in the last RAN4 meeting and WF was approved [1]. This paper is to clarify some aspects in the WF.

2	Meaning of "solving non-backward compatibility issue"

The WF mentioned that, for considering n41 modification to support spectrum sharing, "Non-backwards compatibility issue of Rel-15 n41 UE without UL 7.5kHz frequency shift mentioned in [Huawei: 654] and [KDDI:571] is the key issue to choose solution#2 ." Note that "Solution#2" means to reuse n41 for this sharing operation.

While not always explicit, judging from [1], the intentions of the WF are:

1) REL-15 n41 UE does not have to operate under a NW enabling UL 7.5k shift since a note in page 3 mentions that "Applying UL 7.5kHz frequency shift to Rel-15 n41 UE was not approved in RAN4#89", and

2) However, harmful behaviour of REL-15 UE shall be avoided. The harmful behaviour here is to send UL without 7.5k shift that will cause interference to shifted UL due to the lack of orthogonality,

[Proposal-1] The sentences in bold letters above are confirmed as consensus of the group.

In this context, for REL-15 UEs, Alternative #1 (adding "frequencyShift7p5khz" IE to REL-15, probably more precisely, changing applicability of the IE not only for FDD but also for n41) is to instruct not to transmit random access messages even if the UE finds an SSB. While it is straightforward, the fundamental issue is that some UEs, esp. to be deployed in early phase, could not comply with the rule.

Alternative #2 is to introduce additional SSB blocks not supported by REL-15 UE so that REL-15 UE cannot detect a cell as long as the additional SSB is used for UL 7.5k shift operation.

3	Consideration on Alternative #2

Alternative #2 is as below [1]:

To solve the compatibility issue of sulotion#2, verify feasibility of the following alternative plans Alternative#2: deploy SSB of spectrum sharing NW where Rel-15 UE cannot find it. For spectrum sharing, we need to enable repetition of sync raster (M=1,3,5) in Rel-16 which has more sync raster entries compared to Rel-15. - Feasibility of this proposal needs to be further verified and it will make constrains on operator’s NW deployment.

In the current spec, sync raster positions for < 3GHz are defined as 1200*N+50*M[kHz]{M=1,3,5} and for bands with SCS-based channel raster including n41, only M=3 is applied. The three shifts (M=1, 3, 5) correspond to the residues of (channel raster frequency/3) in 100kHz channel raster to accommodate 100k/15k incompatibility.

Based on the definition above, sync raster points derived from M=3 alone are detectable from REL-15 UE and the points correspond to 100k channel raster frequencies with multiple of 3. Then the Alternative #2 will put a limitation to prohibit the channel raster points of 2496000+300*p [kHz] from operating with UL 7.5k shift under 100k channel raster scheme (p: positive integer).

[Observation-1] Alternative #2 requests that, under 100k channel raster scheme, the channel raster points of 2496000+300*p [kHz] should be excluded from UL 7.5k shift operation.

To support UL 7.5k shift for a CC (Component Carrier) corresponding to a prohibited channel raster, a possible remedy is to shift the center frequency of the CC by 100kHz upward or downward. For alignment between LTE and NR subcarriers, the center frequency of LTE CC would also be shifted by the same amount. But this is not always possible: in a single LTE CC for example, the shift would violate regulation such as SEM. Then this solution seems only likely when two (or more) adjacent LTE CCs are occupied by an operator and it is possible to shift a CC "inward" or toward harmless direction. In addition, if LTE intra-band contiguous CA has already been introduced in these CCs, shifting possibility would be limited to guarantee 300kHz*n spacing required for the CA. So,

[Observation-2] To support UL 7.5k shift corresponding to a prohibited channel raster, shifting channel raster position by 100kHz can be considered but applicable scenarios are limited, largely depending on operator's CC occupancy.

Thus, alternative #2 looks feasible but with constraints, i.e., one third of 100k channel raster points should be excluded from LTE/NR sharing operation, or rearranged if feasible.

4	Conclusion

This paper is intended to confirm group's understanding of the WF and address some issues around Alternative #2. Further discussion is needed to finalise the scheme to be adopted.

Reference

R4-1902450		WF on LTE/NR spectrum sharing in Band 41/n41, KDDI, CMCC, Sprint