3GPP TSG RAN WG4 Meeting #90bis	R4- 1903174

Xi’an, China, April 8 – 12, 2019

Agenda Item:	8.1

Source:	Huawei, HiSilicon

Title:	Spectrum utilization improvement in unlicensed bands

Document for:	Discussion and decision

Introduction

In RAN4 #90 meeting [1], the following agreement on spectrum utilization for unlicensed bands has been achieved:

Agreement:

Spectrum utilization for NR-U for 20MHz with 15KHz SCS and 30KHz SCS are agreed as:

|  | 20 MHz |
| --- | --- |
| 15 kHz | 106 |
| 30 kHz | 51 |

The spectrum utilization for 20MHz with 60KHz SCS is still an open issue. In this contribution, we study the achievable spectrum utilization value for 20MHz with 60KHz SCS, considering the relevant RF requirements.

Spectrum utilization for 20MHz with 60KHz SCS

The transmission bandwidth configuration NRB for different channel bandwidth and subcarrier spacing is specified in R15 as following [2]:

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N.A | N.A | N.A | N.A | N.A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N.A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 121 | 135 |

If the transmission bandwidths follow the same configurations as NR R15, the corresponding spectrum utilization should be:

Table 1: SU for 20MHz bandwidth in R15

| SCS [kHz] | 20 MHz |
| --- | --- |
|  | NRB (SU) |
| 15 | 106 (95.4%) |
| 30 | 51 (91.8%) |
| 60 | 24 (86.4%) |

It can be found that the spectrum utilization(SU) of 60 kHz SCS with available 24 PRBs is about 10% less than that of 15 kHz SCS and 5% less than that of 30 kHz in 20MHz bandwidth. Even a dot11ax station has a higher spectrum utilization (94.5%) with 78.125 kHz SCS in 20MHz bandwidth. It is reasonable to increase the number of available to improve the SU without violating the RF requirement, such as out of band (OOB) emission, ACLR, EVM, etc.

RF requirement consideration

Out of band emission

In figure 1 and 2, we evaluate both DL and UL out of band emission for 20MHz carrier bandwidth assuming increased number of available RBs. In our simulation, the Rapp PA model assuming 23dBm total output power and 34.6 dBm saturation power for DL, and polynomial PA model for UL are used.

Note that there is no additional spectrum mask defined for unlicensed band in current stage, the spectrum mask for band 46 defined in LTE could be considered as a reference. Besides, the general spectrum mask is also considered. It is observed that increasing the number of available RBs to 26 in 20MHz carrier bandwidth does not violate the transmitter spectrum mask, regardless of the spectrum mask in band 46 or the general spectrum mask.

Observation 1: For 20MHz bandwidth with 60 kHz SCS, increasing the number of available RBs to 26 does not violate transmitter spectrum mask, and is beneficial for the interlace design in NR-U.

Figure 1. DL Operating band unwanted emissions of 60 kHz SCS in 20MHz with 26 usable RBs

Figure 2. UL Operating band unwanted emissions of 60 kHz SCS in 20MHz with 26 usable RBs

ACLR

For operation in Band 46, the ACLR shall be higher than the value specified in 36.104[4]:

Table 2: Base Station ACLR in Band 46

| Channel bandwidth of E-UTRA lowest/highest carrier transmitted BWChannel [MHz] | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 10, 20 | BWChannel | E-UTRA of same BW | Square (BWConfig) | 35 dB |
|  | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 40 dB |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA lowest/highest carrier transmitted on the assigned channel frequency. |  |  |  |  |

The ACLR evaluation for 20MHz with 26PRB is provide as following. In the simulation, both windowing and filtering are evaluated and the parameters applied are:

- Window length: 2% OFDM symbol duration

- Filter length: 127 order

It can be found that increasing number of available RBs to 26 does not violate the ACLR requirement, no matter for widowing or filtering.

|  | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | ACLR（dB） (26PRB) | ACLR（dB） (24PRB) |
| --- | --- | --- | --- |
| W-OFDM | BWChannel | 49.62 | 58.24 |
|  | 2 x BWChannel | 75.95 | 78.11 |
| F-OFDM | BWChannel | 57.66 | 58.88 |
|  | 2 x BWChannel | 76.02 | 78.18 |

EVM

In R15, EVM requirements is provides in 38.104 as following:

Table 3: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |

The following table lists the comparison of EVM performance (64QAM) for 20MHz with 24 PRBs and 26 PRBs for baseband signal, it can be found that for W-OFDM, the impact on the EVM by increasing the PRB number is slight, and both W-OFDM and F-OFDM could meet the EVM requirement.

|  | 24PRB avg EVM [%] | 26PRB avg EVM [%] |
| --- | --- | --- |
| W-OFDM | 0.26 | 0.27 |
| F-OFDM | 1.34 | 1.28 |

Proposal 1:  RAN4 should consider to increase number of PRBs (e.g. 26 PRBs) for 20MHz carrier bandwidth configured with 60 kHz SCS to improve the spectrum utilization.

Conclusions

We discussed spectrum utilization for NR-U in this contribution, based on our evaluation on out of band emission, ACLR and EVM performance for 20MHz bandwidth with 60kHz SCS, the following observation and proposal were made:

Observation 1: For 20MHz bandwidth with 60 kHz SCS, increasing the number of available RBs to 26 does not violate transmitter spectrum mask, and is beneficial for the interlace design in NR-U.

Proposal 1:  RAN4 should consider to increase number of PRBs (e.g. 26 PRBs) for 20MHz carrier bandwidth configured with 60 kHz SCS to improve the spectrum utilization.

- References

RAN4#90 Meeting report, Athens, Greece ,25th February – 1st March, 2019

3GPP TS 38.104 V15.3.0, “Base Station (BS) radio transmission and reception”

3GPP TS 36.101 V15.4.0, “User Equipment (UE) radio transmission and reception”

3GPP TS 36.104 V15.4.0, “Base Station (BS) radio transmission and reception”