3GPP TSG-RAN WG4 Meeting #90bis                                                            R4-1903190

Xi’an, China, 8 Apr - 12 Apr 2019

Source:	Huawei, HiSilicon

Title:	TP to TR 38.716.02-00 for CA_n79A-n258A

Agenda item:	9.2.3

Document for:	Approval

Background

This contribution provides text proposal on the NR CA band combination CA_n79A-n258A as defined in Revised WID on NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) RP-190494 [1].

Text Proposal

---Start of changes---

8.x	CA_n79-n258

8.x.1	Common for 1 band UL and 2 bands UL CA

8.x.1.1 Operating bands for CA

Table 8.x.1-1: CA band combination of band n79 + n258

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n79-n258 | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

8.x.1.2	Channel bandwidths per operating band for CA

Table 8.x.2-1: Supported bandwidths per CA band combination of band n79+n258

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n79A-n258A | - | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

8.x.1.3	Co-existence studies

Table 8.x.3-1 gives the UL 2nd, 3rd, 4th, 5th, 6th, 7th harmonic for CA_n79A-n258A. The 5th  and 6th harmonic produced by UL band n79 may fall into Band n258 DL.

Table 8.x.1.3-1: Band n79 and Band n258 UL harmonics products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency(MHz) | 4400 | 5000 | 24250 | 27500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits(MHz) | 8800 | 10000 | 48500 | 55000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits(MHz) | 13200 | 15000 | 72750 | 82500 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits(MHz) | 17600 | 20000 | 97000 | 110000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits(MHz) | 22000 | 25000 | 121250 | 137500 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6*fy_low | 6*fy_high |
| 6th harmonics frequency limits(MHz) | 26400 | 30000 | 145500 | 165000 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7*fy_low | 7*fy_high |
| 7th harmonics frequency limits(MHz) | 30800 | 35000 | 169750 | 192500 |

8.x.1.4	∆TIB and ∆RIB values

For CA_n79A-n258A, the TIB,c and RIB values are given in the tables below.

Table 8.x.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n79-n258 | n79 | 0 |
|  | n258 | 0 |

Table 8.x.4-2: ΔRIB

| Inter-band CA Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n79-n258 | n79 | 0 |
|  | n258 | 0 |

8.x.1.5	REFSENs requirements

There is no specific REFSENs requirement.

---End of changes---

Reference

[1]	RP-190494, “Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation