3GPP TSG-RAN WG4 Meeting #90b                    	   R4-1903270

Xi’an, China, Apr 8th – 12th, 2019

Agenda item:            6.12.1.4.3

Source:	MediaTek Inc.

Title:	NR CSI PMI simulation result

Document for:          Discussion

# 1	Introduction

In this paper, we present the simulation results of CSI PMI.

# 2	Discussion

Based on the simulation assumption in [1], we provide the simulation result of CSI PMI test cases. Table A.1~Table A.3 (in Appendix) are the simulation assumption for PMI. Table A.4~Table A.7 contain other detailed parameter setting in the simulation.

## 2.1 FR1 4TX

Table 2.1.1~Table 2.1.4 are the simulation results of wideband PMI test 1 and test 2 with 4TX (FR1).

Table 2.1.1 FR1 4TX Test 1 - 2RX

| SNR [dB] | Follow PMI Relative Tput | Random PMI Relative Tput | Relative Tput Ratio (Follow/Random) |
| --- | --- | --- | --- |
| -6 | 0.28 | 0.03 | 10.18 |
| -4 | 0.42 | 0.09 | 4.52 |
| -2 | 0.58 | 0.19 | 3.00 |
| 0 | 0.79 | 0.31 | 2.50 |
| 2 | 0.92 | 0.44 | 2.10 |
| 4 | 0.98 | 0.55 | 1.78 |
| 6 | 1.00 | 0.65 | 1.52 |
| 8 | 1.00 | 0.74 | 1.35 |
| 10 | 1.00 | 0.82 | 1.22 |
| 12 | 1.00 | 0.88 | 1.14 |
| 14 | 1.00 | 0.93 | 1.07 |
| 16 | 1.00 | 0.97 | 1.03 |
| 18 | 1.00 | 0.99 | 1.01 |
| 20 | 1.00 | 1.00 | 1.00 |

Table 2.1.2 FR1 4TX Test 1 - 4RX

| SNR [dB] | Follow PMI Relative Tput | Random PMI Relative Tput | Relative Tput Ratio (Follow/Random) |
| --- | --- | --- | --- |
| -6 | 0.50 | 0.13 | 3.74 |
| -4 | 0.67 | 0.25 | 2.70 |
| -2 | 0.87 | 0.37 | 2.34 |
| 0 | 0.96 | 0.50 | 1.95 |
| 2 | 1.00 | 0.61 | 1.63 |
| 4 | 1.00 | 0.71 | 1.41 |
| 6 | 1.00 | 0.79 | 1.26 |
| 8 | 1.00 | 0.86 | 1.17 |
| 10 | 1.00 | 0.91 | 1.09 |
| 12 | 1.00 | 0.96 | 1.04 |
| 14 | 1.00 | 0.99 | 1.01 |
| 16 | 1.00 | 1.00 | 1.00 |
| 18 | 1.00 | 1.00 | 1.00 |
| 20 | 1.00 | 1.00 | 1.00 |

Table 2.1.3 FR1 4TX Test 2 - 2RX

| SNR [dB] | Follow PMI Relative Tput | Random PMI Relative Tput | Relative Tput Ratio (Follow/Random) |
| --- | --- | --- | --- |
| -6 | 0.28 | 0.02 | 11.36 |
| -4 | 0.42 | 0.09 | 4.77 |
| -2 | 0.58 | 0.20 | 2.92 |
| 0 | 0.77 | 0.32 | 2.43 |
| 2 | 0.92 | 0.44 | 2.10 |
| 4 | 0.98 | 0.56 | 1.77 |
| 6 | 1.00 | 0.66 | 1.51 |
| 8 | 1.00 | 0.75 | 1.34 |
| 10 | 1.00 | 0.82 | 1.22 |
| 12 | 1.00 | 0.88 | 1.14 |
| 14 | 1.00 | 0.94 | 1.07 |
| 16 | 1.00 | 0.98 | 1.02 |
| 18 | 1.00 | 0.99 | 1.01 |
| 20 | 1.00 | 1.00 | 1.00 |

Table 2.1.4 FR1 4TX Test 2 - 4RX

| SNR [dB] | Follow PMI Relative Tput | Random PMI Relative Tput | Relative Tput Ratio (Follow/Random) |
| --- | --- | --- | --- |
| -6 | 0.47 | 0.12 | 3.86 |
| -4 | 0.63 | 0.24 | 2.62 |
| -2 | 0.83 | 0.37 | 2.26 |
| 0 | 0.96 | 0.49 | 1.95 |
| 2 | 1.00 | 0.61 | 1.64 |
| 4 | 1.00 | 0.71 | 1.41 |
| 6 | 1.00 | 0.79 | 1.27 |
| 8 | 1.00 | 0.86 | 1.17 |
| 10 | 1.00 | 0.92 | 1.09 |
| 12 | 1.00 | 0.97 | 1.04 |
| 14 | 1.00 | 0.99 | 1.01 |
| 16 | 1.00 | 1.00 | 1.00 |
| 18 | 1.00 | 1.00 | 1.00 |
| 20 | 1.00 | 1.00 | 1.00 |

## 2.2 FR1 8TX

Table 2.2.1~Table 2.2.4 are the simulation results of wideband PMI test 1 and test 2 with 8TX (FR1).

Table 2.2.1 FR1 8TX Test 1 - 2RX

| SNR [dB] | Follow PMI Relative Tput | Random PMI Relative Tput | Relative Tput Ratio (Follow/Random) |
| --- | --- | --- | --- |
| -6 | 0.08 | 0.00 | 1114.05 |
| -4 | 0.20 | 0.00 | 78.24 |
| -2 | 0.33 | 0.02 | 19.28 |
| 0 | 0.44 | 0.05 | 8.09 |
| 2 | 0.56 | 0.12 | 4.78 |
| 4 | 0.70 | 0.19 | 3.58 |
| 6 | 0.83 | 0.28 | 3.02 |
| 8 | 0.92 | 0.36 | 2.57 |
| 10 | 0.96 | 0.44 | 2.20 |
| 12 | 0.99 | 0.51 | 1.92 |
| 14 | 1.00 | 0.59 | 1.68 |
| 16 | 1.00 | 0.67 | 1.49 |
| 18 | 1.00 | 0.74 | 1.34 |
| 20 | 1.00 | 0.81 | 1.23 |

Table 2.2.2 FR1 8TX Test 1 - 4RX

| SNR [dB] | Follow PMI Relative Tput | Random PMI Relative Tput | Relative Tput Ratio (Follow/Random) |
| --- | --- | --- | --- |
| -6 | 0.30 | 0.01 | 33.81 |
| -4 | 0.42 | 0.04 | 11.27 |
| -2 | 0.54 | 0.09 | 5.82 |
| 0 | 0.70 | 0.17 | 4.21 |
| 2 | 0.85 | 0.25 | 3.37 |
| 4 | 0.95 | 0.34 | 2.82 |
| 6 | 0.99 | 0.42 | 2.36 |
| 8 | 1.00 | 0.50 | 1.98 |
| 10 | 1.00 | 0.59 | 1.69 |
| 12 | 0.99 | 0.68 | 1.47 |
| 14 | 0.99 | 0.75 | 1.32 |
| 16 | 0.99 | 0.82 | 1.21 |
| 18 | 0.99 | 0.88 | 1.13 |
| 20 | 0.99 | 0.93 | 1.07 |

Table 2.2.3 FR1 8TX Test 2 - 2RX

| SNR [dB] | Follow PMI Relative Tput | Random PMI Relative Tput | Relative Tput Ratio (Follow/Random) |
| --- | --- | --- | --- |
| -6 | 0.06 | 0.00 | 415.05 |
| -4 | 0.20 | 0.00 | 95.77 |
| -2 | 0.33 | 0.01 | 23.90 |
| 0 | 0.43 | 0.05 | 8.63 |
| 2 | 0.54 | 0.12 | 4.66 |
| 4 | 0.68 | 0.20 | 3.45 |
| 6 | 0.85 | 0.28 | 3.01 |
| 8 | 0.96 | 0.36 | 2.68 |
| 10 | 0.99 | 0.44 | 2.26 |
| 12 | 1.00 | 0.51 | 1.94 |
| 14 | 1.00 | 0.60 | 1.68 |
| 16 | 1.00 | 0.68 | 1.48 |
| 18 | 1.00 | 0.75 | 1.32 |
| 20 | 1.00 | 0.83 | 1.21 |

Table 2.2.4 FR1 8TX Test 2 - 4RX

| SNR [dB] | Follow PMI Relative Tput | Random PMI Relative Tput | Relative Tput Ratio (Follow/Random) |
| --- | --- | --- | --- |
| -6 | 0.29 | 0.01 | 44.82 |
| -4 | 0.41 | 0.03 | 12.50 |
| -2 | 0.52 | 0.09 | 6.00 |
| 0 | 0.67 | 0.17 | 4.03 |
| 2 | 0.85 | 0.25 | 3.40 |
| 4 | 0.97 | 0.33 | 2.91 |
| 6 | 1.00 | 0.42 | 2.40 |
| 8 | 1.00 | 0.50 | 2.01 |
| 10 | 1.00 | 0.59 | 1.71 |
| 12 | 1.00 | 0.67 | 1.49 |
| 14 | 1.00 | 0.76 | 1.32 |
| 16 | 1.00 | 0.83 | 1.21 |
| 18 | 1.00 | 0.89 | 1.13 |
| 20 | 1.00 | 0.94 | 1.07 |

## 2.3 FR2 2TX

Table 2.3.1~Table 2.3.4 are the simulation result of wideband PMI test 1 and test 2 with 2TX (FR2).

Table 2.3.1 FR1 2TX Test 1 - 2RX ULA Low

| SNR [dB] | Follow PMI Relative Tput | Random PMI Relative Tput | Relative Tput Ratio (Follow/Random) |
| --- | --- | --- | --- |
| -6 | 0.04 | 0.01 | 5.58 |
| -4 | 0.19 | 0.09 | 2.16 |
| -2 | 0.35 | 0.26 | 1.35 |
| 0 | 0.48 | 0.40 | 1.19 |
| 2 | 0.64 | 0.53 | 1.22 |
| 4 | 0.85 | 0.68 | 1.25 |
| 6 | 0.97 | 0.86 | 1.13 |
| 8 | 1.00 | 0.97 | 1.03 |
| 10 | 1.00 | 1.00 | 1.00 |
| 12 | 1.00 | 1.00 | 1.00 |
| 14 | 1.00 | 1.00 | 1.00 |
| 16 | 1.00 | 1.00 | 1.00 |
| 18 | 1.00 | 1.00 | 1.00 |
| 20 | 1.00 | 1.00 | 1.00 |

Table 2.3.2 FR1 2TX Test 1 - 2RX ULA Medium

| SNR [dB] | Follow PMI Relative Tput | Random PMI Relative Tput | Relative Tput Ratio (Follow/Random) |
| --- | --- | --- | --- |
| -6 | 0.06 | 0.01 | 5.01 |
| -4 | 0.20 | 0.08 | 2.55 |
| -2 | 0.35 | 0.21 | 1.67 |
| 0 | 0.49 | 0.35 | 1.37 |
| 2 | 0.64 | 0.48 | 1.33 |
| 4 | 0.79 | 0.61 | 1.31 |
| 6 | 0.92 | 0.74 | 1.24 |
| 8 | 0.98 | 0.87 | 1.13 |
| 10 | 1.00 | 0.97 | 1.03 |
| 12 | 1.00 | 1.00 | 1.00 |
| 14 | 1.00 | 1.00 | 1.00 |
| 16 | 1.00 | 1.00 | 1.00 |
| 18 | 1.00 | 1.00 | 1.00 |
| 20 | 1.00 | 1.00 | 1.00 |

Table 2.3.3 FR1 2TX Test 2 - 2RX ULA Low

| SNR [dB] | Follow PMI Relative Tput | Random PMI Relative Tput | Relative Tput Ratio (Follow/Random) |
| --- | --- | --- | --- |
| -6 | 0.03 | 0.01 | 5.13 |
| -4 | 0.18 | 0.08 | 2.21 |
| -2 | 0.35 | 0.26 | 1.34 |
| 0 | 0.48 | 0.40 | 1.19 |
| 2 | 0.64 | 0.53 | 1.21 |
| 4 | 0.85 | 0.68 | 1.25 |
| 6 | 0.97 | 0.86 | 1.13 |
| 8 | 1.00 | 0.97 | 1.03 |
| 10 | 1.00 | 1.00 | 1.00 |
| 12 | 1.00 | 1.00 | 1.00 |
| 14 | 1.00 | 1.00 | 1.00 |
| 16 | 1.00 | 1.00 | 1.00 |
| 18 | 1.00 | 1.00 | 1.00 |
| 20 | 1.00 | 1.00 | 1.00 |

Table 2.3.4 FR1 2TX Test 2 - 2RX ULA Medium

| SNR [dB] | Follow PMI Relative Tput | Random PMI Relative Tput | Relative Tput Ratio (Follow/Random) |
| --- | --- | --- | --- |
| -6 | 0.06 | 0.01 | 4.42 |
| -4 | 0.19 | 0.07 | 2.59 |
| -2 | 0.34 | 0.20 | 1.68 |
| 0 | 0.48 | 0.35 | 1.36 |
| 2 | 0.64 | 0.48 | 1.32 |
| 4 | 0.79 | 0.61 | 1.31 |
| 6 | 0.92 | 0.74 | 1.24 |
| 8 | 0.98 | 0.88 | 1.12 |
| 10 | 1.00 | 0.97 | 1.03 |
| 12 | 1.00 | 1.00 | 1.00 |
| 14 | 1.00 | 1.00 | 1.00 |
| 16 | 1.00 | 1.00 | 1.00 |
| 18 | 1.00 | 1.00 | 1.00 |
| 20 | 1.00 | 1.00 | 1.00 |

# 3	Conclusions

In the contribution, we provide our simulation results of CSI wideband PMI tests

# 4	Reference

[1] R4-1902404, “Way forward on NR UE CSI requirements”, Samsung

Appendix

Table A.1 Wideband PMI test cases (FR1 4TX)

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) |
| --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz |
| Transmission Scheme | 1 | 1 |
| Duplex Mode | FDD | TDD |
| DLUL configuration | N.A | 7D1S2U (FR1.30-1) |
| Propagation channel | TDL_A 30ns, 5Hz | TDL_A 30ns, 5Hz |
| Correlation and antenna configuration | 4*2 4*4 [XP spatially High] | 4*2 4*4 [XP spatially High] |
| resourceConfigType | aperiodic | aperiodic |
| NZP CSI-RS ports | 4 port {3000…3003} | 4 port {3000…30003} |
| reportConfigType | aperiodic | aperiodic |
| Reporting periodicity and slot offset | 5/1 | 10/1 |
| (N1,N2) | (2,1) | (2,1) |
| (O1,O2) | (4,-) | (4,-) |
| CodebookMode | 1 | 1 |
| Codebook Restriction | 11111111 | 11111111 |
| Reporting granularity | Wide-band | Wide-band |
| MCS Table | 64QAM | 64QAM |
| Rank | 1 | 1 |
| FRC | MCS13 | MCS13 |
| Physical channel for reporting | PUSCH | PUSCH |
| CQI table | Table 1 with 256QAM | Table 1 with 256QAM |
| Minimum CSI delay | 8 ms | 9.5 ms |

Table A.2 Wideband PMI test cases (FR1 8TX)

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) |
| --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz |
| Transmission Scheme | 1 | 1 |
| Duplex Mode | FDD | TDD |
| DLUL configuration | N.A | 7D1S2U |
| Propagation channel | TDL_A 30ns, 5Hz | TDL_A 30ns, 5Hz |
| Correlation and antenna configuration | 8*2 8*4 [XP spatially High] | 8*2 8*4 [XP spatially High] |
| resourceConfigType | aperiodic | aperiodic |
| NZP CSI-RS ports | 8 port {3000…3007} | 8 port {3000…30007} |
| reportConfigType | aperiodic | aperiodic |
| Reporting periodicity and slot offset | 3/1 | 5/1 |
| (N1,N2) | (4,1) | (4,1) |
| (O1,O2) | (4,1) | (4,1) |
| CodebookMode | 1 | 1 |
| Codebook Restriction | 0xFFFF | 0xFFFF |
| Reporting granularity | Wide-band | Wide-band |
| MCS Table | 64QAM | 64QAM |
| Rank | 2 | 2 |
| FRC | MCS13 | MCS13 |
| Physical channel for reporting | PUSCH | PUSCH |
| CQI table | Table 1 without 256QAM | Table 1 without 256QAM |
| Minimum CSI delay | 8ms | 7ms |

Table A.3 Wideband PMI test cases (FR2 2TX)

| Parameter | Test 1 FR2 TDD with TDD DL-UL pattern DDSU | Test 2 FR2 TDD with TDD DL-UL pattern DDDSU |
| --- | --- | --- |
| Bandwidth/SCS | 100MHz/120kHz | 100MHz/120kHz |
| Transmission Scheme | 1 | 1 |
| Duplex Mode | TDD | TDD |
| DLUL configuration | DDSU (FR2.120-2) | DDDSU (FR2.120-1) |
| Propagation channel | TDL_A 30ns, 35Hz | TDL_A 30ns, 35Hz |
| Correlation and antenna configuration | 2*2 Option 1: ULA Low Option 2: ULA Medium | 2*2 Option 1: ULA Low Option 2: ULA Medium |
| resourceConfigType | aperiodic | aperiodic |
| NZP CSI-RS ports | 2 port {3000,3001} | 2 port {3000,3001} |
| reportConfigType | aperiodic | aperiodic |
| Reporting periodicity and slot offset | 8/1 | 5/1 |
| (N1,N2) | N.A | N.A |
| (O1,O2) | N.A | N.A |
| CodebookMode | N.A | N.A |
| Codebook Restriction | 001111 | 001111 |
| Reporting granularity | Wide-band | Wide-band |
| MCS Table | 64QAM | 64QAM |
| Rank | 1 | 1 |
| FRC | MCS13 (baseline) | MCS13 (baseline) |
| Physical channel for reporting | PUSCH | PUSCH |
| CQI table | Table 1 without 256QAM | Table 1 without 256QAM |
| Minimum CSI delay | 1.375ms | 1.75ms |

Table A.4 CSI simulation detailed parameter setting

| Items | Setting | Notes |
| --- | --- | --- |
| TDD UL/DL configurations | - FR1 TDD: 7D1S2U, S = 6D:4G:4U - FR2 TDD: DDSU, S=11D+3G |  |
| HARQ | - For PMI, RV sequence ={0,2,3,1} - For RI and CQI test cases ,no retransmission |  |
| CSI-RS resource/CSI reporting periodicity slot offset for periodic CSI reporting | - FR1 FDD 15 kHz: 5/1 - FR1 TDD 30 kHz: Option 1: 10/1 - FR2 TDD 120 kHz: - Option 1: 16/1 - Option 2: 8/1 |  |
| Minimum CSI delay for periodic CSI | 8ms |  |
| CSI-RS resource/CSI reporting interval and slot offset for aperiodic CSI reporting | - FR1 FDD 15 kHz: 3/1 with total CSI delay [8]slots - FR1 TDD 30 kHz: 5/1 with total CSI delay [12] slots - FR2 TDD 120 kHz: 12/1 with total CSI delay [27] slots |  |
| CSI reporting and CSI-RS resources Type for fading CQI test cases and RI test cases of FR2 | - Option 1: Periodic (previous meeting agreements) - Option 2: aperiodic (baseline) - Companies are encouraged to bring analysis for above options considering different CSI delay assumptions for periodic and aperiodic CSI reporting types |  |
| Number of HARQ process number | - FR1 FDD: 4 - FR1 TDD: 8 - FR2 TDD: 8 |  |

Table A.5 NZP CSI-RS configurations

| First subcarrier index in the PRB used for CSI-RS (k0, k1) | Row 3, (6,-) | Row 4,(0,-) | Row 8,(4,6) |
| --- | --- | --- | --- |
| First OFDM symbol in the PRB used for CSI-RS (l0) | 13 | 13 | 5 |
| Number of CSI-RS ports (X) | 2 | 4 | 8 |
| CDM Type | FD-CDM2 | FD-CDM2 | CDM4 (FD2,TD2) |
| Density (ρ) | 1 | 1 | 1 |

Table A.6 ZP CSI-RS configurations

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| First subcarrier index in the PRB used for CSI-RS (k0) | Row 5, 4 | Row 4, 8 |
| First OFDM symbol in the PRB used for CSI-RS (l0) | 9 | 13 |
| Number of CSI-RS ports (X) | 4 | 4 |
| CDM Type | FD-CDM2 | FD-CDM2 |
| Density (ρ) | 1 | 1 |

Table A.7 CSI-IM configurations

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| CSI-IM-RE-pattern | Pattern 0 | Pattern 1 |
| CSI-IM-ResourceMapping (kCSI-IM,lCSI-IM) | (4,9) | (8,13) |

Table A.8 FDD FRC for PMI test cases

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| FRC |  | FDD with 16QAM Rank1 | FDD with 16QAM Rank2 |
| Channel bandwidth | MHz | 10 | 10 |
| Subcarrier spacing | kHz | 15 | 15 |
| Number of allocated resource blocks | PRBs | 52 | 52 |
| Number of consecutive PDSCH symbols |  | 12 | 12 |
| Allocated slots per 2 frames | Slots | 15 | 15 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 13 | 13 |
| Modulation |  | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 |
| Number of MIMO layer |  | 1 | 2 |
| Number of additional DMRS |  | 1 | 1 |
| Overhead for TBS determination |  | 0 | 0 |
| Maximum number of HARQ transmissions |  | 4 | 4 |
| Information Bit Payload per Slot |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..19} | Bits | 12040 | 24072 |
| Transport block CRC per Slot |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..19} | Bits | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..,19} | CBs | 2 | 3 |
| Binary Channel Bits Per Slot |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |
| For CSI Slots i, if mod (i,5) =1, i={0,…,19} |  | N/A | N/A |
| For Slots i = 10 | Bits | 23712 | 47424 |
| For Non CSI-RS Slot i, if mod (i,5) ={0,2,3,4}, i={1,..9,11,…,19} | Bits | 24960 | 49920 |
| Max. Throughput averaged over 2 frames | Mbps | 9.030 | 18.054 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |

Table A.9 FR1 TDD FRC for PMI test cases with TDD DL-UL pattern FR1.30-1 (7D1S2U)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | FR1 TDD with 16QAM rank1 | FR1 TDD with 16QAM rank2 |
| Subcarrier spacing | kHz | 30 | 30 |
| Allocated resource blocks | PRBs | 106 | 106 |
| Number of consecutive PDSCH symbols |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  | 12 | 12 |
| Allocated slots per 2 frames |  | 23 | 23 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 13 | 13 |
| Modulation |  | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 |
| Number of MIMO layers |  | 1 | 2 |
| Number of additional DMRS |  | 1 | 1 |
| Overhead for TBS determination |  | 0 | 0 |
| Maximum number of HARQ transmissions |  | 4 | 4 |
| Information Bit Payload per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A |
| For Slot i = 20 | Bits | 24576 | 49176 |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6) for i from {1,…,19,22,…,39} |  | 24576 | 49176 |
| Transport block CRC per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} |  | N/A | N/A |
| For Slot i = 20 | Bits | 24 | 24 |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6) for i from {1,…,19,22,…,39} | Bits | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | CBs | N/A | N/A |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} |  | N/A | N/A |
| For Slot i = 20 | CBs | 3 | 6 |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6) for i from {1,…,19,22,…,39} | CBs | 3 | 6 |
| Binary Channel Bits Per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A |
| For Slot i = 20 | Bits | 48336 | 96672 |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6) for i from {1,…,19,22,…,39} | Bits | 50880 | 101760 |
| Max. Throughput averaged over 2 frames | Mbps | 28.2624 | 56.5524 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |

Table A.10  FR2 TDD PMI FRC with TDD DL-UL pattern FR2.120-2 (DDSU)

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | FR2 TDD with 16QAM rank1 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {1,…, 159} |  | 8 |
| For Slot i, if mod(i, 5) = {0,1,2) for i from {1,…,159} |  | 12 |
| Allocated slots per 2 frames |  | 59 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 1 |
| Overhead for TBS determination |  | 6 |
| Maximum number of HARQ transmissions |  | 4 |
| Information Bit Payload per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 14344 |
| For CSI-RS Slot i, if mod(i, 8) = 1 for i from {0,…, 159} | Bits | N/A |
| For Slot i, if mod(i, 8) = {0,4,5) for i from {1,…,79,82,…,159} |  | 14344 |
| Transport block CRC per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 24 |
| For CSI-RS Slot i, if mod(i, 8) = 1 for i from {0,…, 159} |  | N/A |
| For Slot i, if mod(i, 8) = {0,4,5) for i from {1,…,79,82,…,159} | Bits | 24 |
| Number of Code Blocks per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | CBs | N/A |
| For Slot i = 80 | CBs | 2 |
| For CSI-RS Slot i, if mod(i, 8) = 1 for i from {0,…, 159} | CBs | N/A |
| For Slot i, if mod(i, 8) = {0,4,5) for i from {1,…,79,82,…,159} |  | 2 |
| Binary Channel Bits Per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 28776 |
| For CSI-RS Slot i, if mod(i, 8) = 1 for i from {0,…, 159} | Bits | N/A |
| For Slot i, if mod(i, 8) = {0,4,5) for i from {1,…,79,82,…,159} | Bits | 30360 |
| Max. Throughput averaged over 2 frames | Mbps | 42.3148 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |

Table A.11  FR2 TDD PMI FRC with TDD DL-UL pattern FR2.120-1 (DDDSU)

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | FR2 TDD with 16QAM rank1 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {1,…, 159} |  | 9 |
| For Slot i, if mod(i, 5) = {0,1,2) for i from {1,…,159} |  | 12 |
| Allocated slots per 2 frames |  | 63 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 1 |
| Overhead for TBS determination |  | 6 |
| Maximum number of HARQ transmissions |  | 4 |
| Information Bit Payload per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 14344 |
| For CSI-RS Slot i, if mod(i, 5) = 1 for i from {0,…, 159} | Bits | N/A |
| For Slot i, if mod(i, 5) = {0,2) for i from {1,…,79,82,…,159} |  | 14344 |
| Transport block CRC per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 24 |
| For CSI-RS Slot i, if mod(i, 5) = 1 for i from {0,…, 159} |  | N/A |
| For Slot i, if mod(i, 5) = {0,2) for i from {1,…,79,82,…,159} | Bits | 24 |
| Number of Code Blocks per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | CBs | N/A |
| For Slot i = 80 | CBs | 2 |
| For CSI-RS Slot i, if mod(i, 5) = 1 for i from {0,…, 159} | CBs | N/A |
| For Slot i, if mod(i, 5) = {0,2) for i from {1,…,79,82,…,159} |  | 2 |
| Binary Channel Bits Per Slot |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |
| For Slot i = 80 | Bits | 28776 |
| For CSI-RS Slot i, if mod(i, 5) = 1 for i from {0,…, 159} | Bits | N/A |
| For Slot i, if mod(i, 5) = {0,2) for i from {1,…,79,82,…,159} | Bits | 30360 |
| Max. Throughput averaged over 2 frames | Mbps | 45.1836 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |