GPP TSG-RAN WG4 Meeting #90Bis                                   R4-1903345

Xi’an, China, 8th – 12th April, 2019

Agenda item:			8.5.1

Source:			Samsung

Title:	Discussion on OTA timing alignment for IAB

Document for:		Discussion

# 1	Introduction

In RAN1#96, RAN1 has sent RAN4 an LS [1] to ask RAN4’s input on OTA timing alignment issue, and the content is captured as below:

Agreements (RAN1 #Ad Hoc1901): - An IAB node should set its DL TX timing ahead of its DL Rx timing by TA/2 + T_delta - T_delta is signalled from the parent node, where the value is intended to account for factors such as the offset between parent DL Tx and UL Rx, if any due to factors such as Tx to Rx switching time, HW impairments, etc. - TA is the timing gap between UL Tx timing and DL Rx timing, which is derived based on existing Rel-15 mechanism Agreements (RAN1 #96): - T_delta is indicated by a parent to the child node independently from the existing Rel.15 TA indication from the parent node used to set the UL Tx timing of the child IAB node’s MT - T_delta is updated on an aperiodic basis determined by the parent node - The child IAB node should trigger its DL TX timing adjustment by TA/2 + T_delta after it receives the timing offset T_delta indication from its parent node, if it is using OTA Timing Case 1 to obtain its DL timing. - FFS: behavior if TA/2 + T_delta results in an effective negative timing offset - FFS: delay between receiving T_delta and application of T_delta at the child node - Separate value ranges/granularities may be considered for T_delta in FR1 and T_delta in FR2 RAN1 made the following agreements on the over-the-air (OTA) timing alignment for IAB RAN1 has agreed on the basic mechanism of over-the-air (OTA) timing alignment. As shown in Figure 1, when the switching gap Tg is equal to the TA_offset as defined in TS 38.133, i.e. UL Rx and DL Tx are well aligned, the child node can set its DL TX timing ahead of its DL Rx timing by TA/2, i.e. T_delta = 0. However, when the switching gap Tg is not equal to the TA_offset, i.e. UL Rx and DL Tx are not well aligned, the child node can set its DL TX timing ahead of its DL Rx timing by TA/2 + T_delta, where T_delta = (TA_offset - Tg)/2. RAN1 has not agreed on the exact values of T_delta in terms of its range and granularity though RAN1 agreed that separate values will be defined for FR1 and FR2. It is RAN1’s understanding that it is RAN4’s expertise to determine the exact values of T_delta, which is intended to account for factors such as the gap between uplink reception and downlink transmission timing at the parent node, hardware impairment, etc. It is also RAN1’s understanding that the granularity of T_delta may have an impact to the DL synchronization accuracy across multiple hops for IAB. To RAN WG4. ACTION 1: RAN1 would like RAN4 to provide input on the range and granularity of T_delta in FR1 and FR2. ACTION 2: RAN1 would like RAN4 to confirm whether the DL synchronization accuracy requirement defined in the current specification should also be applied for multi-hop scenarios for IAB. ACTION 3: RAN1 would like RAN4 to provide input on the requirement of OTA timing alignment across multiple hops in order to fulfil the DL synchronization accuracy requirements defined in the current specification.

In this discussion paper, we would like to share our view on the IAB timing alignment issue, which could serve part of RAN4’s response to the incoming LS.

# IAB OTA Timing Alignment

Based on the IAB WID [2], the following bullets related to IAB OTA timing alignment is included in IAB work item’s scope. It should be noted that 7 kinds of OTA timing alignment methods are proposed in IAB Study Item phase [3], while only “case-1” OTA timing alignment is needed to be specified in this Rel-16 IAB work item:

- Physical layer specification [RAN1-led, RAN2, RAN3, RAN4]: - Specification of mechanism to support the “case-1” OTA timing alignment. - Specification of RF and RRM requirements [RAN4-led]: - Define timing requirements for IAB specific network synchronization. This may include (a) requirement for “case 1” timing (e.g. accuracy of DL transmission timing alignment between an IAB-node and its parent node), and (b) cell phase synchronization accuracy for multi-hop IAB network.

Based on that, the discussion should be limited to case-1 OTA timing alignment, i.e., DL transmission timing alignment across IAB-nodes and IAB-donors (If DL TX and UL RX are not well aligned at the parent node, additional information about the alignment is needed for the child node to properly set its DL TX timing for OTA based timing & synchronization).

Observation 1: The discussion on IAB OTA timing alignment should be limited to Case-1 OTA timing alignment method.

#### 2.1 Analysis on Range of T_delta

For the basic mechanism of OTA timing alignment for case-1, the switching gap Tg is targeted to equal to the TA_offset, however due to TA adjustment step size, TA adjustment accuracy and IAB cell phase synchronization accuracy, the discrepancy between Tg and TA_offset could exist. To further DL transmission timing alignment across IAB-nodes and IAB-donors, T_delta is introduced: if UL Rx and DL Tx are well aligned at IAB-donor, the child node can set its DL TX timing ahead of its DL Rx timing by TA/2, i.e. T_delta = 0; if the switching gap Tg is not equal to the TA_offset, i.e. UL Rx and DL Tx are not well aligned, the child node can set its DL TX timing ahead of its DL Rx timing by TA/2 + T_delta, where T_delta = (TA_offset - Tg)/2. The figure contained in RAN1’s LS [1] illustrate the timing relationship well:

Figure-1. Alignment of DL TX and UL RX timing at the parent node (copied from RAN1 LS [1])

To consider the range of Tg, it should be noted that Tg should be no smaller than RX-TX switching time requirement, which can be served as the first principle for the lower limit of Tg. Based on the conclusion from Rel-15 gNB’s RF requirement, the RX-to-TX switching time should be no larger than 10us for FR1 gNB and 3us for FR2 gNB. Considering IAB parent node can be either gNB or other IAB node and IAB node’s RX-to-TX switching time is not specified yet, we can expect IAB node’s performance should be no better than normal gNB and no worse than normal FR2 UE (i.e., 5us), to considering the worst case in defining the largest possible range of T_delta, 3us can be regarded as the lower limit for FR2 Tg:

Table-1: Tg lower limitation due to Parent Node’s Rx-to-Tx switching time

|  | (a) Tg_min (limited by Parent Node’s RX-TX switching time) |
| --- | --- |
| FDD FR1 (N_TA_offset = 0) | N/A for FDD |
| FDD/TDD FR1 (N_TA_offset = 13us) | 10us for TDD N/A for FDD |
| TDD FR1 (N_TA_offset = 20us) | 10us |
| FR2 (N_TA_offset = 7us) | 3us |

On the other hand, another limitation for Tg is due to TA adjustment step size, TA adjustment accuracy and IAB cell phase synchronization accuracy. Specifically, Tg should satisfy:

- Tg ≤ TA_offset + IAB cell phase sync accuracy + (TA step size)/2 + Timing Advance adjustment accuracy

- Tg ≥ TA_offset - IAB cell phase sync accuracy - (TA step size)/2 - Timing Advance adjustment accuracy

Hence, considering the three factors mentioned above, Tg’s range could be derived as below:

Table-2: Tg value range due to TA adjustment step size, TA adjustment accuracy and 
IAB cell phase synchronization accuracy

|  | (b) IAB cell phase sync accuracy | (c) TA step size | (d) Timing Advance adjustment accuracy | Tg range due to (b)(c)(d) |
| --- | --- | --- | --- | --- |
| FDD FR1 (N_TA_offset = 0) | 3us | 16*64*Tc | 256Tc (for 15kHz SCS) | [-3us-512Tc-256Tc, +3us+512Tc+256Tc] = [-3.39us, 3.39us] |
| FDD/TDD FR1 (N_TA_offset = 13us) | 3us | 16*64*Tc | 256Tc (for 15kHz SCS) | [13us-3us-512Tc-256Tc, 13us+3us+512Tc+256Tc] = [9.61us, 16.39us] |
| TDD FR1 (N_TA_offset = 20us) | 3us | 16*64*Tc | 256Tc (for 15kHz SCS) | [20us-3us-512Tc-256Tc, 20us+3us+512Tc+256Tc] =[16.61us, 23.39us] |
| FR2 (N_TA_offset = 7us) | 3us | 4*64*Tc | 128Tc (for 60kHz SCS) | [7us-3us-128Tc-128Tc, 7us+3us+128Tc+128Tc] = [3.87us, 10.13us] |

To combine all the factors which limit the range of Tg, the Tg and accordingly T_delta’s ranges can be updated as below:

|  | Updated Tg range due to (a)(b)(c)(d), i.e., the range’s low limit should be larger than Tg_min in (a) | T_delta range due to factor (a)(b)(c)(d) |
| --- | --- | --- |
| FDD FR1 (N_TA_offset = 0) | [-3.39us, 3.39us] | [-1.695us, 1.695us] |
| FDD/TDD FR1 (N_TA_offset = 13us) | [9.61us, 16.39us] for FDD [10us, 16.39us] for TDD | [-1.695us, 1.695us] for FDD [-1.695us, 1.5us] for TDD |
| TDD FR1 (N_TA_offset = 20us) | [16.61us, 23.39us] | [-1.695us, 1.695us] |
| FR2 (N_TA_offset = 7us) | [3.87us, 10.13us] | [-1.565us, 1.565us] |

Proposal 1: The range of T_delta should be +/-1.7us for FR1 and +/-1.6us for FR2.

#### 2.2 Analysis on Granularity of T_delta

The granularity of T_delta should be limited by practical implementation, i.e., at least the granularity of T_delta should be no larger than TA adjustment accuracy, i.e, ±256 Tc for 15kHz and 30kHz SCS, ±128 Tc for 60kHz and ±32 Tc for 120kHz. Therefore, we propose to use 128 Tc for all FR1 case and 32 Tc for FR2 case.

Proposal 2: The granularity of T_delta should be 128 Tc for FR1 and 32 Tc for FR2.

For Action 2&3 in RAN1’s incoming LS, it is actually dependent on RAN4’s further discussion on IAB DL synchronization accuracy requirement for multi-hop cases, and we suggest to reply RAN1 for those two actions after RAN4 made conclusion accordingly.

Proposal 3: Postpone replying Action 2&3 in RAN1 LS (R1-1903810) after RAN4 studied the accuracy requirement for multi-hop IAB case.

# Conclusion

In this paper, we provided our view on the IAB timing alignment issue, which could serve part of RAN4’s response to the incoming LS:

Observation 1: The discussion on IAB OTA timing alignment should be limited to Case-1 OTA timing alignment method.

Proposal 1: The range of T_delta should be +/-1.7us for FR1 and +/-1.6us for FR2.

Proposal 2: The granularity of T_delta should be 128 Tc for FR1 and 32 Tc for FR2.

Proposal 3: Postpone replying Action 2&3 in RAN1 LS (R1-1903810) after RAN4 studied the accuracy requirement for multi-hop IAB case.

To summarize, the range and granularity of T_delta for Case-1 OTA timing alignment is proposed as below table:

|  | Range | Granularity |
| --- | --- | --- |
| FR1 | [-1.7us, +1.7us] | 128Tc |
| FR2 | [-1.6us, +1.6us] | 32Tc |

# Reference

[1] R1-1903810, “LS on OTA timing alignment for IAB”, RAN1#96.

[2] RP-190712, “Revised WID: Integrated Access and Backhaul for NR”, Qualcomm.

[3] 3GPP TR 38.874, “Study on Integrated Access and Backhaul”.