3GPP TSG-RAN WG4 Meeting #90b	R4-1903357

# Xi’an, China, 8th Apr. – 12th Apr. 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Measurement Gap Sharing in NR-DC (Section 9.1.2, 9.1.5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2019-04-01 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The mechanism of mesuarement gap sharing for NR-DC in Rel-15 late drop is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The mechanism of measurement gap sharing in NR-DC; and its corresponding performance requirements for gap-shared measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement gap sharing mechanism for NR-DC in Rel-15 late drop is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2 and 9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

>>>>>Start of Change 1<<<<<

9.1.2.1c	NR-DC: Measurement Gap Sharing

For NR standalone UE with NR-DC operation and configured with per-UE measurement gap, measurement gap sharing shall be applies when UE requires measurement gaps to identify and measure cells on FR1 intra-frequency carriers or when SMTC configured for FR1 intra-frequency measurement are fully overlapping with per-UE measurement gaps, and when UE requires measurement gaps to identify and measure cells on FR2 intra-frequency carriers, when SMTC configured for FR2 intra-frequency measurement are fully overlapping with per-UE measurement gaps or when UE is configured to identify and measure cells on inter-frequency carriers, and/or inter-RAT E-UTRAN carriers.

For NR standalone UE with NR-DC operation and configured with per-FR1 measurement gap, measurement gap sharing shall be applied when UE requires measurement gaps to identify and measure cells on FR1 intra-frequency carriers or when SMTC configured for FR1 intra-frequency measurement are fully overlapping with per-FR1 measurement gaps, and when UE is configured to identify and measure cells on FR1 inter-frequency carriers and/or inter-RAT E-UTRAN carriers.

For NR standalone UE with NR-DC operation and configured with per-FR2 measurement gap, measurement gap sharing shall be applied when UE requires measurement gaps to identify and measure cells on FR2 intra-frequency carriers or when SMTC configured for FR2 intra-frequency measurement are fully overlapping with per-FR2 measurement gaps, and when UE is configured to identify and measure cells on FR2 inter-frequency carriers.

When network signals “01”, “10” or “11” with RRC parameter measGapSharingConfig [2] and the value of X is defined as in Table 9.1.2.1c-1, and

-	Kintra-FR1 = 1 / X * 100,

-	Kintra-FR2 =  1 / (100 – X) * 100,

-	Kinter = 1 / (100 – X) * 100,

When network signals “00” indicating equal splitting gap sharing, X is not applied.

The RRC parameter MeasGapSharingScheme shall be applied to the calculation of carrier specific scaling factor as specified in clause 9.1.5.2.4.

Table 9.1.2.1c-1: Value of parameter X for NR-DC measurement gap sharing

| measGapSharingConfig | Value of X (%) |
| --- | --- |
| ‘00’ | Equal splitting |
| ‘01’ | 25 |
| ‘10’ | 50 |
| ‘11’ | 75 |

>>>>>End of Change 1<<<<<

>>>>>Start of Change 2<<<<<

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this section.

If measurement object i refers to an RSTD measurement with periodicity Tprs>160ms, CSSFwithin_gap,i=1. Otherwise, the the CSSFwithin_gap,i for other measurement objects (including RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms within an arbitrary 160ms period, count the total number of intrafrequency measurement objects and interfrequency/interRAT measurement objects which are candidates to be measured within the gap j.

-	An NR carrier is a candidate to be measured in a gap if its SMTC occasion fully occurs in time within the considered measurement gap excluding RF switching time. For intrafrequency NR carriers, if the high layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An interRAT measurement object is a candidate to be measured in all meausrement gaps.

Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for RSTD measurement with periodicity Tprs>160ms within an arbitrary 1280ms period.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis.

Per gap j:

Mintra-FR1,i,j: Number of FR1 intrafrequency measurement objects which are candidates to be measured in gap j (per FR1 gap or per UE gap) where the measurement object i is also a candidate. Otherwise Mintra-FR1,i,j  equals 0.

Mintra-FR2,i,j: Number of FR2 intrafrequency measurement objects which are candidates to be measured in gap j (per FR2 gap or per UE gap) where the measurement object i is also a candidate. Otherwise Mintra-FR2,i,j  equals 0.

Minter,i,j : Number of NR interfrequency and EUTRA interRAT measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter,i,j  equals 0.

Mtot,i,j = Mintra-FR1,i,j  + Mintra-FR2,i,j + Minter,i,j : Total number of intrafrequency, interfrequency and interRAT measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intrafrequency measurement object on FR1, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra-FR1×Mintra-FR1,i,j) in gaps where Mintra-FR1,i,j≠Mtot,i,j, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra-FR1,i,j) in gaps where Mintra-FR1,i,j= Mtot,i,j, where j=0…(160/MGRP)-1

-	measurement object i is an intrafrequency measurement object on FR2, interfrequency measurement object or interRAT measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×(Mintra-FR2,i,j +Minter,i,j)) in gaps where Mintra-FR1,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×(Mintra-FR2,i,j +Minter,i,j)) in gaps where Mintra-FR1,i,j=0, where j=0…(160/MGRP)-1

>>>>>End of Change 2<<<<<