3GPP TSG-RAN WG4 Meeting #90b              	R4-1903360

Xi’an, China, 8th – 12th April, 2019

Agenda item:	6.12.2.2.1

Source:	Samsung

Title:	Updated simulation results for NR PUSCH

Document for:	Discussion

# 1	Introduction

During RAN4#90 meeting, the remained issue for PUSCH requirements was discussed. Additional test cases for PUSCH are added and captured in WF [1]:

-	FR1 PUSCH resource mapping type B

  - The performance requirements for Type B should be defined as well as in FR1 besides the requirements agreed to define for Type A

  - The same number of test cases should be introduced for Type B as those agreed cases defined for type A

  - The performance requirements for Type B FR1 shall be for slot based symbol allocation

-	DMRS configuration

  - PUSCH FR1

    - Keep original agreements (both DMRS 1+0 and 1+1)

  - PUSCH FR2 with DMRS 1+1

    - For FR2 requirements with DMRS 1+1, company agreed to introduce the requirements, but need to down prioritize it. Down prioritization means the requirements for DMRS 1+1 can be introduced in Release 15 WI (completed by June, 2019) or in TEI-15by December,2019

-	PTRS configuration

  - For FR2 requirements with QPSK including cases for both CP-OFDM and DFT-s-OFDM, PT-RS is not configured, but RAN4 needs to rerun the related simulations

  - For FR2 PUSCH requirements without PT-RS additionally configured for 16QAM and 64QAM, company agreed to introduce the requirements, but need to down prioritize it. down prioritization means the FR2 PUSCH requirements without PT-RS additionally configured for 16QAM and 64QAM can be introduced in Release 15 WI (completed by June, 2019) or in TEI-15 by December,2019

Also, for the BS testing, the applicability rules for PUSCH with different configuration are specified.

In this contribution, the updated ideal and impairment simulation results are provided.

# 2	Simulation Results

In this section, based on the agreed simulation assumption for NR PUSCH as table 1, table 2. The ideal and impairment simulation results of PUSCH are presented as table 3, to table 11.

Table 1: Simulation assumption for NR PUSCH for CP-OFDM

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1,2 | 1,2 |
| Number of Rx | 2, 4,8 | 2 |
| Transmission scheme | Identity matrix (TMPI index 0) | n.a. |
| Number of layers | 1, 2 | 1,2 |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1(2), 1+1(2,11) for type A 1(0), 1+1(0,10) for type A | 1(0), 1+1(0,8) |
| symbols length | 14 | 10 |
| Time domain resource allocation type | type A, type B | type B |
| MCS index | 2, 16, 20 (20 only for 1Tx) | 2, 16, 20 (20 only for 1 Tx) |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | MCS 2: TDL-B 100ns, 400 Hz MCS 16: TDL-C 300ns, 100 Hz MCS 20: TDL-A 30ns, 10Hz | MCS 2 : TDL-A 30 ns, 300Hz MCS 16: TDL-A 30 ns,300Hz MCS 20: TDL-A 30 ns, 75HZ |
| SCS and BW | 15kHz: 5,10, 20 MHz 30kHz: 10, 20, 40, 100 MHz | 60kHz: 50, 100 MHz 120kHz: 50,100,200 MHz |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| PTRS | Not configured | QPSK: Not configured 16 and 64 QAM: (Configured with KPTRS =2, LPTRS =1) and (Not configured, low prority) |

Table 2: Simulation assumption for NR PUSCH for DFT-s-OFDM

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1,2 | 1 |
| Number of Rx | 2, 4,8 | 1,2 |
| Transmission scheme | Identity matrix (TMPI index 0) | n.a. |
| Number of layers | 1, 2 | 1 |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1(2), 1+1(2,11) for type A 1(0), 1+1(0,10) for type B | 1(0), 1+1(0,8) |
| symbols length | 14 | 10 |
| Time domain resource allocation type | type A, type B | type B |
| MCS index | 2, 16, 20 | 2, 16, 20 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | MCS 2: TDL-B 100ns, 400 Hz | MCS 2 : TDL-A 30 ns, 300Hz |
| SCS and BW | 15kHz: 5MHz 30kHz: 10MHz 15kHz: 25 PRB (contig., middle of test BW) 30kHz: 24 PRB (contig., middle of test BW) | 60kHz: 50MHz 120kHz: 50MHz 60kHz: 30 PRB (contig., middle of test BW) 120kHz: 30 PRB (contig., middle of test BW) |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| PTRS | Not configured | Not configured |

Table3: The target SNR performance for NR PUSCH for FR1 with 1 Tx with CP-OFDM for type A

| Tx/Rx | SCS (KHz) | Number of DMRS symbol | BW (MHz) | Target SNR for MCS2 (dB) (ideal) | Target SNR for MCS16 (dB) (ideal) | Target SNR for MCS20 (dB) (ideal) | Target SNR for MCS2 (dB) (impairment) | Target SNR for MCS16 (dB) (impairment) | Target SNR for MCS20 (dB) (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1T2Rx | 15 | 1 | 5 | 1.57 | 11.69 | 10.94 | 3.57 | 13.69 | 12.94 |
|  |  | 1+1 | 5 | -3.01 | 8.46 | 10.81 | -1.01 | 10.46 | 12.81 |
|  |  | 1 | 10 | 1.54 | 14.82 | 11.09 | 3.54 | 16.82 | 13.09 |
|  |  | 1+1 | 10 | -2.90 | 8.53 | 10.89 | -0.9 | 10.53 | 12.89 |
|  |  | 1 | 20 | N.A | N.A | 10.92 | N.A | N.A | 12.92 |
|  |  | 1+1 | 20 | -3.56 | 8.40 | 10.78 | -1.56 | 10.4 | 12.78 |
|  | 30 | 1 | 10 | -3.13 | 8.88 | 10.42 | -1.13 | 10.88 | 12.42 |
|  |  | 1+1 | 10 | -3.58 | 8.64 | 10.45 | -1.58 | 10.64 | 12.45 |
|  |  | 1 | 20 | -3.37 | 9.14 | 10.77 | -1.37 | 11.14 | 12.77 |
|  |  | 1+1 | 20 | -4.51 | 8.58 | 10.55 | -2.51 | 10.58 | 12.55 |
|  |  | 1 | 40 | -2.57 | 9.12 | 10.75 | -0.57 | 11.12 | 12.75 |
|  |  | 1+1 | 40 | -3.99 | 8.50 | 10.37 | -1.99 | 10.5 | 12.37 |
|  |  | 1 | 100 | -1.90 | 9.25 | 11.57 | 0.1 | 11.25 | 13.57 |
|  |  | 1+1 | 100 | -4.53 | 8.49 | 11.24 | -2.53 | 10.49 | 13.24 |
| 1Tx4Rx | 15 | 1 | 5 | -2.88 | 6.02 | 7.40 | -0.88 | 8.02 | 9.4 |
|  |  | 1+1 | 5 | -7.08 | 4.56 | 7.21 | -5.08 | 6.56 | 9.21 |
|  |  | 1 | 10 | -3.05 | 6.57 | 7.47 | -1.05 | 8.57 | 9.47 |
|  |  | 1+1 | 10 | -7.29 | 4.63 | 7.23 | -5.29 | 6.63 | 9.23 |
|  |  | 1 | 20 | N.A | 6.68 | 7.56 | N.A | 8.68 | 9.56 |
|  |  | 1+1 | 20 | -7.08 | 4.52 | 7.09 | -5.08 | 6.52 | 9.09 |
|  | 30 | 1 | 10 | -6.63 | 4.89 | 6.71 | -4.63 | 6.89 | 8.71 |
|  |  | 1+1 | 10 | -6.8 | 4.70 | 6.73 | -4.8 | 6.7 | 8.73 |
|  |  | 1 | 20 | -6.78 | 5.07 | 7.12 | -4.78 | 7.07 | 9.12 |
|  |  | 1+1 | 20 | -7.70 | 4.63 | 6.87 | -5.7 | 6.63 | 8.87 |
|  |  | 1 | 40 | -5.97 | 4.98 | 7.22 | -3.97 | 6.98 | 9.22 |
|  |  | 1+1 | 40 | -7.51 | 4.56 | 6.75 | -5.51 | 6.56 | 8.75 |
|  |  | 1 | 100 | -5.37 | 5.08 | 7.96 | -3.37 | 7.08 | 9.96 |
|  |  | 1+1 | 100 | -7.79 | 4.54 | 7.55 | -5.79 | 6.54 | 9.55 |
| 1Tx8Rx | 15 | 1 | 5 | -6.25 | 2.69 | 4.37 | -4.25 | 4.69 | 6.37 |
|  |  | 1+1 | 5 | -9.88 | 1.37 | 3.92 | -7.88 | 3.37 | 5.92 |
|  |  | 1 | 10 | -6.23 | 3.04 | 4.42 | -4.23 | 5.04 | 6.42 |
|  |  | 1+1 | 10 | -9.68 | 1.59 | 3.91 | -7.68 | 3.59 | 5.91 |
|  |  | 1 | 20 | N.A | 3.00 | 4.51 | N.A | 5 | 6.51 |
|  |  | 1+1 | 20 | -9.47 | 1.33 | 3.77 | -7.47 | 3.33 | 5.77 |
|  | 30 | 1 | 10 | -9.20 | 2.02 | 3.59 | -7.2 | 4.02 | 5.59 |
|  |  | 1+1 | 10 | -9.76 | 1.67 | 3.48 | -7.76 | 3.67 | 5.48 |
|  |  | 1 | 20 | -9.34 | 2.12 | 3.91 | -7.34 | 4.12 | 5.91 |
|  |  | 1+1 | 20 | -10.13 | 1.61 | 3.58 | -8.13 | 3.61 | 5.58 |
|  |  | 1 | 40 | -8.63 | 2.08 | 4.02 | -6.63 | 4.08 | 6.02 |
|  |  | 1+1 | 40 | -10.25 | 1.51 | 3.51 | -8.25 | 3.51 | 5.51 |
|  |  | 1 | 100 | -7.97 | 2.20 | 4.83 | -5.97 | 4.2 | 6.83 |
|  |  | 1+1 | 100 | -10.36 | 1.68 | 4.17 | -8.36 | 3.68 | 6.17 |

Table4: The target SNR performance for NR PUSCH for FR1 with 2 Tx with CP-OFDM for type A

| Tx/Rx | SCS (KHz) | Number of DMRS symbol | BW(MHz) | Target SNR for MCS2 (dB)(ideal) | Target SNR for MCS16 (dB)(ideal) | Target SNR for MCS2 (dB)(impairment) | Target SNR for MCS16 (dB)(impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Tx2Rx | 15 | 1+1 | 5 | 0.84 | 16.23 | 2.84 | 18.23 |
|  |  | 1+1 | 10 | 1.01 | 16.60 | 3.01 | 18.6 |
|  |  | 1+1 | 20 | 0.82 | 16.49 | 2.82 | 18.49 |
|  | 30 | 1+1 | 10 | 0.03 | 18.39 | 2.03 | 20.39 |
|  |  | 1+1 | 20 | -0.73 | 18.27 | 1.27 | 20.27 |
|  |  | 1+1 | 40 | -0.35 | 18.09 | 1.65 | 20.09 |
|  |  | 1+1 | 100 | -1.01 | 18.81 | 0.99 | 20.81 |
| 2Tx4Rx | 15 | 1 | 5 | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 5 | -3.26 | 9.19 | -1.26 | 11.19 |
|  |  | 1 | 10 | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 10 | -3.2 | 9.48 | -1.2 | 11.48 |
|  |  | 1 | 20 | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 20 | -3.23 | 9.22 | -1.23 | 11.22 |
|  | 30 | 1 | 10 | -3 | 10.8 | -1 | 12.8 |
|  |  | 1+1 | 10 | -3.35 | 9.86 | -1.35 | 11.86 |
|  |  | 1 | 20 | -2.38 | 10.79 | -0.38 | 12.79 |
|  |  | 1+1 | 20 | -3.83 | 9.81 | -1.83 | 11.81 |
|  |  | 1 | 40 | -1.6 | 10.74 | 0.4 | 12.74 |
|  |  | 1+1 | 40 | -3.84 | 9.75 | -1.84 | 11.75 |
|  |  | 1 | 100 | -1.04 | 11.53 | 0.96 | 13.53 |
|  |  | 1+1 | 100 | -4.53 | 10.03 | -2.53 | 12.03 |
| 2Tx8Rx | 15 | 1 | 5 | -2.8 | 7.74 | -0.8 | 9.74 |
|  |  | 1+1 | 5 | -5.88 | 5.19 | -3.88 | 7.19 |
|  |  | 1 | 10 | N.A | 8.68 | N.A | 10.68 |
|  |  | 1+1 | 10 | -5.9 | 5.23 | -3.9 | 7.23 |
|  |  | 1 | 20 | N.A | 9.04 | N.A | 11.04 |
|  |  | 1+1 | 20 | -5.68 | 5.16 | -3.68 | 7.16 |
|  | 30 | 1 | 10 | -5.53 | 6.25 | -3.53 | 8.25 |
|  |  | 1+1 | 10 | -6.15 | 5.42 | -4.15 | 7.42 |
|  |  | 1 | 20 | -5.16 | 6.17 | -3.16 | 8.17 |
|  |  | 1+1 | 20 | -6.5 | 5.28 | -4.5 | 7.28 |
|  |  | 1 | 40 | -4.65 | 6.15 | -2.65 | 8.15 |
|  |  | 1+1 | 40 | -6.44 | 5.26 | -4.44 | 7.26 |
|  |  | 1 | 100 | -4.08 | 6.53 | -2.08 | 8.53 |
|  |  | 1+1 | 100 | -7.02 | 5.34 | -5.02 | 7.34 |

Table5: The target SNR performance for NR PUSCH for FR1with DFT-OFDM for type A

| Tx/Rx | SCS | Number of DMRS symbol | BW | Target SNR for MCS2 (dB) (ideal) | Target SNR for MCS2 (dB) (impairment) |
| --- | --- | --- | --- | --- | --- |
| 1T2Rx | 15 | 1 | 5 | 3.67 | 5.67 |
|  | 15 | 1+1 | 5 | -3.82 | -1.82 |
|  | 30 | 1 | 10 | -2.97 | -0.97 |
|  | 30 | 1+1 | 10 | -3.77 | -1.77 |
| 1Tx4Rx | 15 | 1 | 5 | -2.85 | -0.85 |
|  | 15 | 1+1 | 5 | -6.37 | -4.37 |
|  | 30 | 1 | 10 | -5.39 | -3.39 |
|  | 30 | 1+1 | 10 | -6.73 | -4.73 |
| 1Tx8Rx | 15 | 1 | 5 | -6.36 | -4.36 |
|  | 15 | 1+1 | 5 | -9.82 | -7.82 |
|  | 30 | 1 | 10 | -8.97 | -6.79 |
|  | 30 | 1+1 | 10 | -9.56 | -7.56 |

Table6: The target SNR performance for NR PUSCH for FR1 with 1 Tx with CP-OFDM for type B

| Tx/Rx | SCS (KHz) | Number of DMRS symbol | BW (MHz) | Target SNR for MCS2 (dB) (ideal) | Target SNR for MCS16 (dB) (ideal) | Target SNR for MCS20 (dB) (ideal) | Target SNR for MCS2 (dB) (impairment) | Target SNR for MCS16 (dB) (impairment) | Target SNR for MCS20 (dB) (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1T2Rx | 15 | 1 | 5 | N.A | N.A | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 5 | -2.77 | 7.66 | 10.44 | -0.77 | 9.66 | 12.44 |
|  |  | 1 | 10 | N.A | N.A | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 10 | -3.21 | 8.15 | 10.52 | -1.21 | 10.15 | 12.52 |
|  |  | 1 | 20 | N.A | N.A | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 20 | -2.6 | 8.03 | 10.48 | -0.6 | 10.03 | 12.48 |
|  | 30 | 1 | 10 | -2.95 | 8.49 | 10.3 | -0.95 | 10.49 | 12.3 |
|  |  | 1+1 | 10 | -3.83 | 8.10 | 10.4 | -1.83 | 10.10 | 12.4 |
|  |  | 1 | 20 | -3.49 | 8.78 | N.A | -1.49 | 10.78 | N.A |
|  |  | 1+1 | 20 | -4.16 | 8.01 | 10.4 | -2.16 | 10.01 | 12.4 |
|  |  | 1 | 40 | -2.4 | N.A | N.A | -0.4 | N.A | N.A |
|  |  | 1+1 | 40 | -4.23 | 7.93 | 10.3 | -2.23 | 9.93 | 12.3 |
|  |  | 1 | 100 | N.A | N.A | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 100 | -4.11 | 7.89 | 11.08 | -2.11 | 9.89 | 13.08 |
| 1Tx4Rx | 15 | 1 | 5 | -4.12 | 6.34 | 7.05 | -2.12 | 8.34 | 9.05 |
|  |  | 1+1 | 5 | -6.33 | 4.20 | 6.94 | -4.33 | 6.20 | 8.94 |
|  |  | 1 | 10 | N.A | 7.11 | 7.13 | N.A | 9.11 | 9.13 |
|  |  | 1+1 | 10 | -6.53 | 4.25 | 7.03 | -4.53 | 6.25 | 9.03 |
|  |  | 1 | 20 | N.A | 7.47 | 7.26 | N.A | 9.47 | 9.26 |
|  |  | 1+1 | 20 | -6.16 | 4.13 | 6.98 | -4.16 | 6.13 | 8.98 |
|  | 30 | 1 | 10 | -6.63 | 4.37 | 6.55 | -4.63 | 6.37 | 8.55 |
|  |  | 1+1 | 10 | -7.3 | 4.16 | 6.85 | -5.3 | 6.13 | 8.85 |
|  |  | 1 | 20 | -6.93 | 4.57 | N.A | -4.93 | 6.57 | N.A |
|  |  | 1+1 | 20 | -7.51 | 4.10 | 6.92 | -5.51 | 6.10 | 8.92 |
|  |  | 1 | 40 | -6.06 | N.A | N.A | -4.06 | N.A | N.A |
|  |  | 1+1 | 40 | -7.42 | 4.01 | 6.79 | -5.42 | 6.01 | 8.79 |
|  |  | 1 | 100 | N.A | N.A | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 100 | -7.31 | 3.99 | N.A | -5.31 | 5.99 | N.A |
| 1Tx8Rx | 15 | 1 | 5 | -7.39 | 2.39 | 3.98 | -5.39 | 4.39 | 5.98 |
|  |  | 1+1 | 5 | -9.64 | 0.92 | 3.81 | -7.74 | 2.92 | 5.81 |
|  |  | 1 | 10 | -4.02 | 2.78 | 4.03 | -2.02 | 4.78 | 6.03 |
|  |  | 1+1 | 10 | -9.62 | 1.13 | 3.85 | -7.62 | 3.13 | 5.85 |
|  |  | 1 | 20 | N.A | N.A | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 20 | -9.21 | 0.94 | 3.72 | -7.21 | 2.94 | 5.72 |
|  | 30 | 1 | 10 | -9.8 | 1.24 | 3.59 | -7.8 | 3.24 | 5.59 |
|  |  | 1+1 | 10 | -10.5 | 0.99 | 3.82 | -8.5 | 2.99 | 5.82 |
|  |  | 1 | 20 | -9.94 | 1.43 | 3.85 | -7.94 | 3.43 | 5.85 |
|  |  | 1+1 | 20 | -10.6 | 0.90 | 3.85 | -8.6 | 2.9 | 5.85 |
|  |  | 1 | 40 | N.A | N.A | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 40 | -10.4 | 0.74 | 3.77 | -8.4 | 2.74 | 5.77 |
|  |  | 1 | 100 | N.A | N.A | N.A | N.A | N.A | N.A |
|  |  | 1+1 | 100 | -10.3 | 0.85 | 4.15 | -8.3 | 2.85 | 6.15 |

Table7: The target SNR performance for NR PUSCH for FR1 with 2 Tx with CP-OFDM for type B (to be updated)

| Tx/Rx | SCS (KHz) | Number of DMRS symbol | BW(MHz) | Target SNR for MCS2 (dB)(ideal) | Target SNR for MCS16 (dB)(ideal) | Target SNR for MCS2 (dB)(impairment) | Target SNR for MCS16 (dB)(impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Tx2Rx | 15 | 1+1 | 5 |  |  |  |  |
|  |  | 1+1 | 10 |  |  |  |  |
|  |  | 1+1 | 20 |  |  |  |  |
|  | 30 | 1+1 | 10 |  |  |  |  |
|  |  | 1+1 | 20 |  |  |  |  |
|  |  | 1+1 | 40 |  |  |  |  |
|  |  | 1+1 | 100 |  |  |  |  |
| 2Tx4Rx | 15 | 1 | 5 |  |  |  |  |
|  |  | 1+1 | 5 |  |  |  |  |
|  |  | 1 | 10 |  |  |  |  |
|  |  | 1+1 | 10 |  |  |  |  |
|  |  | 1 | 20 |  |  |  |  |
|  |  | 1+1 | 20 |  |  |  |  |
|  | 30 | 1 | 10 |  |  |  |  |
|  |  | 1+1 | 10 |  |  |  |  |
|  |  | 1 | 20 |  |  |  |  |
|  |  | 1+1 | 20 |  |  |  |  |
|  |  | 1 | 40 |  |  |  |  |
|  |  | 1+1 | 40 |  |  |  |  |
|  |  | 1 | 100 |  |  |  |  |
|  |  | 1+1 | 100 |  |  |  |  |
| 2Tx8Rx | 15 | 1 | 5 |  |  |  |  |
|  |  | 1+1 | 5 |  |  |  |  |
|  |  | 1 | 10 |  |  |  |  |
|  |  | 1+1 | 10 |  |  |  |  |
|  |  | 1 | 20 |  |  |  |  |
|  |  | 1+1 | 20 |  |  |  |  |
|  | 30 | 1 | 10 |  |  |  |  |
|  |  | 1+1 | 10 |  |  |  |  |
|  |  | 1 | 20 |  |  |  |  |
|  |  | 1+1 | 20 |  |  |  |  |
|  |  | 1 | 40 |  |  |  |  |
|  |  | 1+1 | 40 |  |  |  |  |
|  |  | 1 | 100 |  |  |  |  |
|  |  | 1+1 | 100 |  |  |  |  |

Table8: The target SNR performance for NR PUSCH for FR1with DFT-OFDM for type B

| Tx/Rx | SCS | Number of DMRS symbol | BW | Target SNR for MCS2 (dB) (ideal) | Target SNR for MCS2 (dB) (impairment) |
| --- | --- | --- | --- | --- | --- |
| 1T2Rx | 15 | 1 | 5 | N.A | N.A |
|  | 15 | 1+1 | 5 | -3.34 | -1.34 |
|  | 30 | 1 | 10 | -3.08 | -1.08 |
|  | 30 | 1+1 | 10 | -4.55 | -2.55 |
| 1Tx4Rx | 15 | 1 | 5 | N.A | N.A |
|  | 15 | 1+1 | 5 | -7.09 | -5.09 |
|  | 30 | 1 | 10 | -6.84 | -4.84 |
|  | 30 | 1+1 | 10 | -8.12 | -6.12 |
| 1Tx8Rx | 15 | 1 | 5 | N.A | N.A |
|  | 15 | 1+1 | 5 | -10.4 | -8.4 |
|  | 30 | 1 | 10 | -9.78 | -7.78 |
|  | 30 | 1+1 | 10 | -11.3 | -9.3 |

Table9 The target SNR performance for NR PUSCH for FR2 with CP-OFDM without PTRS

| Tx/Rx | SCS | BW | Number of DMRS symbol | Target SNR for MCS2 (dB) (ideal) | Target SNR for MCS16 (dB) (ideal) | Target SNR for MCS20 (dB) (ideal) | Target SNR for MCS2 (dB) (impairment) | Target SNR for MCS16 (dB) (impairment) | Target SNR for MCS20 (dB) (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1T2Rx | 60 | 50 | 1 | -4.4 | 9.2 | 11.3 | -1.9 | 11.7 | 13.8 |
|  | 60 | 100 | 1 | -4.4 | 9.9 | 11.7 | -1.9 | 12.4 | 14.2 |
|  | 120 | 50 | 1 | -3.7 | 9.4 | 11.4 | -1.2 | 11.9 | 13.9 |
|  | 120 | 100 | 1 | -4.3 | 9.5 | 11.5 | -1.8 | 12.0 | 14.0 |
|  | 120 | 200 | 1 | -4.2 | 9.6 | 11.5 | -1.7 | 12.1 | 14.0 |
| 2T2Rx | 60 | 50 | 1 | -0.4 | N.A | N.A | 2.1 | N.A | N.A |
|  | 60 | 100 | 1 | -0.4 | 7.0 | N.A | 2.1 | 9.5 | N.A |
|  | 120 | 50 | 1 | -0.3 | 18.8 | N.A | 2.2 | 21.3 | N.A |
|  | 120 | 100 | 1 | -0.1 | 19.0 | N.A | 2.4 | 21.5 | N.A |
|  | 120 | 200 | 1 | -0.3 | 19.0 | N.A | 2.2 | 21.5 | N.A |

Table10 The target SNR performance for NR PUSCH for FR2 with CP-OFDM with PTRS

| Tx/Rx | SCS | BW | Number of DMRS symbol | Target SNR for MCS16 (dB) (ideal) | Target SNR for MCS20 (dB) (ideal) | Target SNR for MCS16 (dB) (impairment) | Target SNR for MCS20 (dB) (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1T2Rx | 60 | 50 | 1 | 9.2 | 11.3 | 11.7 | 13.8 |
|  | 60 | 100 | 1 | 9.9 | 11.7 | 12.4 | 14.2 |
|  | 120 | 50 | 1 | 9.4 | 11.4 | 11.9 | 13.9 |
|  | 120 | 100 | 1 | 9.5 | 11.5 | 12.0 | 14.0 |
|  | 120 | 200 | 1 | 9.6 | 11.5 | 12.1 | 14.0 |
| 2T2Rx | 60 | 50 | 1 | N.A | N.A | N.A | N.A |
|  | 60 | 100 | 1 | N.A | N.A | N.A | N.A |
|  | 120 | 50 | 1 | N.A | N.A | N.A | N.A |
|  | 120 | 100 | 1 | N.A | N.A | N.A | N.A |
|  | 120 | 200 | 1 | N.A | N.A | N.A | N.A |

Table11: The target SNR performance for NR PUSCH for FR2 with DFT-s-OFDM

| Tx/Rx | SCS | Number of DMRS symbol | BW | Target SNR for MCS2 (dB) (ideal) | Target SNR for MCS2 (dB) (impairment) |
| --- | --- | --- | --- | --- | --- |
| 1T2Rx | 60 | 1 | 50 | -4.1 | -1.6 |
|  | 120 | 1 | 50 | -3.6 | -1.1 |

Observation 1: Based on the simulation results, some test cases with 1 only DMRS configuration is unreliable, which has bad performance and cannot achieve the 70% of maximum throughput due to the chosen channel conditions. Removing these cases in current specification should be considered.

# 3	Conclusion

In this contribution, the ideal and impairment simulation results are provided.

Observation 1: Base on the simulation results, some test cases with only 1 DMRS configuration are unreliable, which has bad performance and cannot achieve the 70% of maximum throughput due to the chosen channel condition. Removing these cases in current specification should be considered.

# References

R4-1902434, WF on NR PUSCH demodulation requirements, Nokia