3GPP TSG-RAN WG4 Meeting #90bis												  	         R4-1903529

Xian, China 8th – 12th of April, 2019

Source: 	Nokia

Title: 					LTE 410 self band protection

Agenda Item: 			7.10.1

Document for:	Approval

# 1	Introduction

This contribution presents simulation results for LTE 410 self band protection.

# 2	Discussion

LTE 410 – 430 MHz WI will introduce two new bands which are band 87 and 88.

Table 1: Bands 87 and 88

| 87 | 410 MHz | – | 415 MHz | 420 MHz | – | 425 MHz | FDD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 88 | 412 MHz | – | 417 MHz | 422 MHz | – | 427 MHz | FDD |

Frequency arrangement is presented in Figure 1 from which we can see that three different uplink to downlink separation scenarios exists.

Band 87 UL --> band 87 DL = 5 MHz

Band 88 UL --> band 88 DL = 5 MHz

Band 87 UL --> band 88 DL = 8 MHz

Band 88 UL --> band 87 DL = 3 MHz

This is important from UE to UE co-existence requirement point of view as we need to study whether it is possible to meet standard -50 dBm/1 MHz requirement.

Figure 1: Band 87 and 88 frequency arrengement

Normal LTE simulation assumptions were used and following band 31 filter attenuation was assumed

https://en.rf360jv.com/inf/40/ds/is/B1220.pdf

Figure 1: Simulation results for emissions.

Based on results presented in Figure 2 we can see that emissions are

25 RB = -30 dBm/MHz @5MHz offset and -33 dBm/MHz @8MHz offset

1 RB = -48 dBm/MHz @5MHz offset and -51 dBm/MHz @8MHz offset

When filter attenuation is taken into account it is possible to meet -50 dBm/1 MHz requriement.

However, @3MHz offset the emissions are roughly -19 dBm/MHz and the filter data sheet does not guarantee any attenuation hence to be safe only -20 dBm/1MHz can be specified for the case of Band 88 UL --> band 87 DL.

# 3 	Conclusion

Based on studies presented in this paper we propose to define UE to UE co-existence requirements for bands 87 and 88 as follows

Band 87 UL --> band 87 DL = 5 MHz --> -50 dBm/1MHz

Band 88 UL --> band 88 DL = 5 MHz --> -50 dBm/1MHz

Band 87 UL --> band 88 DL = 8 MHz --> -50 dBm/1MHz

Band 88 UL --> band 87 DL = 3 MHz --> -20 dBm/1MHz