3GPP TSG-RAN WG4 Meeting #90bis												  	         R4-1903536

Xian, China 8th – 12th of April, 2019

Source: 	Nokia, AMX

Title: 					TP to TR 36.716-03-01: 3-5-7 and 3-3-5-7

Agenda Item: 			7.3.3

Document for:	Approval

# 1	Introduction

This contribution is a TP for

| 3-5-7 | 4 | 3A-3A-5A-7A_BCS0 | Rel-11 | Israel Soto, AMX | israel.soto@ americamovil.com | Nokia, Ericsson, Huawei, Samsung | new | 3B_3CC_3A-5A-7A_BCS1(new) 2B_3CC_3A-3A-5A_BCS0 (new) 2B_3CC_3A-3A-7A_BCS0 (completed) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3-5-7 | 3 | 3A-5A-7A_BCS1 | Rel-10 | Israel Soto, AMX | israel.soto@ americamovil.com | Nokia, Ericsson, Huawei, Samsung | new | 2B_2CC_3A-5A_BCS2 (completed) 2B_2CC_3A-7A_BCS0 (completed) 2B_2CC_5A-7A_BCS1 (completed) |
| 3-5-7 | 4 | 3A-5A-7C_BCS0 | Rel-10 | Israel Soto, AMX | israel.soto@ americamovil.com | Nokia, Ericsson, Huawei, Samsung | new | 3B_3CC_3A-5A-7A_BSC0 (completed) 2B_3CC_3A-7C_BCS0 (completed) 2B_3CC_5A-7C_BCS0 (completed) |

****************************** Start of the TP*********************************************

## 5.X	3-5-7 and 3-3-5-7

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| CA_3A-3A-5A-7A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 70 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
| CA_3A-5A-7A | - | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
| CA_3A-5A-7C | - | 3 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

### 5.X.2 	Co-existence studies

Table 5.X.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7520 |
| 5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 |
| 7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 |

No uplink harmonic interfering downlink.

Table 5.X.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 |
| 5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 |
| 7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 7860 | 10480 | 10760 |

No receiver harmonic mixing interfering downlink.

### 5.X.3	∆TIB and ∆RIB values

dTib and dRib have been already specified

Table 5.X.3-1: IB,c

| CA_3-5-7, CA_3-3-5-7 | 3 | 0.5 |
| --- | --- | --- |
|  | 5 | 0.3 |
|  | 7 | 0.5 |

Table 5.X.3-2: R IB,c

| CA_3-5-7, CA_3-3-5-7 | 3 | 0 |
| --- | --- | --- |
|  | 5 | 0 |
|  | 7 | 0 |

### 5.X.4 	REFSENS

No MSD is necessary.

****************************** End of the TP*********************************************