3GPP TSG-RAN WG4 Meeting #90bis	R4-1903571

Xi'an, China, 8th – 12th April 2019

Agenda item:	6.12.2.2	PUSCH		[NR_newRAT-Perf]

Source: 		NTT DOCOMO, INC.

Title: 	NR PUSCH for high speed in Rel.15

Document for:	Approval

- Introduction

In the past meetings, the following agreements were reached for BS performance under HST scenarios [1, 2]. In this contribution, we provide our views on PUSCH for HST.

RAN#81 [1]

The UE and BS demodulation performance requirements under HST scenarios are treated in TEI15 after December 2018.

RAN4#90 [2]

- Companies are encourage to evaluate for RAN4#90bis when they could provide evaluation results  for the following scenarios :

  - PUSCH requirements for high speed scenarios

    - Option 1 HST (up to 300km/h or 350km/h at Band 7 (2.7GHz) [and  Band n77 (3.6GHz)])

    - Option 2: Use the same band as LTE to reduce the workload for BS side (i.e., maximum Doppler shift is 1340Hz).

  - PRACH requirements for high speed scenarios

    - Preamble format 0 with restricted set A and B

NOTE: For above scenarios, 1 additional DMRS is baseline. Other assumption is not precluded.

- Operators are encourage to provide the motivation to introduce HST related tests as well as their priority on the test to be introduced in Rel-15 in RAN4 #90bis

2.	Discussion

In the separate contribution [3], the priority for discussion on HST in Rel.15 is proposed. As PUSCH requirements for HST, the following requirements should be considered.

1st Priority: PUSCH requirement for HST (up to 350km/h at 2.1GHz (Band n1)), which is equivalent to LTE

2nd Priority: PUSCH requirement for HST (up to 350km/h at 3.6GHz (Band n77))

3rd Priority: PUSCH for UL timing adjustment (up to 350km/h)

In the following, deployment scenarios and parameters for HST and UL timing adjustment are provided.

2.1.1	HST

For Ds and Dmin, the same values can be used since it is expected that NR cells for HST will be deployed at similar scenarios as LTE, as mentioned in [3]. For maximum Doppler frequency fd, different values can be considered to different subcarrier spacing, as proposal 2 in [3]. Our preference for deployment scenarios and parameters are shown bellow.

Proposal 1: Define parameters for NR HST as follows:

Table 1: Proposed parameters for NR high speed train scenario

| Parameter | Value |  |
| --- | --- | --- |
|  | Scenario X | Scenario Y |
|  | 1000 m | 300 m |
|  | 50 m | 2 m |
|  | 350 km/h | 350 km/h |
|  | 15kHz SCS: 1340Hz 30kHz SCS: 2334Hz | 15kHz SCS: 1340Hz 30kHz SCS: 2334Hz |

Table 2: Proposed parameters for NR high speed train test

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR1 |
| Transform precoding | Disabled | Enabled |
| Number of Tx | 1 TBD: 2 | 1 TBD: 2 |
| Number of Rx | 2 TBD: 4, 8 | 2 TBD: 4, 8 |
| Number of layers | 1 TBD: 2 | 1 TBD: 2 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1 | 1+1 |
| symbols length | 14 | 14 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type A |
| Frequency domain resource | Full applicable test bandwidth | 15kHz: 25 PRB; 30kHz: 24 PRB (middle of test BW) |
| MCS index | 2 TBD: 16, 20 (20 only for 1Tx) | 2 TBD: 16, 20 (20 only for 1Tx) |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz |
| Propagation condition | Scenario X/Y | Scenario X/Y |
| SCS and BW | 15kHz: 5MHz, 10MHz, 20MHz; 30kHz: 10MHz, 20MHz, 40MHz, 100MHz |  |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @30% of maximum throughput SNR @70% of maximum throughput | SNR @30% of maximum throughput SNR @70% of maximum throughput |

2.1.2	UL timing adjustment

In the last meeting, UL timing adjustment was proposed in [4]. The following parameters can be considered.

Proposal 2: Define parameters for NR UL timing adjustment as follows:

Table 3. Proposed parameters for NR UL timing adjustment test.

| Parameter | Scenario X | Scenario Y |
| --- | --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: TDLC300-400 | Stationary UE: AWGN Moving UE: AWGN |
| UE speed | 120 km/h | 350 km/h |
| CP length | Normal | Normal |
| A | 10*15/SCS μs ,where SCS is Sub Carrier Spacing in kHz. | 10*15/SCS s ,where SCS is Sub Carrier Spacing in kHz. |
|  | 0.04 s-1 | 0.18 s-1 |

3.	Conclusion

In this contribution, we provide parameters for PUSCH under high speed scenarios. The following proposals are obtained.

For HST

Proposal 1: Define parameters for NR HST as follows:

Table 1: Proposed parameters for NR high speed train scenario

| Parameter | Value |  |
| --- | --- | --- |
|  | Scenario X | Scenario Y |
|  | 1000 m | 300 m |
|  | 50 m | 2 m |
|  | 350 km/h | 350 km/h |
|  | 15kHz SCS: 1340Hz 30kHz SCS: 2334Hz | 15kHz SCS: 1340Hz 30kHz SCS: 2334Hz |

Table 2: Proposed parameters for NR high speed train test

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR1 |
| Transform precoding | Disabled | Enabled |
| Number of Tx | 1 TBD: 2 | 1 TBD: 2 |
| Number of Rx | 2 TBD: 4, 8 | 2 TBD: 4, 8 |
| Number of layers | 1 TBD: 2 | 1 TBD: 2 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1 | 1+1 |
| symbols length | 14 | 14 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type A |
| Frequency domain resource | Full applicable test bandwidth | 15kHz: 25 PRB; 30kHz: 24 PRB (middle of test BW) |
| MCS index | 2 TBD: 16, 20 (20 only for 1Tx) | 2 TBD: 16, 20 (20 only for 1Tx) |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz |
| Propagation condition | Scenario X/Y | Scenario X/Y |
| SCS and BW | 15kHz: 5MHz, 10MHz, 20MHz; 30kHz: 10MHz, 20MHz, 40MHz, 100MHz |  |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @30% of maximum throughput SNR @70% of maximum throughput | SNR @30% of maximum throughput SNR @70% of maximum throughput |

For UL timing adjustment

Proposal 2: Define parameters for NR UL timing adjustment as follows:

Table 3. Proposed parameters for NR UL timing adjustment test.

| Parameter | Scenario X | Scenario Y |
| --- | --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: TDLC300-400 | Stationary UE: AWGN Moving UE: AWGN |
| UE speed | 120 km/h | 350 km/h |
| CP length | Normal | Normal |
| A | 10*15/SCS μs ,where SCS is Sub Carrier Spacing in kHz. | 10*15/SCS s ,where SCS is Sub Carrier Spacing in kHz. |
|  | 0.04 s-1 | 0.18 s-1 |

References

- RP-182149, “Way forward on RAN4 work for Core and Perf. part for NR”, RAN4 Chairman, RAN #81, September 2019

- R4-1902442, “WF on high speed related requirements for NR BS demodulation in Rel.15”, NTT DOCOMO, INC., China Telecom, RAN4 #90, February 2019

- R4-1903570, “Motivation and discussion priority for NR high speed in Rel.15”, NTT DOCOMO, INC., RAN4 #90bis, April 2019

- R4-1900554, “NR PUSCH for high speed”, NTT DOCOMO, INC., RAN4 #90, February 2019