3GPP TSG-RAN4 Meeting #90bis 	R4-1903682

# Xi’an, China, 8th – 12th Apr, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 36.133 | CR | 6378 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance on TDD inter-frequency re-establishment test cases R15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Perf |  |  |  |  |  | Date: |  |  | 2019-03-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clarification is needed to ensure that the UE is aware of the target inter-frequency in the NB RRC re-establishment test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Maintenance work, - Clarify that the UE is provided by the network the context of the target inter-frequency carrier before the UE declares RLF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.1.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change 1 >>

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage

A.6.1.23.1	Test Purpose and Environment

The purpose is to verify that the NB-IoT TDD inter-frequency RRC re-establishment delay is within the specified limits. These tests will verify the requirements for Cat-NB1 UE in clause 6.5.

The test parameters are given in table A.6.1.23.1-1 and table A.6.1.23.1-2 below. nCell1 and nCell2 are NB-IoT cells on different frequency carriers. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be indicated in SIB with the carrier frequency of nCell 2 to ensure that the UE has the context of the carrier frequency of nCell 2 by the end of T1.

Table A.6.1.23.1-1: General test parameters for TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | In-band |  |
| Initial condition | Active cell |  | nCell1 |  |
|  | Neighbour cells |  | eCell1, nCell2 |  |
| Final condition | Active cell |  | nCell2 |  |
| E-UTRA RF Channel Number |  |  | 1 | One carrier frequency is used for eCell. |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211 [16] |
| NPRACH Configuration |  |  | NPRACH.R-2 | As specified in A.3.18 |
| NPDCCH repetition level |  |  | 16 | The value shall be used for all cells in the test. |
| N310 |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 15000 | RRC re-establishment timer |
| DRX |  |  | OFF |  |
| T1 |  | s | 5 |  |
| T2 |  | ms | 400 |  |
| T3 |  | s | 15 |  |

Table A.6.1.23.1-2: nCell 1, nCell 2 specific test parameters for TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage

| Parameter | Unit | nCell 1 |  |  | nCell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | kHz | 200 |  |  | 200 |  |  |
| PRB location within eCell | - | eCell1 BWchannel 10MHz: 30 |  |  | eCell1 BWchannel 10MHz: 35 |  |  |
| NPDSCH parameters |  | eCell1 BWchannel 10MHz: R.14 NB-TDD |  |  | eCell1 BWchannel 10MHz: R.14 NB-TDD |  |  |
| NPDCCH parameters |  | eCell1 BWchannel 10MHz: R.26 NB-TDD |  |  | eCell1 BWchannel 10MHz: R.26 NB-TDD |  |  |
| NPBCH_RA | dB | 0 |  |  | 0 |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.6.1.23.1-3 |  |  |  |  |  |
|  | dB | 7 | -Infinity | -Infinity | -Infinity | 4 | 4 |
| Note2 | dB | 7 | -Infinity | -Infinity | -Infinity | 4 | 4 |
| NRSRP Note2 | dBm/15 kHz | -91 | -Infinity | -Infinity | -Infinity | -94 | -94 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to nCell 1 | s | - |  |  | 3 |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.6.1.23.1-3: eCell 1 specific test parameters for TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage

| Parameter | Unit | eCell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| BWchannel | MHz | 10 |  |  |
| NOCNG Patterns |  | BWchannel 10MHz: NOP.1 TDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
|  | dB | 4 | 4 | 4 |
| Note2 | dB | 4 | 4 | 4 |
| RSRP Note2 | dBm/15 kHz | -94 | -94 | -94 |
| Propagation Condition |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . Note 3: Es/Iot and RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

A.6.1.23.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send NPRACH preambles to cell 2 for sending the RRCConnectionReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NB-IoT TDD inter frequency cell shall be less than 12 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE-re-establish_delay_NB-IoT.

Where:

-	TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The NPRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

-	TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB-IoT + TSI_NB-IoT + TRACH_NB-IoT

-	NNB-Iot-freq = 2

-	Tsearch_NB-IoT = 1400 ms

-	TSI_NB-IoT = 8320 ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 [2] for the target NB-IoT TDD cell.

-	TPRACH_NB-IoT = 80 ms; it is the additional delay caused by the random access procedure.

<< End of Change 1 >>