3GPP TSG-RAN4 Meeting #90bis 	R4-1903685

# Xi’an, China, 8th – 12th Apr, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 36.133 | CR | 6381 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance on side conditions for NSSS measurement accuracy requirements R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Perf |  |  |  |  |  | Date: |  |  | 2019-03-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current version of spec wrongly addresses that the side conditions for NB1 measurement accuracy is not applied for NSSS-based measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | changes are listed below, - Correct the applicability for side conditions in measurement accuracy requirements for NB1 UE - Correct editorial errors |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change 1 >>

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1

The requirements for absolute accuracy of NRSRP in this clause apply to a cell on the same frequency as that of the serving cell for UE Category NB1 for stand-alone, guard-band and in-band deployments. For a UE capable of NSSS-based RRM measurement, provided that nsss-NumOccDiffPrecoders value n1 has been indicated by higher layers, the accuracy requirement as specified in Table. 9.1.22.1-2 shall apply. Otherwise, the accuracy requirement as specified in Table 9.1.22.1-1 shall apply.

The accuracy requirements in Table 9.1.22.1-1 and Table 9.1.22.1-2 are valid under the following conditions:

Narrowband reference signals are transmitted either from one or two ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

NRSRP|dBm according to Annex B.3.25 for a corresponding Band

At least 1 DL subframe per radio frame of measured cell is available at the UE for NRSRP measurement assuming measured cell is identified cell.

Table 9.1.22.1-1: NRSRP Intra frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 6 | 9 | -6 dB | NFDD_G, NTDD_G | -122.9 | N/A | -70 |
| 8 | 11 | -6 dB | NFDD_G, NTDD_G | N/A | -70 | -50 |
| 10.3 | 13.3 | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | - 122.9 | N/A | -70 |
| 12.3 | 15.3 | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.22.1-2: NRSRP Intra frequency absolute accuracy for UE Category NB1 under NSSS-based measurement

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| [4] | [7] | -6 dB | NFDD_G, NTDD_G | -122.9 | N/A | -70 |
| [6] | [9] | -6 dB | NFDD_G, NTDD_G | N/A | -70 | -50 |
| [6] | [9] | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | -122.9 | N/A | -70 |
| [8] | [11] | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

9.1.22.2	Void

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1

The requirements for absolute accuracy of NRSRQ in this clause apply to a cell on the same frequency as that of the serving cell for NB-IoT UE for stand-alone, guard-band and in-band deployments. For a UE capable of NSSS-based RRM measurement, provided that nsss-NumOccDiffPrecoders value n1 has been indicated by higher layers, the accuracy requirement as specified in Table. 9.1.22.3-2 shall apply. Otherwise, the accuracy requirement as specified in Table 9.1.22.3-1 shall apply.

The accuracy requirements in Table 9.1.22.3-1 and Table 9.1.22.3-2 are valid under the following conditions:

Narrowband reference signals are transmitted either from one or two antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

NRSRP|dBm according to Annex B.3.25 for a corresponding Band

At least 1 DL subframe per radio frame of measured cell is available at the UE for NRSRQ measurement assuming measured cell is identified cell.

Table 9.1.22.3-1: NRSRQ Intra frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel |
| 5.2 | 8.2 | -3 dB | NFDD_G, NTDD_G | -122.9 | -50 |
| 7.2 | 10.2 | -6 dB | Note 2 | Note 2 | Note 2 |
| 9.5 | 12.5 | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | -122.9 | -50 |
| 11.5 | 14.5 | -15≤Ês/Iot≤--6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.22.3-2: NRSRQ Intra frequency absolute accuracy for UE Category NB1 under NSSS-based measurement

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| [3.2] | [6.2] | -6 dB | NFDD_G, NTDD_G | -122.9 | N/A | -70 |
| [5.2] | [8.2] | -6 dB | NFDD_G, NTDD_G | N/A | -70 | -50 |
| [5.2] | [8.2] | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | -122.9 | N/A | -70 |
| [7.2] | [10.2] | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

9.1.22.4	Void

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1

The requirements for absolute accuracy of NRSRP in this clause apply to a cell that has different carrier frequency from the serving cell. For a UE capable of NSSS-based RRM measurement, provided that nsss-NumOccDiffPrecoders value n1 has been indicated by higher layers, the accuracy requirement as specified in Table. 9.1.22.5-2 shall apply. Otherwise, the accuracy requirement as specified in Table 9.1.22.5-1 shall apply.

The accuracy requirements in Table 9.1.22.5-1 and Table 9.1.22.5-2 are valid under the following conditions:

Narrowband reference signals are transmitted either from one or two antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

NRSRP|dBm according to Annex B.3.26 for a corresponding Band

At least 1 DL subframe per radio frame of measured cell is available at the UE for NRSRP measurement assuming measured cell is identified cell.

Table 9.1.22.5-1: NRSRP Inter frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 6 | 9 | -6 dB | NFDD_G, NTDD_G | -122.9 | N/A | -70 |
| 8 | 11 | -6 dB | NFDD_G, NTDD_G | N/A | -70 | -50 |
| 10.3 | 13.3 | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | -122.9 | N/A | -70 |
| 12.3 | 15.3 | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.22.5-2: NRSRP Inter frequency absolute accuracy for UE Category NB1 under NSSS-based measurement

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| [4] | [7] | -6 dB | NFDD_G, NTDD_G | -122.9 | N/A | -70 |
| [6] | [9] | -6 dB | NFDD_G, NTDD_G | N/A | -70 | -50 |
| [6] | [9] | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | -122.9 | N/A | -70 |
| [8] | [11] | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

9.1.22.6	Void

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1

The requirements for absolute accuracy of NRSRQ in this clause apply to a cell that has different carrier frequency from the serving cell. For a UE capable of NSSS-based RRM measurement, provided that nsss-NumOccDiffPrecoders value n1 has been indicated by higher layers, the accuracy requirement as specified in Table. 9.1.22.7-2 shall apply. Otherwise, the accuracy requirement as specified in Table 9.1.22.7-1 shall apply.

The accuracy requirements in Table 9.1.22.7-1 and Table 9.1.22.7-2 are valid under the following conditions:

Narrowband reference signals are transmitted either from one or two antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

NRSRP|dBm according to Annex B.3.26 for a corresponding Band

At least 1 DL subframe per radio frame of measured cell is available at the UE for NRSRQ measurement assuming measured cell is identified cell.

Table 9.1.22.7-1: NRSRQ Inter frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel |
| 5.2 | 8.2 | -3 dB | NFDD_G, NTDD_G | -122.9 | -50 |
| 7.2 | 10.2 | -6 dB | Note 2 | Note 2 | Note 2 |
| 9.5 | 12.5 | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | -122.9 | -50 |
| 11.5 | 14.5 | -15≤Ês/Iot≤--6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.22.7-2: NRSRQ Inter frequency absolute accuracy for UE Category NB1 under NSSS-based measurement

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| [3.2] | [6.2] | -6 dB | NFDD_G, NTDD_G | -122.9 | N/A | -70 |
| [5.2] | [8.2] | -6 dB | NFDD_G, NTDD_G | N/A | -70 | -50 |
| [5.2] | [8.2] | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | -122.9 | N/A | -70 |
| [7.2] | [10.2] | -15≤Ês/Iot≤--6 dB | NFDD_G, NTDD_G | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

9.1.22.8	Void

<< End of Change 1 >>