3GPP TSG-RAN WG4 Meeting #90bis	R4-1903925

# Xi’an, China, April 8th – 12th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for L1-RSRP Reporting Delay Requirements in section 9.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction CR to section 9.5.3 L1-RSRP measurements |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces report delay for the initial periodic L1-RSRP report, initial semi-persisten L1-RSRP report and aperiodic report requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Section 9.5.3 on L1-RSRP measurment reporting is inaccurate and not aligned with agreements in other WG’s. There will be no reporting delay requirements for the initial L1-RSRP reportings. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 9.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause

### 9.5.3	Measurement Reporting Requirements

The UE shall only send L1-RSRP reports for report configurations configured for the active BWP.

The UE shall report the L1-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1dB step size according to section 10.1.x if nrofReportedRS is configured to one. Otherwise, and additionally if groupBasedBeamReporting is enabled the UE shall use differential L1-RSRP based reporting as defined in section 10.1.x.

#### 9.5.3.1	Periodic Reporting

Reported L1-RSRP measurements contained in periodic L1-RSRP reports shall meet the requirements in sections 10.x, 10.y and 10.z, respectively.

The UE shall not send any periodic L1-RSRP reports for a non-active BWP.

The L1-RSRP reporting delay is defined as the time between a request or command that will trigger an L1-RSRP report is received by the UE and the point when the UE starts to transmit the L1-RSRP report over the air interface. This requirement assumes that the measurement report is not delayed by other PUCCH or PUSCH signaling on the UL. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The UE shall be able to transmit periodic L1-RSRP measurement report at every UL resource allocated for the report. The UE shall be able to transmit the initial periodic measurement report at n + TBWPswitchDelay + TBM_Measurement_Period when triggered at n. TBWPswitchDelay is BWP switch delay, in ms, as defined in sections 8.6.2 and 8.6.3. TBM_Measurement_Period is the L1-RSRP measurement period as defined in table 9.5.4.1-1 and 9.5.4.1-2 (for SSB based L1-RSRP), and table 9.5.4.2-1 and 9.5.4.2-2 (for CSI-RS based L1-RSRP) with M=1.

.

#### 9.5.3.2	Semi-Persistent Reporting

Reported L1-RSRP measurements contained in Semi-Persistent L1-RSRP reports shall meet the requirements in sections 10.x, 10.y and 10.z, respectively. This requirement applies for semi-persistent L1-RSRP reports send on PUSCH or PUCCH.

The UE shall not send any semi-persistent L1-RSRP reports on PUSCH, as long as no DCI request have been received.

The UE shall not send any semi-persistent L1-RSRP reports on PUCCH, as long as no activation command [38.321] have been received.

The L1-RSRP reporting delay is defined as the time between a request or command that will trigger an L1-RSRP report is received by the UE and the point when the UE starts to transmit the L1-RSRP report over the air interface. This requirement assumes that the measurement report is not delayed by other PUCCH or PUSCH signaling on the UL. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The UE shall be able to transmit semi-persistent L1-RSRP measurement report at every UL resource allocated for the report. The UE shall be able to transmit the initial semi-persistent measurement report at n + Ttrigger_Delay + TBM_Measurement_Period when triggered at n. Ttrigger_Delay is delay, in ms, it takes the UE to process the DCI or MAC command. TBM_Measurement_Period is the L1-RSRP measurement period as defined in table 9.5.4.1-1 and 9.5.4.1-2 (for SSB based L1-RSRP), and table 9.5.4.2-1 and 9.5.4.2-2 (for CSI-RS based L1-RSRP) with M=1.

#### 9.5.3.3	Aperiodic Reporting

Reported L1-RSRP measurements contained in Aperiodic triggered, Aperiodic triggered periodic and Aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in sections 10.x, 10.y and 10.z, respectively.

The UE shall not send any aperiodic L1-RSRP reports, as long as no DCI trigger have been received.

The L1-RSRP reporting delay is defined as the time between a DCI request that will trigger an L1-RSRP report and the point when the UE starts to transmit the L1-RSRP report over the air interface.

The UE shall be able to transmit the aperiodic measurement report at n + Ttrigger_Delay + TBM_Measurement_Period when triggered at n. Ttrigger_Delay is delay, in ms, it takes the UE to process the DCI or MAC command. TBM_Measurement_Period is the L1-RSRP measurement period as defined in table 9.5.4.1-1 and 9.5.4.1-2 (for SSB based L1-RSRP), and table 9.5.4.2-1 and 9.5.4.2-2 (for CSI-RS based L1-RSRP) with M=1.

Next Modified Subclause