3GPP TSG-RAN WG4 Meeting#90bis                	R4-1904102

Xi’an, China, April 8th – 12th 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Demodulation performance for NPRACH TDD preamble formats

Agenda item:	5.3.3

Document for:	Agreement

Introduction

Preliminary missed detection requirements for NPRACH TDD preamble formats 0, 1, 0-a and 1-a were agreed at RAN4 #90 in [1].

This contribution lists the proposals for performance requirements from the sourcing companies based on simulation results and provides a reasoning for the deviation from the preliminary requirements.

Simulation results

Simulations were carried out for NPRACH TDD preamble formats 0, 1, 0-a and 1-a as specified in 3GPP TS 36.211. The results including impairments are shown in Table 1.

Table 1: Simulation results for missed detection for NPRACH TDD preamble formats 0, 1, 0-a and 1-a

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR[dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 0-a | Preamble format 1-a |
| 1 | 2 | 8 | AWGN | 0 | 5.7 | 2.8 | 4.5 | 1.5 |
|  |  |  | EPA1 Low | 200 Hz | 14.6 | 10.2 | 12.9 | 9.2 |
|  |  | 32 | AWGN | 0 | 1.2 | -1.8 | -0.3 | -3.3 |
|  |  |  | EPA1 Low | 200 Hz | 9.3 | 3.8 | 7.9 | 2.7 |

Discussion

Performance Comparison

The above results are compared to preliminary performance requirements in 36.104 as shown in Table 2.

Table 2: Preliminary requirement for NPRACH TDD preamble formats 0, 1, 0-a and 1-a

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR[dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 0-a | Preamble format 1-a |
| 1 | 2 | 8 | AWGN | 0 | [-17.8] | [-20.4] | [-19.3] | [-23.0] |
|  |  |  | EPA1 Low | 200 Hz | [-8.6] | [-11.1] | [-9.5] | [-13.3] |
|  |  | 32 | AWGN | 0 | [-20.2] | [-22.8] | [-20.1] | [-23.0] |
|  |  |  | EPA1 Low | 200 Hz | [-13.6] | [-17.5] | [-13.3] | [-17.7] |

It is observed that there is a considerable difference of 20 to 25 dB both for AWGN and EPA1 Low channels. It is unclear, due to what reason NPRACH TDD preambles exhibit that good performance, captured in stringent preliminary requirements, which is also superior by around 15 dB over NPRACH FDD performance, although the same modulation scheme (QPSK) is used for both.

Let’s compare the NPRACH demodulation performance for TDD preamble 0 against FDD preamble 0 in an analytical way. The NPRACH demodulation performance is achieved by coherently combining symbols in the same symbol group and non-coherently combining symbols of different symbol groups. The non-coherent gain is assumed to be half of the coherent gain. The comparison analysis is shown in Table 3 for the AWGN channel.

Table 3: Comparison of preamble gain for NPRACH preamble format 0 for FDD and TDD (AWGN).

| Parameter | FDD preamble format 0 | TDD preamble format 0 |
| --- | --- | --- |
| Number of symbols in symbol group N | 5 | 1 |
| Number of symbol groups per preamble P | 4 | 4 |
| Number of subcarriers in 45 kHz narrow band | 12 | 12 |
| Coherent gain in symbol group | 7 dB (=10 * log10(5)) | 0 dB |
| Non-coherent gain between symbol groups | 3 dB (= 0.5 * 10 * log10(4)) | 3 dB (= 0.5 * 10 * log10(4)) |
| Total gain of preamble over symbol | 10 dB | 3 dB |

Table 3 depicts a difference of 7 dB in the total gain for the above taken assumptions. This means that for AWGN channel a relative loss of 7 dB is observed for NPRACH TDD preamble format 0 versus NPRACH FDD preamble format 0. Thus, it is not plausible to assume improved performance for NPRACH TDD preamble 0 versus NPRACH FDD preamble 0 as given in the preliminary requirements.

It is not plausible to assume improved performance for NPRACH TDD preamble 0 versus FDD preamble 0 as given in the preliminary requirements.

Agree the proposed figures from Nokia in Table 1 for updated preliminary requirements.

Conclusion

This contribution lists our proposals for NPRACH performance requirements for TDD preamble formats 0,1, 0-a, 1-a, as introduced in 3GPP Rel-15. It is proposed to agree the following proposal:

- Agree the proposed figures from Nokia in Table 1 for updated preliminary requirements.

The companion CRs are submitted in [2] and [3].

References

- R4-1902061, CR 36.104-4834 Addition of demodulation performance for new NPRACH formats, Huawei, HiSilicon

- R4-1904104, CR 36.104-4854 Corrections to demodulation performance for new NPRACH formats (Rel-15), source: Nokia, Nokia Shanghai Bell

- R4-1904107, CR 36.141-1212 Corrections to demodulation performance for new NPRACH formats (Rel-15), source: Nokia, Nokia Shanghai Bell