3GPP TSG-RAN4 Meeting # 90bis	R4-1904214

# Xian, China,  8th - 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS37.145-2 | CR | 0101 | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of RIB declaration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RIB declaration is confusing, as it can be both in far-field or near-field, depending on which requirement is considered and method used for measurement. Reverberation chamber cannot be included under this declaration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of the RIB declarations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification leads to a confusing manufacturer declaration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.141-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--- start of changes--------

Table 4.10-2: OTA AAS BS manufacturers declarations for radiated test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D11.1 | AAS BS requirements set | Declaration of either hybrid AAS BS architecture conforming to the hybrid requirement set, or OTA AAS BS architecture conforming to the OTA requirement set. |
| D11.2 | BS class | BS Class of the AAS BS, declared as Wide Area BS, Medium Range BS, or Local Area BS. |
| D11.3 | OTA coverage range | Declared as a single range within which selected TX OTA requirements are intended to be met. NOTE 1: OTA coverage range is used for conformance testing of such TX OTA requirements as occupied bandwidth, frequency error, TAE or EVM. |
| D11.4 | OTA coverage range reference direction | The direction describing the reference direction of the OTA converge range (D11.2). NOTE 2: The OTA coverage reference direction may be the same as the Reference beam direction pair (D9.7) but does not have to be. |
| D11.5 | OTA coverage range maximum directions | The directions corresponding to the following points: 1) The direction determined by the maximum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. |
| D11.6 | The rated carrier OTA BS power, Pmax,c,TRP | Pmax,c,TRP is declared as TRP OTA power per carrier, declared per supported operating band, per supported RAT. |
| D11.7 | Worst-case side of the AAS BS on which the co-location test antenna is placed | Declare the worst-case side of the AAS BS on which the co-location test antenna is placed and test will be done only on the declared side. |
| D11.8 | Spurious emission category | Declare the OTA AAS BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [16]. |
| D11.9 | Geographic area support | The manufacturer shall declare the regions the OTA AAS BS may operate in. e.g. CEPT. |
| D11.10 | Band 20 or Band XX support, operating in geographical areas allocated to broadcasting (DTT) | If the OTA AAS BS supports Band 20/XX or Band 32/XXXII, the manufacturer shall declare if the OTA AAS BS may operate in geographical areas allocated to broadcasting (DTT). |
| D11.11 | Band 20 or Band XX support, emission level for channel N (PEM,N) | If the OTA AAS BS supports Band 20 or Band XX and has been declared to operate in geographical areas allocated to broadcasting (DTT; declaration D11.7), the emission level for channel N (as defined in annex G of 3GPP TS 36.104 [4]) shall be declared. |
| D11.12 | Band 20 or Band XX support, Maximum output Power in 10 MHz (P10MHz) | If the OTA AAS BS supports Band 20 or Band XX and has been declared to operate in geographical areas allocated to broadcasting (DTT; declaration D11.7), the maximum output power in 10 MHz (annex G of 3GPP TS 36.104 [4]) shall be declared. |
| D11.13 | Band 32 or Band XXXII support, Declared emission level in Band 32/XXXII (PEM,B32,ind) | If the OTA AAS BS supports Band 32 or Band XXXII and has been declared to operate in geographical areas allocated to broadcasting (DTT; declaration D11.7), the emission level in Band 32/XXXII (PEM,B32,ind, ind = a, b, c, d, e) shall be declared. |
| D11.14 | Co-existence with other systems | The manufacturer shall declare whether the OTA AAS BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or PHS operating in another operating band are deployed. |
| D11.15 | Co-location with other base stations | The manufacturer shall declare whether the OTA AAS BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD and/or E-UTRA operating in another operating band. |
| D11.16 | void | void |
| D11.17 | Single or multiple carrier | OTA AAS BS capability to operate with a single carrier (only) or multiple carriers. Declared per supported operating band, per RAT, per RIB. |
| D11.18 | Maximum number of supported carriers per band | Maximum number of supported carriers. Declared per supported operating band, per RAT, per RIB. |
| D11.19 | Total maximum number of supported carriers | Maximum number of supported carriers for all supported operating bands. Declared per RIB. |
| D11.20 | Other band combination multi-band restrictions | Declare any other limitation under simultaneous operation in the declared band combinations (D9.16), which have any impact on the test configuration generation. |
| D11.21 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by an OTA AAS BS in a particular operating band. Declared per RIB (D11.13). |
| D11.22 | Maximum supported power difference between carriers | Maximum supported TRP power difference between carriers in each supported operating band. Declared per RIB. |
| D11.23 | Maximum supported power difference between carriers is different operating bands | Maximum supported power difference between any two carriers in any two different supported operating bands. Declared per operating bands combination (D9.16). |
| D11.24 | UTRA FDD MIMO support | Number of 'antennas' supported by the UTRA FDD MIMO mode (i.e. 2 or 4). Declared per supported UTRA FDD operating band (D9.4). NOTE 3: The concept of "antenna 2", "antenna 3" and "antenna 4" is described in 3GPP TS 25.104 [2]. |
| D11.25 | UTRA Inner loop power control dynamic range | Power control dynamic range for UTRA inner loop power control. Declared per supported UTRA FDD operating band, per RIB. |
| D11.26 | Inter-band CA or inter-band HSDPA | Declaration of operating band combinations supporting inter-band CA or multi-band HSDPA. Declared per operating band combination (D9.16). |
| D11.27 | Intra-band contiguous CA or intra-band contiguous HSDPA | Declaration of operating band(s) supporting intra-band contiguous CA, or intra-band contiguous HSDPA. Declared per operating band with CA support. |
| D11.28 | Intra-band non-contiguous CA or intra-band contiguous HSDPA | Declaration of operating band(s) supporting intra-band non-contiguous CA, or intra-band non-contiguous HSDPA. Declared per operating band with CA support. |
| D11.29 | OTA REFSENS RoAoA | The REFSENS RoAoA associated with the receiver target reference direction (D11.30). |
| D11.30 | OTA REFSENS receiver target reference direction | An associated direction inside the OTA REFSENS RoAoA (D11.29). |
| D11.31 | OTA REFSENS conformance test directions | Four conformance test directions for the OTA REFSENS: 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the receiver target reference direction. |
| D11.32 | Supported frequency range of the NR operating band | List of supported frequency ranges representing fractional bandwidths (FBW) of operating bands with FBW larger than 6%. |
| D11.33 | Rated beam EIRP at lower frequency range of the fractional bandwidth (Prated,c,FBWlow ) | The rated EIRP level per carrier at lower frequency range of the fractional bandwidth (Prated,c,FBWlow ), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D9.9), as well as the reference beam direction pair (D9.7). Declared per beam for all supported frequency ranges (D11.32). NOTE 13: if D11.33 is declared for certain frequency range (D11.32), there shall be no “Rated beam EIRP” declaration (D9.10) for the operating band containing that particular frequency range. |
| D11.34 | Rated beam EIRP at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh ) | The rated EIRP level per carrier at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D9.9), as well as the reference beam direction pair (D9.7). Declared per beam for all supported frequency ranges in (D11.32). NOTE 14: if D11.34 is declared for certain frequency range (D11.32), there shall be no “Rated beam EIRP” declaration (D9.10) for the operating band containing that particular frequency range. |

--- end of changes -------