3GPP TSG-RAN WG4 Meeting #90bis	R4-1904273

# Xi'an, China, 8 April 2019 - 12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | xxxx | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-4 on SNR, Es and Noc setup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SNR, Noc and Es definition for FR1 and FR2 requirements is incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce description of Mode 1 and 2 side conditions (with and without external noise) - Clarify the SNR definition for Mode 1 side conditions for conducted and radiated requirements - Add Noc calculation for multi-band capable devices for radiated requirements - Add placeholder for Es values definition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SNR, Noc and Es definition for conducted and radiated requirements is incomplete and requirements cannot be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.4, 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Es	The averaged received energy per RE of the wanted signal during the useful part of the symbol, i.e. excluding the cyclic prefix, at the UE antenna connector; average power is computed within a set of REs used for the transmission of physical, divided by the number of REs within the set

μ	Subcarrier spacing configuration as defined in TS 38.211 [9, Section 4.2]

Noc	The power spectral density of a white noise source with average power per RE normalized to the subcarrier spacing as defined in Section 4.4.3 for conducted requirements and Section 4.5.3 for radiated requirements

END OF CHANGE 1

START OF CHANGE 2

## 4.4	Conducted requirements

### 4.4.0	Introduction

The requirements are defined for the following modes of side conditions:

-	Mode 1: Side conditions with external noise source

-	Wanted signal with power level Es is transmitted.

-	External white noise source with power spectral density Noc is used.

-	Es and Noc levels are selected to achieve target SNR as described in Clause 4.4.2.

-	Mode 2: Noise free side conditions

-	Wanted signal with power level Es is transmitted.

-	No external noise transmitted.

### 4.4.1	Reference point

The reference point for SNR, Es and Noc of DL signal is the UE antenna connector or connectors.

### 4.4.2	SNR definition

For Mode 1 side conditions conducted UE demodulation and CSI requirements the SNR is defined as:

SNR=j=1NRXEEs(j)j=1NRXNoc(j)

where

-	NRX denotes the number of receiver antenna connectors and the superscript receiver antenna connector j.

-	The above SNR definition assumes that the REs are not precoded, and does not account for any gain which can be associated to the precoding operation.

-	Es denotes the averaged received energy per resource element (EPRE) of the wanted signal.

-	Unless otherwise stated, the SNR refers to the SSS wanted signal.

-	The downlink SSS transmit power is defined as the linear average over the power contributions in [W] of all resource elements that carry the SSS within the operating system bandwidth.

-	The power ratio of other wanted signals to the SSS is defined in Clause C.3.1.

-

Noc denotes the power spectral density of a white noise source, with average power per RE normalized to the subcarrier spacing and defined in Clause 4.4.3.

### 4.4.3	Noc

The Noc power spectrum density for Mode 1 side conditions UE demodulation and CSI requirements requirements shall be larger or equal to the minimum Noc power level defined in this clause.

### <Editor’s note: Minimum Noc values and derivation procedure are TBD>

### 4.4.4	Es

The Es power spectrum density for Mode 2 side conditions UE demodulation and CSI requirements shall be larger or equal to the minimum Es power level defined in this clause.

<Editor’s note: Minimum Es values and derivation procedure are TBD>

## 4.5	Radiated requirements

### 4.5.0	Introduction

The requirements are defined for the following modes of side conditions:

-	Mode 1: Side conditions with external noise source

-	Wanted signal with power level Es is transmitted.

-	External white noise source with power spectral density Noc is used.

-	Es and Noc levels are selected to achieve target SNR as described in Clause 4.5.2.

-	Mode 2: Noise free side conditions

-	Wanted signal with power level Es is transmitted.

-	No external noise transmitted.

### 4.5.1	Reference point

The reference point for SNR, Es and Noc of DL signal from the UE perspective is the input of UE antenna array.

Figure 4.5.1-1: Reference point for radiated Demodulation and CSI requirements

### 4.5.2	SNR definition

For Mode 1 side conditions UE demodulation and CSI requirements, the Minimum performance requirement in clause 7, 8, 9 and 10 are defined relative to the baseband SNR level SNRBB. The SNR at the reference point is defined as

SNR = SNRBB + ∆BB

where ∆BB is specified in clause 4.5.3.

The reference point SNR is defined as:

SNR=j=1NRXEs(j)j=1NRXNoc(j)

SNR<signal>=j=1NRXE<signal>(j)j=1NRXNoc(j)

-	NRX denotes the number of receiver reference points, and the super script receiver reference point j.

-	The above SNR definition assumes that the REs are not precoded, and does not account for any gain which can be associated to the precoding operation.

-	EsE<signal> denotes the averaged received energy per resource element (EPRE) of the wanted signal.

-	Unless otherwise stated, the SNR refers to the SSS wanted signal.

-	The downlink SSS transmit power is defined as the linear average over the power contributions in [W] of all resource elements that carry the SSS within the operating system bandwidth.

-	The power ratio of other wanted signals to the SSS is defined in Clause C.3.1.

-	Noc denotes the power spectral density of a white noise source, with average power per RE normalized to the subcarrier spacing and defined in Clause 4.5.3.

### 4.5.3	Noc

#### 4.5.3.1	Introduction

For Mode 1 side conditions radiated testing of demodulation and CSI requirements it is not feasible in practice to use signal levels high enough to make the noise contribution of the UE negligible. Demodulation requirements are, therefore, specified with the applied noise higher than the UE peak EIS level in TS 38.101-2 [7] by a defined amount, so that the impact of UE noise floor is limited to no greater than a value ∆BB at the specified Noc level. As UEs have EIS levels that are dependent on operating band and power class, Noc level is dependent on operating band and power class.

#### 4.5.3.2	Noc for NR operating bands in FR2

Values for Noc according to operating band and power class for single carrier requirements are specified in Table 4.5.3.2-1 for ∆BB =1dB.

Table 4.5.3.2-1: Noc power level for different UE power classes and frequency bands

| Operating band | UE Power class |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| n257 | -166.8 | -163.8 | -157.6 | -166.3 |
| n258 | -166.8 | -163.8 | -157.6 | -166.3 |
| n260 | -163.8 |  | -155.0 | -164.3 |
| n261 | -166.8 | -163.8 | -157.6 | -166.3 |
| Note 1: Noc levels are specified in dBm/Hz |  |  |  |  |

  - For PC3 multi-band devices, the Noc power level (NocMB) shall increase by multi-band relaxation defined in TS 38.101-2 [2] Table 6.2.1.3-4.

  - NocMB = NocSB + ΣMBP

-	NocSB is the Noc defined in Table 4.5.3.2-1

-	ΣMBP values are specified in TS 38.101-2 [16].

<Editor’s note: handling of Carrier Aggregation is FFS >

#### 4.5.3.3	Derivation of Noc values for NR operating bands in FR2

The Noc values in Table 4.5.3.2-1 are based on REFSENS for the operating band and on the UE Power class, and taking a baseline of UE Power class 3 in Band n260.

Noc = REFSENSPC3, n260, 50MHz -10Log10(SCSREFSENS x PRBREFSENS x 12) – SNRREFSENS + ∆thermal

where:

-	REFSENSPC3, n260, 50MHz is the REFSENS value in dBm specified for Power Class 3 UE in Band n260 for 50MHz Channel bandwidth in TS 38.101-2 [7, Table 7.3.2.3-1].

-	SCSREFSENS is a subcarrier spacing associated with NRB for 50MHz in TS 38.101-2 [7, Table 5.3.2-1], chosen as 120 kHz.

-	PRBREFSENS is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-2 [7, Table 5.3.2-1] and is 32.

-	12 is the number of subcarriers in a PRB

-	SNRREFSENS = -1 dB is the SNR used for simulation of REFSENS

-	∆thermal is the amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB.

The calculated Noc value for the baseline of UE Power class 3 in Band n260 is rounded to -155 dBm/Hz.

The following methodology to define the Noc level for UE power class X (PC_X) and operating band Y (Band_Y) is used for the single carrier case and single band devices:

Noc(PC_X, Band_Y) = -155 dBm/Hz + REFSENSPC_X, Band_Y, 50MHz – REFSENSPC3, n260, 50MHz

where REFSENS values are specified in TS 38.101-2 [7].

### 4.5.4	Angle of arrival

Unless otherwise stated, the downlink signal and noise are aligned to arrive in the UE Rx beam peak direction as defined in TS 38.101-2 [7].

### 4.5.5	Es

<Editor’s note: Minimum Es values for Mode 2 side condition are TBD>

END OF CHANGE 2