3GPP TSG-RAN WG4 Meeting #90bis	R4-1904382

Xi’an, Shaanxi, China, 8 – 12 April 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	On Rel-16 HST scenarios for BS demodulation

Agenda item:	7.15.5

Document for:	Discussion

# 1	Introduction

During the RAN4#90 meeting, the way forward on BS demodulation scenarios for HST [1] was approved, which captured some agreements and open issues.

This document discusses the open issue on enhanced PUSCH requirements for different mobility scenarios, in particular, the high speed train scenario. It outlines our observations and proposals.

# 2	Discussion

According to [1], the following were agreed:

(Note: preceding agreements are omitted as they are considered as not relevant for the discussion in the document.)

- Define enhanced PUSCH requirements for the following scenarios:

  - High speed train (HST) scenario

  - FFS multi-path fading propagation scenario (i.e., define ETU with Doppler shift more than 600 Hz);

  - FFS moving propagation scenario;

  - Other scenarios are not precluded.

- Define PRACH requirements for HST scenarios is FFS

- Define PUCCH requirements for multi-path fading propagation scenario is FFS

In Rel-15 TS 36.104 B.3, two HST scenarios are defined:

- Scenario 1: Open space

- Scenario 3: Tunnel for multi-antennas

The parameters for the two HST scenarios are shown in Table 1.

Table 1: Parameters for high speed train conditions (Rel-15 TS36.104, B.3)

| Parameter | Value |  |
| --- | --- | --- |
|  | Scenario 1: open space | Scenario 3: Tunnel for multi-antenna |
|  | 1000 m | 300 m |
|  | 50 m | 2 m |
|  | 350 km/h | 300 km/h |
|  | 1340 Hz | 1150 Hz |

Referring to [2], about 90% of the HST railway track is inside tunnels. The remaining 10% is not mentioned but it is assumed to be in open space. As such, 3 different options can be considered for HST scenarios:

Option 1:  Consider a combined open space and tunnel scenario with only one Ds and Dmin, that is

| Parameter | Scenario (combined open space and tunnel) |
| --- | --- |
|  | TBD |
|  | TBD |
|  | 500 km/h |
|  | TBD |

Option 2: Consider to down select Rel-14 HST scenarios to Scenario 3 (tunnel for multi-antennas) only; and may be different considering the Doppler shift rate of change is faster for higher speed

Option 3: Consider open space and tunnel scenarios separately and adopt Rel-14 Scenario 1 (open space) and Scenario 3 (tunnel for multi-antennas) as shown in Table 1; and may be different considering the Doppler shift rate of change is faster for higher speed.

To minimize the number of HST scenarios, either Option 1 or Option 2 is preferred.

# 3	Conclusions

The document has discussed the HST scenario for BS demodulation and 3 different options can be identified:

Option 1:  Consider a combined open space and tunnel scenario with only one Ds and Dmin, that is

| Parameter | Scenario (combined open space and tunnel) |
| --- | --- |
|  | TBD |
|  | TBD |
|  | 500 km/h |
|  | TBD |

Option 2: Consider to down select Rel-14 HST scenarios to Scenario 3 (tunnel for multi-antennas) only; and may be different considering the Doppler shift rate of change is faster for higher speed

Option 3: Consider open space and tunnel scenarios separately and adopt Rel-14 Scenario 1 (open space) and Scenario 3 (tunnel for multi-antennas) as shown in Table 1; and may be different considering the Doppler shift rate of change is faster for higher speed.

Proposal: For PUSCH requirements, it is necessary to choose one of the 3 possible options with a strong preference for Option 1 and Option 2

# References

- R4-1902522, Way forward on BS demodulation scenarios and requirements for HST, Ericsson, NTT DoCoMo

- R4-1900805, High speed scenarios and parameters for 500 km/h velocity, NTT DoCoMo