3GPP TSG RAN WG4 Meeting #90bis	　　　　　　　　　　　　　　　　R4-1904607

Xian, China, 8th – 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for correction of out-of-band blocking exceptions for SUL in TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Out-of-band blocking exceptions approved in R4-1810235 are against the previous RAN4 agreement on the band definition of n77 and n78 filters. The justification given for the exceptions was that n77 and n78 bands may share a common RF filter and that the second order intermodulation product generated from the SUL carrier and a blocker (which would be sufficiently attenuated by an n78 filter but not an n77 filter) may fall onto the n78 DL carrier. RAN4 agreement on the n77 and n78 band definition (as captured in R4-1706893) states, “A UE supporting band X (n78) is not required to support band Z (n77)”. Therefore, RAN4 requirements for the n78 band cannot be derived based on the assumption that UE is also required to support the n77 band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete out-of-band blocking exceptions in subcause 7.6C.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification not aligned with the RAN4 agreement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-------------------- Start of changes #1 --------------------

## 7.6C	Blocking characteristics for SUL

### 7.6C.1	General

### 7.6C.2	In-band blocking for SUL

For SUL operation, the in-band blocking requirement for downlink bands specified in subclause 7.6.2 shall be met.

### 7.6C.3	Out-of-band blocking for SUL

For SUL operation, the out-of-band blocking requirement for downlink bands specified in subclause 7.6.3 shall be met.

| ¦fInterferer- fSUL- fDL¦≤(BWSUL+ BWDL)/2BWSUL BWDL |  |  |
| --- | --- | --- |

### 7.6C.4	Narrow band blocking for SUL

Narrow band blocking is not specified for SUL band combination.

-------------------- End of changes #1 --------------------