3GPP TSG-RAN4 Meeting #90bis	R4-1904692

# Xi’an, China, 8th – 12th  April, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for scheduling availability for RLM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The symbols that partially overlaps with restricted symbols are not considered for intra-band CA. - Some restrictions for SRS/CSI-RS for tracking/CSI-RS for CQI are missing. - It has been agreed in R4-1902032 that requriements for inter-band FR2 CA will not be considered in Rel-15, so the corresponding editor note can be removed. - Unclear wordings |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the scheduling resctriction for intra-band CA. - Correct restrictions for SRS/CSI-RS for tracking/CSI-RS for CQI. - Remove FFS for inter-band FR2 CA - Editorial updates |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Scheduling availability for RLM is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UE which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to radio link monitoring based on SSB as RLM-RS. For UE which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to radio link monitoring based on SSB as RLM-RS.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on SSB symbols to be measured for radio link monitoring.

When intra-band carrier aggregation in FR1 is performed, the scheduling restrictions on FR1 serving PCell or PSCell applies to all serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols. When inter-band carrier aggregation within FR1 is performed, there are no scheduling restrictions on FR1 serving cell(s) in the bands due to radio link monitoring performed on FR1 serving PCell or PSCell in different bands.

#### 8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2

The following scheduling restriction applies due to radio link monitoring on an FR2 serving PCell and/or PSCell.

-	If the RLM-RS is type-D QCLed with active TCI state for PDCCH/PDSCH, and N=1 applies for the RLM-RS as specified in section 8.1.2.2 if the RLM-RS is SSB and in section 8.1.3.2 if the RLM-RS is CSI-RS

-	There are no scheduling restrictions due to radio link monitoring based on SSB or CSI-RS with a same subcarrier spacing as PDSCH/PDCCH.

-	When performing radio link monitoring based on SSB with a different subcarrier spacing than PDSCH/PDCCH, for UE which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to radio link monitoring. For UE which do not support simultaneousRxDataSSB-DiffNumerology [14] the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on SSB symbols to be measured for radio link monitoring.

-	Otherwise

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on RLM-RS symbols to be measured for radio link monitoring.

When intra-band carrier aggregation in FR2 is performed, the scheduling restrictions on FR2 serving PCell or PSCell applies to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

<End of Change 1>