3GPP TSG-RAN4 Meeting #90bis																				R4-1904784

Xi’an, China, 8th – 12th April 2019

Source: 	Huawei

Title: 	Ad Hoc minutes for NR RRM test cases

Agenda Item:	6.11

Document for:	Approval

General (2)

Spec amendment (1)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1904115 | Several corrections to TS 38.133 (multiple clauses affected) | R&S | Return to |

Discussion:

Restructuring A.8 (1)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1904310 | CR 38.133 (A.8) Restructuring of section A.8 TCs E-UTRA standalone | Ericsson | Return to |

Discussion:

1st part in Test categories in Phase I ~ III (21)

IDLE cell reselection test cases (1)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903713 | Maintenance draftCR on idle mode test cases for FR2 (section A.7.1) | Huawei | Revised to |

Discussion:

QC: in principle we need to wait for the X, Y and Z values to derive Noc levels for all the FR2 test cases.

Chair: yes.

NR-NR handovers (#26A) (1)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903397 | draftCR on handover test case in FR1 (section A.6.3.1.1) | Intel | Endorsed |

Discussion:

Huawei: in principle, we think that the TRS is still expected in FR1 by the UE.

Intel: We also think so.

Qualcomm: we probably need that TRS for FR1.

Agreement:

The CSI-RS for tracking is also configured in all the FR1 test cases in CONNECTED mode.

NR handovers to other RATs (1)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903397 | draftCR on test case for handover to E-UTRAN (section A.6.3.1.4) | Intel | Endorsed |

Discussion:

Re-establishment  (4)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903714 | Maintenance draftCR on re-establishment test cases for FR1 (section A.6.3.2.1) | Huawei | endorsed |
| R4-1903715 | Maintenance draftCR on re-establishment test cases for FR2 (A.7.3.2.1) | Huawei | Revised to |
| R4-1903716 | Test case for intra-frequency RRC re-establishment in FR1 without serving cell timing reference (A.6.3.2.1) | Huawei | Endorsed |
| R4-1903717 | Test case for intra-frequency RRC re-establishment in FR2 without serving cell timing reference (A.7.3.2.1) | Huawei | endorsed |

Discussion:

Chair: for the new test case in FR2, we should modify the levels in the next meeting according to agreements.

Random access (1)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903660 | draftCR on msg3 re-transmission test cases(section A.4.3.2, A.5.3., A.6.3.2, A.7.3.2) | MediaTek | Endorsed |

Discussion:

TA accuracy (2)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1904496 | CR on TS38.133 for EN-DC TA accuracy (Section A.4.4.3, Section A.5.4.3) | MediaTek | Endorsed |
| R4-1904497 | CR on TS38.133 for SA TA accuracy (Section A.6.4.3, Section A.7.4.3) | MediaTek | Endorsed |

Discussion:

SSB RLM (4)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903598 | CR on RLM test cases for EN-DC FR1 (Section A.4.5.1) | MediaTek | Endorsed |
| R4-1903599 | CR on RLM test cases for EN-DC FR2 (Section A.5.5.1) | MediaTek | Endorsed |
| R4-1903600 | CR on RLM test cases for SA FR1 (Section A.6.5.1) | MediaTek | Endorsed |
| R4-1903601 | CR on RLM test cases for SA FR2 (Section A.7.5.1) | MediaTek | Endorsed |

Discussion:

CSI RS RLM (3)

Discussions:

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903444 | Discussion on the CSI-RS based RLM test cases | CMCC | Noted |

Proposal 1: for CSI-RS based RLM, the same SNR margins as SSB based RLM can be used, which is 3dB, 5dB, 3dB and 2.5dB for SNR2, 3, 4, 5 respectively.

Discussion:

| R4-1904292 | TC SA CSI-based RLM in FR1 (A.6.5.1) | Nokia | Revised to |
| --- | --- | --- | --- |
| R4-1904293 | TC SA CSI-based RLM in FR2 (A.7.5.1) | Nokia | Revised to |

Discussion:

RLM scheduling restrictions (1)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903718 | Test cases for RLM scheduling restrictions in FR2 (A.5.5.1.9 and A.7.5.1.9) | Huawei | Revised to |

Discussion:

DCM:

QC:

MTK: we need new RMC for that.

Interruptions at DRX transitions (1)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903231 | Updates to test cases for interruption at transitions in EN-DC | CATT | Revised to |

Discussion:

Interruptions at measurements on deactivated SCells (1)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903232 | Updates to test cases for interruption due to deactivated SCell operations in EN-DC | CATT | Revised to |

Discussion:

Interruptions at SCell activation/deactivation (1)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903229 | Updates to test cases for SCell activationdeactivation in SA | CATT | Revised to |

Discussion:

2nd part in Test categories in Phase I ~ III (22)

BWP switching delay and interruptions tests (8)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903230 | Updates to test cases for BWP switching in SA (section A.7.5.6.1) | CATT | Revised to |
| R4-1903661 | draftCR on TS38.133 for BWP switch test case(section A.7.5.6) | MediaTek | Endorsed |
| R4-1904510 | TC A.4.5.6.1.2 EN-DC FR1 DL active BWP switch with SCell | Nokia | Endorsed |
| R4-1904511 | TC A.5.5.6.1.2 EN-DC FR2 DL active BWP switch with SCell | Nokia | Endorsed |

Discussion:

| R4-1904512 | TC A.4.5.6.2.1 EN-DC FR1 DL active RRC BWP switch | Nokia | Noted |
| --- | --- | --- | --- |
| R4-1904513 | TC A.5.5.6.2.1 EN-DC FR2 DL active RRC BWP switch | Nokia | Noted |
| R4-1904514 | TC A.6.5.6.2.1 SA FR1 DL active RRC BWP switch | Nokia | Noted |
| R4-1904515 | TC A.7.5.6.2.1 SA FR2 DL active RRC BWP switch | Nokia | Noted |

Discussion:

MTK: we should remove the editor notes for type 1/2.

BFD and link recovery (5)

Discussions:

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903596 | Discussion on test cases for link recovery | MediaTek | Noted |

Proposal 1: For AWGN channel, the margin between the signal power of candidate beam and the rsrp-Threshold = baseband estimation error + RF margin.

Proposal 2: For a fading channel, the margin between the signal power of candidate beam and the rsrp-Threshold = baseband estimation error + RF margin + margin for variation of fading channel.

Proposal 3: Further additional margin needs to be considered for UE’s Rx beamforming gain in FR2.

Discussion:

Intel: question is why do we need to define accuracy for the tests.

MTK: in CBD we configure the threshold, the point is to verify how accurate the UE can measure.

Huawei: we think it should be that baseband estimation error + RF margin  = the difference between the threshold and the power that in test.

Agreements:

- For FR1,

  - For AWGN channel, the margin between the signal power of candidate beam and the rsrp-Threshold = baseband estimation error + RF margin

  - For a fading channel, the margin between the signal power of candidate beam and the rsrp-Threshold = baseband estimation error + RF margin + margin for variation of fading channel

- For FR2,

  - Further additional margin needs to be considered for UE’s Rx beamforming gain in FR2

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1904294 | TC for CSI-RS based BFD and LR procedures in FR1 in EN-DC (A.4.5.5) | Nokia | Revised to |
| R4-1904295 | TC for CSI-RS based BFD and LR procedures in FR2 in EN-DC (A.5.5.5) | Nokia | Revised to |
| R4-1904296 | TC for CSI-RS based BFD and LR procedures in FR1 in SA(A.6.5.5) | Nokia | Revised to |
| R4-1904297 | TC for CSI-RS based BFD and LR procedures in FR2 in SA (A.7.5.5) | Nokia | Revised to |

Discussion:

SSB BFD Scheduling restrictions (4)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1904298 | TC scheduling restriction during SSB based BFD and LR in FR1 in EN-DC (A.4.5.5.5) | Nokia | Noted |
| R4-1904299 | TC scheduling restriction during SSB based BFD and LR in FR2 in EN-DC (A.5.5.5.5) | Nokia | Noted |
| R4-1904300 | TC scheduling restriction during SSB based BFD and LR in FR1 in SA (A.6.5.5.5) | Nokia | Noted |
| R4-1904301 | TC scheduling restriction during SSB based BFD and LR in FR2 in SA (A.7.5.5.5) | Nokia | Noted |

Discussion:

Intel: RAN4 is waiting for the RAN1 LS for the SSB based BFD. We are not sure we still need this test.

SSB intra cell search and measurement period (2)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903711 | Maintenance draftCR on the intra-frequency cell search and measurement test cases for FR1 (section A.4.6.1 A.6.6.1) | Huawei | Endorsed |
| R4-1903712 | Maintenance draftCR on the intra-frequency cell search and measurement test cases for FR2 (section A.5.6.1 A.7.6.1) | Huawei | Revised to |

Discussion:

SSB inter cell search and measurement period (3)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903399 | draftCR on test cases for inter-frequency measurement in EN-DC with PScell in FR1 (section A.4.6.2) | Intel | Endorsed |
| R4-1903400 | draftCR on test cases for inter-frequency measurement in EN-DC with PScell in FR2 (section A.5.6.2) | Intel | Endorsed |
| R4-1903401 | draftCR on test cases for inter-frequency measurement in SA with PCell in FR1 (section A.6.6.2) | Intel | Endorsed |
| R4-19xxxxx | draftCR on test cases for inter-frequency measurement in SA with PCell in FR2 (section A.7.6.2) | Intel | New tdoc |

Discussion:

3rd part in Test categories in Phase I ~ III (18)

SSB intra RSRP measurement accuracy (4)

Discussions:

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903739 | Discussion on remaining issues on SS-RSRP measurement accuracy test setups | Huawei | Noted |

Proposal 1: Two options are suggested to solve the interference issues in SS-RSRP measurement accuracy tests.

- Option 1: A list of all the interference cell ID combinations are provided, which shall be excluded in SS-RSRP intra-frequency measurement accuracy tests.

  - The SS-RSRP measurement accuracy need to be evaluated under 507528 (=1008*1007/2) cell ID combinations, which will introduce a heavy workload.

- Option 2(Preferred): SSBs from two cells are TDM transmitted

  - The test setup for SS-RSRP intra-frequency measurement accuracy tests need to be modified.

Discussion:

| R4-1903740 | DraftCR on introducing new SSB pattern for intra-freq measurement accuracy tests (section A.3.10) | Huawei | Return to |
| --- | --- | --- | --- |
| R4-1903741 | DraftCR on maintaining SS-RSRP measurement accuracy test for EN-DC (section A.4.7.1.1 and A.5.7.1.1) | Huawei | Return to |
| R4-1903742 | DraftCR on maintaining SS-RSRP measurement accuracy test for SA (sectionA.6.7.1.1 and A.7.7.1.1) | Huawei | Return to |

Discussion:

Ericsson: we prefer to use other PCI than the problematic ones instead of introducing TDM’d SSBs.

Huawei: do you mean to use fixed PCI.

Ericsson: maybe we can address that in the common session if the work load is too huge.

Qualcomm: same understanding with Ericsson. We provide the information to RAN5.

SSB inter RSRP measurement accuracy (2)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903782 | Maintanance of inter-frequency RSRP accuracy test cases for FR1 (section A.4.7.1.2, A.6.7.1.2) | Huawei | Endorsed |
| R4-1903783 | Maintanance of inter-frequency RSRP accuracy test cases for FR2 (section A.5.7.1.2-3, A.7.7.1.2-3) | Huawei | Endorsed |

Discussion:

SSB intra RSRQ measurement accuracy (2)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903439 | Draft CR for Intra-frequency SS-RSRQ Accuarcy Test Cases for EN-DC FR2 (section A.5.7.2) | LGE | Endorsed |
| R4-1903440 | Draft CR for Intra-frequency SS-RSRQ Accuarcy Test Cases for SA FR2 (section A.7.7.2) | LGE | Endorsed |

Discussion:

SSB SINR measurement accuracy (2)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1904498 | CR on TS38.133 for EN-DC SS-SINR tests with PSCell in FR1 (Section A.4.7.3) | MediaTek | Endorsed |
| R4-1904499 | CR on TS38.133 for SA SS-SINR tests with PSCell in FR1 (Section A.6.7.3) | MediaTek | Endorsed |

Discussion:

Beam management: L1 RSRP tests (#29A) (8)

Discussions:

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903597 | Discussion on test cases for L1-RSRP in FR2 | MediaTek |  |
| R4-1903630 | Discussion on test case for L1-RSRP reporting | NTT DoCoMo |  |
| R4-1903784 | Further discussion on L1-RSRP test | Huawei |  |
| R4-1903785 | Way forward on L1-RSRP test | Huawei |  |

Open issues:

Beam mode assumptions

- RAN4 to consider the following options for Rx beam assumption in L1-RSRP tests

  - Option 1: UE is assumed to use rough Rx beam (Huawei)

  - Option 2: UE is assumed to use fine Rx beam, and in the test UE is given enough time to do L3 cell identification before L1-RSRP measurement (Huawei)

  - Option 3: Fine Rx beam should be assumed in test case for L1-RSRP reporting (DCM)

Discussion:

HW: the same with RLM

QC: in RLM it is different there is no need to use fine beam for L1 RSRP reporting.

Intel: fine beam we should use. It is more useful to ensure the performance in L1 rsrp measurement.

MTK: sometimes we use the same set of RS for both RLM and L1-RSRP so we may need consistency.

Test cases

- Whether to define test cases for repetition on

  - Option 1: yes to verify UE Rx beam refinement (DCM)

  - Option 2: no if fine beam is assumed (Huawei)

Discussion:

MTK: to clarify for on cases, there is no reporting.

Huawei: there is rsrp reporting but no index.

Intel: it is independent.

MTK: we are not so sure.

Qualcomm: we don’t know how to test but it should be tested.

Agreement:

FFS in the next meeting if we can test repetition on cases.

- Test with periodic/aperiodic CSI-RS in repetition off cases

  - Option 1: focus on the periodic (MTK)

  - Option 2: delay tests with aperiodic and accuracy tests with periodic (DCM)

Discussion:

MTK: the intention is to reduce the number of cases.

Huawei: so far we only have accuracy defined for periodic CSI-RS.

DCM: we had a WF capturing the agreements. It is for test coverage.

Intel:

Agreement:

Define accuracy tests with periodic.

- TCI states for repetition off cases

  - Option 1: TCI state is configured (MTK)

    - Pointing to SSB (Huawei)

Discussion:

Agreement:

Configure TCI states for repetition off cases and the TCI state is QCL type-D with SSB.

Test methods

- For L1-RSRP delay test, use the same test method as for L3 measurement delay test (Huawei)

Discussion:

Maintenance CRs:

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903786 | Maintanance of L1-RSRP accuracy test cases for FR2 (section A.5.7.4, A.7.7.4) | Huawei | Revised to |

Discussion:

New TCs:

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903787 | Addition of L1-RSRP delay test in FR1 (section A.4.6, A.6.6) | Huawei | Noted |
| R4-1903788 | Addition of L1-RSRP delay test in FR2 (section A.5.6, A.7.6) | Huawei | Noted |

Discussion:

Agreement:

FFS whether to introduce the delay tests for L1 rsrp reporting in the next meeting.

Test categories in Phase IV (10)

TCI switch delay (0)

Discussions:

Inter-RAT E-UTRA measurement accuracies (3)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903743 | DraftCR on RSRP measurement accuracy tests for inter-RAT E-UTRAN (new section A.6.7.5) | Huawei | Endorsed |

Discussion:

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903744 | DraftCR on RSRQ measurement accuracy tests for inter-RAT E-UTRAN (new section A.6.7.6) | Huawei | Endorsed |

Discussion:

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1903745 | DraftCR on RS-SINR measurement accuracy tests for inter-RAT E-UTRAN (new section A.6.7.7) | Huawei | Endorsed |

Discussion:

E-UTRAN cell reselection to NR (1)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1904302 | TC for E-UTRA cell reselection to NR target cell (A.8.x) | Nokia | Revised to |

Discussion:

R&S: We should remove LTE reselect to FR2.

Ericsson: we may think of ways to solve this problem.

Agreement: FFS the test method for LTE reselect to FR2 cases in the next meeting.

E-UTRAN inter-RAT NR cell search and measurement delay (5)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1904353 | Phase IV-47: E-UTRAN inter-RAT NR cell search and measurement delay test case in FR1 | Ericsson | Noted |

Discussion:

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1904354 | Section A.8.4.2.1: Phase IV-47: E-UTRAN inter-RAT NR cell search and measurement delay test case without SSB index and non-DRX in FR1 | Ericsson | Return to |
| R4-1904355 | Section A.8.4.2.2: Phase IV-47: E-UTRAN inter-RAT NR cell search and measurement delay test case without SSB index and in DRX in FR1 | Ericsson | Return to |
| R4-1904356 | Section A.8.4.2.3: Phase IV-47: E-UTRAN inter-RAT NR cell search and measurement delay test case with SSB index and non-DRX in FR1 | Ericsson | Return to |
| R4-1904357 | Section A.8.4.2.4: Phase IV-47: E-UTRAN inter-RAT NR cell search and measurement delay test case with SSB index and in DRX in FR1 | Ericsson | Return to |

Discussion:

Intel: capability should be mentioned regarding to per-FR gap.

Ericsson: the UE has to report.

Qualcomm: can we do reduction of the cases.

Ericsson: it is the same thing in inter-f test cases.

E-UTRAN inter-RAT handover (0)

Discussions:

E-UTRAN inter-RAT NR measurement accuracy (1)

| TDoc | Title | Source | Decision |
| --- | --- | --- | --- |
| R4-1904500 | CR on TS38.133 E-UTRAN – NR inter-RAT measurements test case with FR1 target cell (Section A.8.X) | MediaTek | Revised to |

Discussion:

Fine/Rough beam assumptions ()

Table 1: Suggested AoA setup and rough/fine beam assumption for deriving Noc levels in RRM test cases

| Test case group number | Test purpose | Rough/fine beam | Note |
| --- | --- | --- | --- |
| 1 | EN-DC cell search and L1 measurement period | Rough |  |
| 2 | SA cell search and L1 measurement period | Rough |  |
| 3 | EN-DC Timing accuracy and adjustment | Fine |  |
| 4 | SA Timing accuracy and adjustment | Fine |  |
| 5 | EN-DC TA accuracy | Fine |  |
| 6 | SA TA accuracy | Fine |  |
| 7 | EN-DC SSB RLM for PSCell IS and OOS | Rough |  |
| 9 | SA SSB RLM for PCell IS and OOS | Rough |  |
| 10 | Random access | Rough |  |
| 11 | SSB based Intra-frequency RSRP accuracy for FR1 and FR2 | Rough | L3 |
| 12 | EN-DC SCell activation/deactivation delay | Rough |  |
| 13A | EN-DC CSI RLM for PSCell | Rough |  |
| 13B | SA CSI RLM for PCell | Rough |  |
| 14A | EN-DC interruptions due to DRX transition | Fine |  |
| 14B | EN-DC interruptions due to deactivated SCell operations | Fine assumption for the interrupted cell | Rough assumption for the target SCell if it is interband with the PSCell |
| 17A | Serving NR PSCell and target E-UTRA inter-frequency measurement with LTE PCell | N/A |  |
| 17B | NR Pcell with target inter-RAT E-UTRA measurement | N/A |  |
| 18A | EN-DC NR inter-frequency measurement | Rough |  |
| 18B | SA NR inter-frequency measurement | Rough |  |
| 19 | SSB Inter-frequency RSRP accuracy for FR1 and FR2 | Rough |  |
| 20A | EN-DC interruptions at UL carrier RRC reconfiguration | N/A |  |
| 20B | EN-DC interruptions due to active BWP switching | Fine |  |
| 21A | SA interruptions at SCell addition/release/activation/deactivation | Fine assumption for the interrupted cell |  |
| 21B | SA interruptions at UL carrier RRC reconfiguration | N/A |  |
| 21C | SA interruptions due to Active BWP switching | Fine |  |
| 26A | NR-NR Handovers | Rough |  |
| 26B | NR handovers to other RATs | N/A |  |
| 29A | Beam management: L1-RSRP reporting | Rough |  |
| 29B | Beam management: Beam failure detection and link recovery procedure | Rough |  |
| 31 | Intra-freq RSRQ accuracy for FR1 and FR2 | Rough |  |
| 32 | Inter-freq RSRQ accuracy for FR1 and FR2 | Rough |  |
| 34 | BWP switching interruptions on E-UTRA serving cells in EN-DC | N/A |  |
| 35 | BWP switching delay | Fine |  |
| 36 | NR PSCell addition and release in EN-DC | Rough |  |
| 37 | UL carrier RRC reconfiguration delay | N/A |  |
| 38 | SA RRC_Idle/inactive cell reselection NR to NR (FR1) | N/A |  |
| 39 | SA RRC Idle/inactive cell reselection NR to E-UTRAN (FR1) | N/A |  |