3GPP TSG-RAN WG4 Meeting #90bis	R4-1904866

# Xi’an, China, 8th – 12th Apr, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.133 update test cases for CSI-RS based Beam Failure Detection and Link recovery procedures in FR1(A.5.5.5.3, A.5.5.5.4) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To update test cases for CSI-RS based Beam Failure Detection and Link recovery procedures in FR1 (A.5.5.5.3, A.5.5.5.4) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove some test cases for test time reduction as agreed in RAN4#90 meeting - Update TBD parameters, CSI-RS configuration, DRX configuration, etc - Correct typos |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for CSI-RS based Beam Failure Detection and Link recovery procedures in FR1 are not finished. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.5.5.3, A.5.5.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

#### A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode

##### A.5.5.5.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.5.5.5.3.1-1, A.5.5.5.3.1-2, and A.5.5.5.3.1-3 below. There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.5.5.5.3.1-1 shows the variation of the downlink SNR of the PCell and the SNR of the CSI-RS in set q0 in the active PSCell to emulate CSI-RS based beam failure. Figure A.5.5.5.3.1-1 additionally shows the variation of the downlink SNR of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [2] ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurementswithout gaps.

Table A.5.5.5.3.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, TDD duplex mode, 120 kHz SSB SCS, 100MHz bandwidth |

Table A.5.5.5.3.1-2: General test parameters for FR2 PSCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active E-UTRA PCell |  |  | Cell 1 |  |
| E-UTRA RF Channel Number |  |  | 1 |  |
| Active PSCell |  |  | Cell 2 |  |
| RF Channel Number |  |  | 2 |  |
| Duplex mode | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | A.3.1.2 |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | A.3.10 |
| SMTC Configuration | Config 1 |  | SMTC.1 | A.3.11 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |  |
| csi-RS-Index assigned as RLM RS |  |  | [0] |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  |
| OCNG parameters |  |  | OP.1 | A.3.2.1 |
| CP length |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX |  |  | OFF |  |
| Gap pattern ID |  |  | N.A. |  |
| csi-RS-Index |  |  | 2 | Number of SSB indexes used for beam failure detection |
| rlmInSyncOutOfSyncThreshold |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  |  | TBD | Threshold used for Qout_LR_SSB |
| powerControlOffsetSS |  |  | NA | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | [n2] | see TS 38.321 [7], section 5.17 |
| beamFailureDetectionTimer |  |  | [pbfd4] | see TS 38.321 [7], section 5.17 |
| CSI-RS configuration | Config 1 |  | CSI-RS.3.2 TDD | A.3.14.2 |
| T1 |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 0.4 |  |
| T3 |  | s | [TBD] |  |
| D1 |  | s | [0.24] |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.5.5.5.3.1-3: Cell specific test parameters for FR2 PSCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CSI-RS of set q0 |  |  |  |  | CSI-RS of set q1 |  |  |  |  |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  | dB |  |  |  |  |  |  |  |  |  |  |
| SNR_CSI-RS | Config 1 | dB | [1] | [-7] | [-15] | [-4.5] | [1] | [1] | [-7] | [-15] | [-4.5] | [1] |
|  | Config 2 |  | [1] | [-7] | [-15] | [-4.5] | [1] | [1] | [-7] | [-15] | [-4.5] | [1] |
|  | Config 3 |  | [1] | [-7] | [-15] | [-4.5] | [1] | [1] | [-7] | [-15] | [-4.5] | [1] |
|  | Config 1 | dBm/15KHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
|  | Config 2 |  | [-98] |  |  |  |  | [-98] |  |  |  |  |
|  | Config 3 |  | [-98] |  |  |  |  | [-98] |  |  |  |  |
| SS-RSRPNote 3 |  | dBm/SCS. |  |  |  |  |  |  |  |  |  |  |
| Ês/Iot |  |  |  |  |  |  |  |  |  |  |  |  |
| Ês/Noc |  |  |  |  |  |  |  |  |  |  |  |  |
| Io | config 1, 2 | dBm/9.36MHz |  |  |  |  |  |  |  |  |  |  |
|  | Config 3, 4 | dBm/38.1MHz |  |  |  |  |  |  |  |  |  |  |
| Propagation condition |  |  | [TDLA30-75] |  |  |  |  | [TDLA30-75] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |

Figure A.5.5.5.3.1-1: SNR variation SSB for CSI-RS based beam failure detection and link recovery testing in non-DRX mode

##### A.5.5.5.3.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = [TBD] ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode

##### A.5.5.5.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.5.5.5.4.1-1, A.5.5.5.4.1-2, A.5.5.5.4.1-3, and A.5.5.5.4.1-4 below. There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.5.5.5.4.1-1 shows the variation of the downlink SNR of the PCell and the SNR of the CSI-RS in set q0 in the active PSCell to emulate CSI-RS based beam failure. Figure A.5.5.5.4.1-1 additionally shows the variation of the downlink SNR of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [2] ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms).

Table A.5.5.5.4.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, TDD duplex mode, 120 kHz SSB SCS, 100MHz bandwidth |

Table A.5.5.5.4.1-2: General test parameters for FR2 PSCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |  |  |
| Active E-UTRA PCell |  |  |  |  | Cell 1 |  |  |  |
| E-UTRA RF Channel Number |  |  |  |  | 1 |  |  |  |
| Active PSCell |  |  |  |  | Cell 2 |  |  |  |
| RF Channel Number |  |  |  |  | 2 |  |  |  |
| Duplex mode |  | Config 1 |  |  | TDD |  |  |  |
| TDD Configuration |  | Config 1 |  |  | TDDConf.3.1 |  |  |  |
| CORESET Reference Channel |  | Config 1 |  |  | CR.3.1 TDD |  |  | A.3.1.2 |
| SSB Configuration |  | Config 1 |  |  | SSB.1 FR2 |  |  | A.3.10 |
| SMTC Configuration |  | Config 1 |  |  | SMTC.1 |  |  | A.3.11 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  |  | 120 KHz |  |  |  |
| csi-RS-Index assigned as RLM RS |  |  |  |  | [0] |  |  |  |
| OCNG parameters |  |  |  |  | OP.1 |  |  | A.3.2.1 |
| CP length |  |  |  |  | Normal |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |  |  |
| Beam failure detection transmission parameters | DCI format |  |  |  | 1-0 |  |  |  |
|  | Number of Control OFDM symbols |  |  |  | 2 |  |  |  |
|  | Aggregation level |  |  | CCE | 8 |  |  |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 |  |  |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 |  |  |  |
|  | DMRS precoder granularity |  |  |  | REG bundle size |  |  |  |
|  | REG bundle size |  |  |  | 6 |  |  |  |
| DRX |  |  |  |  | DRX.7 |  |  | A.3.3.7 |
| Gap pattern ID |  |  |  |  | *[gp0] |  |  |  |
| csi-RS-Index |  |  |  |  | 2 |  |  | Number of SSB indexes used for beam failure detection |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | absent |  |  | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  |  |  |  | TBD |  |  | Threshold used for Qout_LR_SSB |
| powerControlOffsetSS |  |  |  |  | NA |  |  | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | [n2] |  |  | see TS 38.321 [7], section 5.17 |
| beamFailureDetectionTimer |  |  |  |  | [pbfd4] |  |  | see TS 38.321 [7], section 5.17 |
| CSI-RS configuration | Config 1 |  | CSI-RS.3.2 TDD | A.3.14.2 |
| T1 |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 0.4 |  |
| T3 |  | s | [TBD] |  |
| D1 |  | s | [0.44] |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.5.5.5.4.1-3: Cell specific test parameters for FR2 PSCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CSI-RS of set q0 |  |  |  |  | CSI-RS of set q1 |  |  |  |  |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  | dB |  |  |  |  |  |  |  |  |  |  |
| SNR_CSI-RS | Config 1 | dB | [1] | [-7] | [-15] | [-4.5] | [1] | [1] | [-7] | [-15] | [-4.5] | [1] |
|  | Config 2 |  | [1] | [-7] | [-15] | [-4.5] | [1] | [1] | [-7] | [-15] | [-4.5] | [1] |
|  | Config 3 |  | [1] | [-7] | [-15] | [-4.5] | [1] | [1] | [-7] | [-15] | [-4.5] | [1] |
|  | Config 1 | dBm/15KHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
|  | Config 2 |  | [-98] |  |  |  |  | [-98] |  |  |  |  |
|  | Config 3 |  | [-98] |  |  |  |  | [-98] |  |  |  |  |
| SS-RSRPNote 3 |  | dBm/SCS. |  |  |  |  |  |  |  |  |  |  |
| Ês/Iot |  |  |  |  |  |  |  |  |  |  |  |  |
| Ês/Noc |  |  |  |  |  |  |  |  |  |  |  |  |
| Io | config 1, 2 | dBm/9.36MHz |  |  |  |  |  |  |  |  |  |  |
|  | Config 3, 4 | dBm/38.1MHz |  |  |  |  |  |  |  |  |  |  |
| Propagation condition |  |  | [TDLA30-75] |  |  |  |  | [TDLA30-75] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.5.5.5.4.1-4: Measurement gap configuration for FR2 PSCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | [0] |

Figure A.5.5.5.4.1-1: SNR variation SSB for CSI-RS-based beam failure detection and link recovery testing in DRX mode

##### A.5.5.5.4.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = [TBD] ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

<< End of change 1>>