3GPP TSG-RAN WG4 Meeting #90bis												  	         R4-1904984

Xian, China 8th – 12th of April, 2019

Source: 	Nokia, AMX

Title: 					TP to TR 36.716-03-01: 5A-7C-28A_BCS0

Agenda Item: 			7.3.3

Document for:	Approval

# 1	Introduction

This contribution is a TP for

| 5-7-28 | 4 | 5A-7C-28A_BCS0 | Rel-10 | Israel Soto, AMX | israel.soto@ americamovil.com | Nokia, Ericsson, Huawei, Samsung | new | 2B_3CC_5A-7C_BCS0 (completed) 2B_3CC_7C-28A_BCS0 (completed) 3B_3CC_5A-7A-28A_BCS0 (completed) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

****************************** Start of the TP*********************************************¨

## 5.X	5A-7C-28A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.5A-2a: Inter-band CA operating bands (three bands)

| E-UTRA CA Band | E-UTRA Band (Table 5.5.1) |
| --- | --- |
| CA_5-7-28 | 5, 7, 28 |
| NOTE: The frequency range in band 28 is restricted for this CA band combination to 718-748 MHz for the UL and 773-803 MHz for the DL |  |

Table 5.X.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_5A-7C-28A | - | 5 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.X.2 	Co-existence studies

Table 5.X.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 |
| 7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 |
| 28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |

No harmonic interference.

Table 5.X.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 |
| 7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 |
| 28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2274 | 3032 | 3212 |

No receiver mixing issues.

### 5.X.3	∆TIB and ∆RIB values

dTib and dRib have been already specfied.

Table 5.X.3-1: IB,c

| CA_5-7-28 | 5 | 0.5 |
| --- | --- | --- |
|  | 7 | 0.3 |
|  | 28 | 0.5 |

Table 5.X.3-2: R IB,c

| CA_5-7-28 | 5 | 0 |
| --- | --- | --- |
|  | 7 | 0 |
|  | 28 | 0 |

### 5.X.4 	REFSENS

No MSD is necessary.

****************************** End of the TP*********************************************