3GPP TSG-RAN WG4 Meeting #91                                                                R4-1905395

# Reno, US, 13th – 17th May, 2019

Source: 	CATT

Title: 	Simulation assumption for 256QAM feasibility study

Agenda item:	8.8.1

Document for:	Agreement

- Introduction

In RAN #83 meeting, a new WID[1,2] on adding support of NR DL 256QAM for FR2 was agreed with the following objectives of the core part for phase1 to be solved  no later than Q3 2019.

The objectives of the core part are as follows: Phase 1: Starts from Q2 2019 and is expected to conclude no later than Q3 2019 - Continue to the feasibility and performance benefit study for FR2 DL 256QAM that was carried out in Rel-15 NR WI. - Both system and link simulations as well as RF implementation need to be considered in the Rel-16 evaluation to show the benefits of FR2 DL 256QAM

This paper will give our view on NR DL 256QAM for FR2.

- Discussion

The new WID propose that RAN4 continues to study the feasibility and performance benefit for FR2 DL 256QAM, and considers both system and link simulations as well as RF implementation in Rel-16 evaluation. There were already many simulation results[3,4,5,6,7] for EVM performance for DL 256QAM in Rel-15 showing positive or negative gain of 256QAM by comparing to 64QAM. we think it’s necessary to align the simuation assumptions between companies to converge the feasibility study.

According to simulation assumptions [5,8,9] for FR2 256QAM in Rel-15, the simulation assumptions given in table 2-1 can be as starting point for Rel-16 evaluation on FR2 DL 256QAM feasibility.

Table 2-1 Simulation assumptions for FR2 DL 256QAM

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz |
| CBW | 50MHz |
| SCS | 60kHz |
| Allocated RBs | 66 PRBs |
| Propagation | TDL-A 30ns delay spread TDL-D 30ns delay spread |
| UE speed | 3 km/h |
| MCS | - 64QAM: Throughput with MCS index 16 in Table 2 (TS 38.214 Table 5.1.3.1-2) - 256QAM: Throughput with MCS index 21 in Table 2 (TS 38.214 Table 5.1.3.1-2) |
| Symbol type | CP-OFDM |
| HARQ | None |
| #antenna of BS | 1 |
| #antenna of UE | 1 |
| Channel estimation | practical |
| Receiver type | MMSE |
| DMRS configuration | 1 symbol front loaded + 1 additional |
| PTRS configuration | PTRS high density configuration: KPTRS : 2 (PTRS every 2 RBs) LPTRS : 1 (PTRS every 1 symbol) PTRS low density configuration: KPTRS : 2 (PTRS every 2 RBs) LPTRS : 4 (PTRS every 4 symbols) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX PN model config1: example1 (BS) + example1(UE) PN model config2: example2 (BS) + example2(UE) |
| txEVM + rxEVM for 256QAM | 0%+0%, 2%+2%, 2.5%+2.5%, 3.5%+3.5% |

- Conclusion

This paper presents simulations assumptions given in table 2-1 can be as starting point for Rel-16 evaluation on 256QAM feasibility.

- References

- RP-190253,New WID: Add support of NR DL 256QAM for FR2, China Telecom, RAN#83

- RP-190254,Motivations on NR DL 256QAM support for FR2, China Telecom, RAN#83

- R4-1807874, “Link level simulation results for BS EVM requirements evaluation for 256QAM in FR2”, Nokia, Nokia Shanghai Bell, RAN4#87

- R4-1807520, “256QAM support for FR2”, Huawei, RAN4#87

- R4-1806715, “On 256QAM for FR2”, CATT, RAN4#87

- R4-1806296, “Views on 256QAM support for NR FR2”, Intel Corporation, RAN4#87

- R4-1804987, “On EVM Requirement for FR2”, Ericsson, RAN4#87

- R4-1803032, Link level simulation results for BS EVM requirements evaluation in FR2,Nokia, Nokia Shanghai Bell,RAN4#86

- R4-1801003, WF on 256QAM for FR2,RAN4#86, intel, RAN4 #AH-1801