3GPP TSG-RAN WG4 Meeting #91 	R4-1907182

Reno, Nevada, 13th May -17th May, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | CRNum | rev | - | Current version: | 15.5.0 (R4-1905234) |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements of simultaneous reception of SSB and CSI-RS in FR1 and FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-04-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current version of 38.133 spec seem to suggest that, in both FR1 and FR2, whenever SSB is within the SMTC, no requirements are defined for SSB and CSI-RS based BFD-RS. However, this was not the intention of the CR that led to this text. The discussion related to the original CR wanted to have this restriction when SSB and CSI-RS are FDMed and when SSB falls inside SMTC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | We just make the following change (shown in red color) in two parts of the spec. “For FR1 and FR2, if the SSB is within the SMTC and if SSB and CSI-RS are in the same symbol, longer evaluation period can be expected, and no requirements are defined.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB and/or UE may think that there are no requirements for either SSB or CSI-RS based BFD when SSB falls within the SMTC window. Since, any implementation of Rel-15 will obviously have some SSBs within the SMTC window for RRM, and BFD-RS is crucial in multi-beam scenario, this ambiguity may break the implementation of a multi-beam network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | ……….. |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

#### 8.5.3.3	Measurement restrictions for CSI-RS beam failure detection

For FR1, when the SSB is within the active BWP and has same SCS than CSI-RS, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB is within the active BWP and has different SCS than CSI-RS, the UE shall be able to perform CSI-RS measurement with following restrictions according to its capabilities:

-	If CSI-RS and SSB are in the same symbol, the UE measurement capability depends on the whether the UE supports simultaneousRxDataSSB-DiffNumerology.

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology the UE is not expected to perform CSI-RS measurements,

For FR2, when the SSB not within the SMTC but is within the active BWP and has same SCS than CSI-RS, the UE shall be able to perform CSI-RS measurement without restrictions when CSI-RS measurement are performed with same subcarrier spacing as the SSB, provided that the SSB and CSI-RS are QCL TypeD and QCL information is known to UE.

For FR2, when the SSB is not within the SMTC, when the SSB is within the active BWP and has different SCS than CSI-RS, the UE shall be able to perform CSI-RS measurement with following restrictions according to its capabilities, provided that the SSB and CSI-RS are QCL TypeD and QCL information is known to UE:

-	If CSI-RS and SSB are in the same symbol, the UE measurement capability depends on the whether the UE supports simultaneousRxDataSSB-DiffNumerology.

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology the UE is not expected to perform CSI-RS measurements,

For FR1 and FR2, if the SSB is within the SMTC and if SSB and CSI-RS are in the same symbol, longer evaluation period can be expected, and no requirements are defined. For FR1 and FR2, if the CSI-RS resource is in a resource set configured with repetition ON, longer evaluation period can be expected, and no requirements are defined.

#### 8.5.5.3		Measurement restrictions for SSB candidate beam detection

For FR1, when the SSB is within the active BWP and has same SCS than CSI-RS, the UE shall be able to perform SSB measurement without restrictions.

For FR1, when the SSB is within the active BWP and has different SCS than CSI-RS, the UE shall be able to perform SSB measurement with following restrictions according to its capabilities:

-	If CSI-RS and SSB are in the same symbol, the UE measurement capability depends on the whether the UE supports simultaneousRxDataSSB-DiffNumerology.

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform SSB measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology the UE is not expected to perform CSI-RS measurements,

……

-	If CSI-RS and SSB are in the same symbol, the UE measurement capability depends on the whether the UE supports simultaneousRxDataSSB-DiffNumerology.

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform SSB measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology the UE is not expected to perform CSI-RS measurements,

For FR1 and FR2, if the SSB is within the SMTC and if SSB and CSI-RS are in the same symbol, longer evaluation period can be expected, and no requirements are defined.

<< End of changes>>