3GPP TSG- RAN4 Meeting #92	R4-1907970

# Ljubljana, Slovenia, 26th - 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.810 | CR | - | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update SNR range calculations and spreadsheets for RRM and Demodulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_test_methods |  |  |  |  |  | Date: |  |  | 2019-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The spreadsheets to calculate SNR range currently included in TR 38.810 are missing a number of features which are relevant to test case design. For RRM: - The “rough beam” scenario is not included - The effect of UE multi-band relaxation factor is not included - A 0.5dB ceiling function is wrongly used to calculate the Noc levels - For “100MHz” Ch BW, the power should be calculated in 95.04MHz - The scenario with no applied noise is not included - RAN5 has agreed a slightly better absolute level uncertainty For Demodulation: - The effect of UE multi-band relaxation factor is not included - For “100MHz” Ch BW, the power should be calculated in 95.04MHz - RAN5 has agreed a slightly better absolute level uncertainty The prediction of RRM SNR range using X, Y and Z values in Tables B.2.1.5.4-2 and B.2.2.5.4-2 does not give the correct SNR values when the predicted SNR is low. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update “Spreadsheet 1 - RRM SNR range calculator”: - Include the “rough beam” scenario, with new tabs - Include the effect of UE multi-band relaxation factor - When calculating the Noc levels, the value is rounded to 0.1dB - For “100MHz” Ch BW, calculate the power in 95.04MHz - Include the scenario with no applied noise - Update absolute level uncertainty to +/-6dB - Values used on all tabs are now entered only on the first tab - Add values for Noc/15kHz, Noc/SCS and wanted signal Es/SCS Update “Spreadsheet 2 - Demod SNR range calculator”: - Include the effect of UE multi-band relaxation factor - For “100MHz” Ch BW, calculate the power in 95.04MHz - Update absolute level uncertainty to +/-6dB - Values used on all tabs are now entered only on the first tab - Add values for Noc/15kHz, Noc/SCS and wanted signal Es/SCS Update the Annex B text and explanations, and provide updated SNR values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scenarios and values relevant to test case design would not be covered. Designers of RRM Test cases in TS 38.133 will not have enough information to specify the test cases in a way that can be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table B.2.1.3-2, B.2.1.5.4, Figure B.2.1.5.4-1, Tables B.2.1.5.4-1, B.2.1.5.4-2. Table B.2.2.3-2, B.2.2.5.4, Figure B.2.2.5.4-1, Tables B.2.2.5.4-1, B.2.2.5.4-2. Table B.2.3.3-2 B.3.1.5.4, Figure B.3.1.5.4-1, Table B.3.1.5.4-1. B.3.2.5.4, Figure B.3.2.5.4-1, Table B.3.2.5.4-1. B.3.3.5.4, Figure B.3.3.5.4-1, Table B.3.3.5.4-1. Attached spreadsheets. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.521-4, TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

# B.2	Measurement uncertainty budget for UE RRM testing methodology

## B.2.1	Direct far field (DFF) setup

### B.2.1.1	Uncertainty budget calculation principle

The uncertainty tables cover the actual measurement using the DUT. In some cases, uncertainty may also arise from a calibration or alignment process before the measurements.

When a calibration process is used before the measurements, the uncertainty tables should be presented with two stages:

-	Stage 1: the calibration of the absolute level of the DUT measurement results is performed by means of using a calibration antenna whose absolute gain is known at the frequencies of measurement

-	Stage 2: the actual measurement with the DUT as either the transmitter or receiver is performed.

The MU budget should comprise of a minimum 5 headings:

1)	The uncertainty source,

2)	Uncertainty value,

3)	Distribution of the probability,

4)	Divisor based on distribution shape,

5)	Calculated standard uncertainty (based on uncertainty value and divisor).

### B.2.1.2	Uncertainty budget format

The uncertainty contributions depend on the parameter being controlled and/or the measurement being made. A separate table is provided for each.

Table B.2.1.2-1: Uncertainty contributions for test with defined DL SNR at reference point

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| During measurement |  |  |
| 1 | gNB emulator SNR uncertainty | B.2.1.4.1 |
| 2 | gNB emulator DL EVM | B.2.1.4.2 |
| 3 | gNB emulator Fading model impairments | B.2.1.4.3 |
| Note 1: Handling of effects related to isolation and alignment of Horizontal / Vertical polarisation is not defined Note 2: Handling of effects related to Quality of Quiet zone is not defined |  |  |

Table B.2.1.2-2: Uncertainty contributions for DL absolute power level at reference point (D = 5 cm)

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| During measurement |  |  |
| 1 | Currently assumed to be the same as EIS measurement in Table B.1.1.2-2 | B.1.1.4 |
| Note 1: Additional uncertainty contributions may apply for faded signals Note 2: Contribution from Quality of Quiet zone may be different from EIS measurement |  |  |

### B.2.1.3	Uncertainty assessment

The uncertainty assessment tables are organized as follows:

-	For the purpose of uncertainty assessment, the radiating antenna aperture of the DUT is denoted as D, and the uncertainty assessment has been derived for the case of D = 5 cm

The uncertainty contributions depend on the parameter being controlled and/or the measurement being made. A separate table is provided for each.

Table B.2.1.3-1: Uncertainty assessment for test with defined DL SNR at reference point

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| During measurement |  |  |  |  |  |
| 1 | gNB emulator SNR uncertainty | 0.3dB | Normal | 2.00 | 0.15 |
| 2 | gNB emulator DL EVM | - | One-sided, beneficial | - | 0 |
| 3 | gNB emulator Fading model impairments Note 3 | [0.5dB] | Normal | 2.00 | [0.25] |
| SNR Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [0.57] |
| Note 1: Handling of effects related to isolation and alignment of Horizontal / Vertical polarisation is not defined Note 2: Handling of effects related to Quality of Quiet zone is not defined Note 3: The value is same as for LTE and shall be verified for other channel models by RAN5 during detailed MU assessment |  |  |  |  |  |

Table B.2.1.3-2: Uncertainty assessment for DL absolute power level at reference point (D = 5 cm)

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| During measurement |  |  |  |  |  |
| Currently assumed to be +/-6dB |  |  |  |  |  |
| DL absolute power level Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [6.0] |
| Note 1: Additional uncertainty contributions may apply for faded signals Note 2: Contribution from Quality of Quiet zone may be different from EIS measurement |  |  |  |  |  |

### B.2.1.4	Measurement error contribution descriptions

#### B.2.1.4.1	gNB emulator SNR uncertainty

See B.3.1.4.1.

#### B.2.1.4.2	gNB emulator Downlink EVM

See B.3.1.4.2.

#### B.2.1.4.3	gNB emulator fading model impairments

See B.3.1.4.3.

### B.2.1.5	Assessment of testable SNR range for D=5cm

The signal and the noise provided by the test system are both attenuated by the over-the-air link loss. The UE noise then adds to the noise provided by the test system, hence degrading the SNR seen by the UE and potentially limiting the testable SNR range.

For conducted tests, the noise provided by the test system can be set much higher than the UE noise and the SNR degradation is negligible. However for over-the-air test systems, the power that can realistically be delivered into the test system probe antenna is limited, so the test point is likely to be closer to the UE noise and a small SNR degradation is allowable.

#### B.2.1.5.1	Method and Parameters

The initial rational in this section is elaborated for Scenario 1 (1 Angle of Arrival with signal coming from the RX beam peak direction) for Type 1 Requirements (“Fine” RX beam) and Mode 1 Configuration (TE generating S and N).

For RRM tests, DL SNR is given as a test parameter. Such SNR environment is generated by the test system by injecting both desired signal and artificial AWGN noise. To fulfil the purpose of the test, the configured SNR should be accurate enough at the UE receiver. If the absolute power level of the signal and noise from the test system is too low, then the SNR would be degraded due to the UE’s internal noise.

The calculation of Noc level is shown in clause 6.2.1.4.

The possible transmitted signal and noise strengths from the test antenna depend on the capability of the test system. The feasible transmit power depends on the conducted cable losses, the Test system transmit antenna gain and the final amplifier characteristic, especially the P1dB compression point. Considering the cable losses, probe antenna gain of 12dB and commercially available mmWave Amplifiers, and the crest factor of the downlink signal (not to cause additional EVM error from test system side), the feasible transmit power from the probe antenna is calculated below, which in turn sets the SNR range.

Table B.2.1.5.1-1: Assumed Test system parameters

|  | 43GHz |  |
| --- | --- | --- |
| P1dB amplifier power | +23 | dBm |
| Backoff from P1dB | -13 | dB |
| Cable loss | -8 | dB |
| Probe antenna gain | 12 | dB |
| Transmission bandwidth | 100M | Hz |

The third part to consider is the free space path loss between the probe antenna and the UE antenna, shown below:

Table B.2.1.5.1-2: Free Space path loss

|  | 43GHz |  |
| --- | --- | --- |
| @0.725m separation | -62.3 | dB |

#### B.2.1.5.2	Void

#### B.2.1.5.3	Void

#### B.2.1.5.4	SNR range for SNRRP - SNRBB ≤ 1dB

The initial rational in this section is elaborated for Scenario 1 (1 Angle of Arrival with signal coming from the RX beam peak direction) for Type 1 Requirements (“Fine” RX beam) and Mode 1 Configuration (TE generating S and N).

Based on the method of setting the noise from the Test system to give a maximum of 1dB degradation in overall SNR between reference point and baseband, we can then work back through the signal chain to determine how high the SNR can be set. As the noise is set to a fixed level, the maximum SNR is set by the test system power amplifier and the channel bandwidth to be tested.

The SNR upper bound depends on the type of test system. For the Direct far field (DFF) setup the diagram below illustrates the principle, and is based on the “DFF 100MHz” tab of the accompanying spreadsheet.

The process works back through the signal chain, from left to right in the diagram.

Figure B.2.1.5.4-1: Estimation of multi band UE SNR range for Direct far field (DFF), fine beam, Rx Beam peak direction

The test equipment must supply at least the wanted noise level at the reference point. If the noise was lower, the degradation in SNR would be greater than 1dB, and may cause a conformant UE to fail.

The accuracy of setting the signal and noise levels has been taken as +/-[6.0]dB. It is subject to further analysis by RAN5.

During conformance test, the test system will need to find the UE Rx beam peak. At present an allowance of [0.5] dB has been included, but is subject to further analysis by RAN5.

Inclusion of these two contributions directly reduces the maximum SNR that can be measured by a test system for a given channel bandwidth. To find the maximum SNR that can be measured by a test system with a specific Channel BW, the baseband SNR in the spreadsheet is increased until the value “Wanted signal + headroom, dBm/Ch BW” is just below the “TE Power amplifier 1dB compression, dBm” value. The resulting values are given in Table B.2.1.5.4-1.

Table B.2.1.5.4-1: Predicted SNR upper bound values for Direct far field (DFF), multi band UE

| Angle of Arrival | Beam type | Channel Bandwidth | Maximum SNR |
| --- | --- | --- | --- |
| Rx Beam peak | Fine | 100MHz | [17.8dB] |
|  | Rough | 100MHz | [10.5dB] |
| Spherical coverage | Fine | 100MHz | [4.7dB] |
|  | Rough | 100MHz | Not usable |

Note that these are UE baseband SNR values, so the Reference point figures used in RRM test cases may be 1dB higher. Values are based on UE parameters and currently foreseen test equipment limitations, and could be improved in future.

An example of SNR calculation for DFF method is provided in “Spreadsheet 1 - RRM SNR range calculator.xls” file attached to the TR.

## B.2.2	Indirect far field (IFF) setup

### B.2.2.1	Uncertainty budget calculation principle

Same as in B.2.1.1.

### B.2.2.2	Uncertainty budget format

The uncertainty contributions depend on the parameter being controlled and/or the measurement being made. A separate table is provided for each.

Table B.2.2.2-1: Uncertainty contributions for test with defined DL SNR at reference point

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| During measurement |  |  |
| 1 | gNB emulator SNR uncertainty | B.2.2.4.1 |
| 2 | gNB emulator DL EVM | B.2.2.4.2 |
| 3 | gNB emulator Fading model impairments | B.2.2.4.3 |
| Note 1: Handling of effects related to isolation and alignment of Horizontal / Vertical polarisation is FFS Note 2: Handling of effects related to Quality of Quiet zone is FFS |  |  |

Table B.2.2.2-2: Uncertainty contributions for DL absolute power level at reference point (D = 15 cm)

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| During measurement |  |  |
| 1 | Currently assumed to be the same as EIS measurement in Table B.1.3.2-2 | B.2.1.4 |
| Note 1: Additional uncertainty contributions may apply for faded signals Note 2: Contribution from Quality of Quiet zone may be different from EIS measurement |  |  |

### B.2.2.3	Uncertainty assessment

The uncertainty assessment tables are organized as follows:

-	For the purpose of uncertainty assessment, the radiating antenna aperture of the DUT is denoted as D, and the uncertainty assessment has been derived for the case of D = 15 cm

The uncertainty contributions depend on the parameter being controlled and/or the measurement being made. A separate table is provided for each.

Table B.2.2.3-1: Uncertainty assessment for test with defined DL SNR at reference point

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| During measurement |  |  |  |  |  |
| 1 | gNB emulator SNR uncertainty | 0.3dB | Normal | 2.00 | 0.15 |
| 2 | gNB emulator DL EVM | - | One-sided, beneficial | - | 0 |
| 3 | gNB emulator Fading model impairments Note 3 | [0.5dB] | Normal | 2.00 | [0.25] |
| SNR Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [0.57] |
| Note 1: Handling of effects related to isolation and alignment of Horizontal / Vertical polarisation is not defined Note 2: Handling of effects related to Quality of Quiet zone is not defined Note 3: The value is same as for LTE and shall be verified for other channel models by RAN5 during detailed MU assessment |  |  |  |  |  |

Table B.2.2.3-2: Uncertainty assessment for DL absolute power level at reference point (D = 15 cm)

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| During measurement |  |  |  |  |  |
| Currently assumed to be +/-6dB |  |  |  |  |  |
| DL absolute power level Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [6.0] |
| Note 1: Additional uncertainty contributions may apply for faded signals Note 2: Contribution from Quality of Quiet zone may be different from EIS measurement |  |  |  |  |  |

### B.2.2.4	Measurement error contribution descriptions

#### B.2.2.4.1	gNB emulator SNR uncertainty

See B.3.1.4.1.

#### B.2.1.4.2	gNB emulator Downlink EVM

See B.3.1.4.2.

#### B.2.1.4.3	gNB emulator fading model impairments

See B.3.1.4.3

### B.2.2.5	Assessment of testable SNR range

The signal and the noise provided by the test system are both attenuated by the over-the-air link loss. The UE noise then adds to the noise provided by the test system, hence degrading the SNR seen by the UE and potentially limiting the testable SNR range.

For conducted tests, the noise provided by the test system can be set much higher than the UE noise and the SNR degradation is negligible. However for over-the-air test systems, the power that can realistically be delivered into the test system probe antenna is limited, so the test point is likely to be closer to the UE noise and a small SNR degradation is allowable.

#### B.2.2.5.1	Method and Parameters

The method is the same as B.2.1.5.1, but the values related to the test system are different. The calculation of noise level is in clause 6.2.1.4.3, 6.2.1.4.4, 6.2.1.4.5 and 6.2.1.4.6.

#### B.2.2.5.2	Void

#### B.2.2.5.3	Void

#### B.2.2.5.4	SNR range for SNRRP - SNRBB ≤ 1dB

The initial rational in this section is elaborated for Scenario 1 (1 Angle of Arrival with signal coming from the RX beam peak direction) for Type 1 Requirements (“Fine” RX beam) and Mode 1 Configuration (TE generating S and N).

Based on the method of setting the noise from the Test system to give a maximum of 1dB degradation in overall SNR between reference point and baseband, we can then work back through the signal chain to determine how high the SNR can be set. As the noise is set to a fixed level, the maximum SNR is set by the test system power amplifier and the channel bandwidth to be tested.

The SNR upper bound depends on the type of test system. For the Indirect Far field (IFF) setup the diagram below illustrates the principle, and is based on the “IFF 100MHz” tab of the accompanying spreadsheet.

The process works back through the signal chain, from left to right in the diagram.

Figure B.2.2.5.4-1: Estimation of multi band UE SNR range for Indirect far field (IFF), fine beam, Rx Beam peak direction

The test equipment must supply at least the wanted noise level at the reference point. If the noise was lower, the degradation in SNR would be greater than 1dB, and may cause a conformant UE to fail.

The accuracy of setting the signal and noise levels has been taken as +/-[6.0]dB. The uncertainty is subject to further analysis by RAN5.

During conformance test, the test system will need to find the UE Rx beam peak. At present an allowance of [0.5] dB has been included, but is subject to further analysis by RAN5.

Inclusion of these two contributions directly reduces the maximum SNR that can be measured by a test system for a given channel bandwidth. To find the maximum SNR that can be measured by a test system with a specific Channel BW, the baseband SNR in the spreadsheet is increased until the value “Wanted signal + headroom, dBm/Ch BW” is just below the “TE Power amplifier 1dB compression, dBm” value. The resulting values are given in Table B.2.2.5.4-1.

Table B.2.2.5.4-1: Predicted SNR upper bound values for Indirect far field (IFF) multi band UE

| Angle of Arrival | Beam type | Channel Bandwidth | Maximum SNR |
| --- | --- | --- | --- |
| Rx Beam peak | Fine | 100MHz | [18.1dB] |
|  | Rough | 100MHz | [10.9dB] |
| Spherical coverage | Fine | 100MHz | [5.1dB] |
|  | Rough | 100MHz | Not usable |

Note that these are UE baseband SNR values, so the Reference point figures used in RRM test cases may be 1dB higher. Values are based on UE parameters and currently foreseen test equipment limitations, and could be improved in future.

## B.2.3	Simplified Direct far field (DFF) setup

### B.2.3.1	Uncertainty budget calculation principle

Same as in B.2.1.1.

### B.2.3.2	Uncertainty budget format

The uncertainty contributions depend on the parameter being controlled and/or the measurement being made. A separate table is provided for each.

Table B.2.3.2-1: Uncertainty contributions for test with defined DL SNR at reference point

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| During measurement |  |  |
| 1 | gNB emulator SNR uncertainty | B.2.3.4.1 |
| 2 | gNB emulator DL EVM | B.2.3.4.2 |
| 3 | gNB emulator Fading model impairments | B.2.3.4.3 |
| Note 1: Handling of effects related to isolation and alignment of Horizontal / Vertical polarisation is not defined Note 2: Handling of effects related to Quality of Quiet zone is not defined |  |  |

Table B.2.3.2-2: Uncertainty contributions for DL absolute power level at reference point (D = 5 cm)

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| During measurement |  |  |
| 1 | Currently assumed to be the same as EIS measurement in Table B.1.1.2-2 | B.1.1.4 |
| Note 1: Additional uncertainty contributions may apply for faded signals Note 2: Contribution from Quality of Quiet zone may be different from EIS measurement |  |  |

### B.2.3.3	Uncertainty assessment

The uncertainty assessment tables are organized as follows:

-	For the purpose of uncertainty assessment, the radiating antenna aperture of the DUT is denoted as D, and the uncertainty assessment has been derived for the case of D = 5 cm

The uncertainty contributions depend on the parameter being controlled and/or the measurement being made. A separate table is provided for each.

Table B.2.3.3-1: Uncertainty assessment for test with defined DL SNR at reference point

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| During measurement |  |  |  |  |  |
| 1 | gNB emulator SNR uncertainty | 0.3dB | Normal | 2.00 | 0.15 |
| 2 | gNB emulator DL EVM | - | One-sided, beneficial | - | 0 |
| 3 | gNB emulator Fading model impairments Note 3 | [0.5dB] | Normal | 2.00 | [0.25] |
| SNR Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [0.57] |
| Note 1: Handling of effects related to isolation and alignment of Horizontal / Vertical polarisation is not defined Note 2: Handling of effects related to Quality of Quiet zone is not defined Note 3: The value is same as for LTE and shall be verified for other channel models by RAN5 during detailed MU assessment |  |  |  |  |  |

Table B.2.3.3-2: Uncertainty assessment for DL absolute power level at reference point (D = 5 cm)

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| During measurement |  |  |  |  |  |
| Currently assumed to be +/-6dB |  |  |  |  |  |
| DL absolute power level Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [6.0] |
| Note 1: Additional uncertainty contributions may apply for faded signals Note 2: Contribution from Quality of Quiet zone may be different from EIS measurement |  |  |  |  |  |

### B.2.3.4	Measurement error contribution descriptions

#### B.2.3.4.1	gNB emulator SNR uncertainty

See B.3.1.4.1.

#### B.2.3.4.2	gNB emulator Downlink EVM

See B.3.1.4.2.

#### B.2.3.4.3	gNB emulator fading model impairments

See B.3.1.4.3.

### B.2.3.5	Assessment of testable SNR range for D=5cm

The signal and the noise provided by the test system are both attenuated by the over-the-air link loss. The UE noise then adds to the noise provided by the test system, hence degrading the SNR seen by the UE and potentially limiting the testable SNR range.

For conducted tests, the noise provided by the test system can be set much higher than the UE noise and the SNR degradation is negligible. However for over-the-air test systems, the power that can realistically be delivered into the test system probe antenna is limited, so the test point is likely to be closer to the UE noise and a small SNR degradation is allowable.

#### B.2.3.5.1	Method and Parameters

Same as B.2.1.5.1.

#### B.2.3.5.2	Void

#### B.2.3.5.3	Void

#### B.2.3.5.4	SNR range for SNRRP - SNRBB ≤ 1dB

Same as B.2.1.5.4.

# B.3	Measurement uncertainty budget for UE demodulation testing methodology

## B.3.1	Direct near field (DNF) setup

### B.3.1.1	Uncertainty budget calculation principle

The uncertainty tables cover the actual measurement using the DUT receiver. If applicable, any uncertainty arising from a calibration or alignment process before the measurements should also be included.

The MU budget should comprise of a minimum 5 headings:

1)	The uncertainty source,

2)	Uncertainty value,

3)	Distribution of the probability,

4)	Divisor based on distribution shape,

5)	Calculated standard uncertainty (based on uncertainty value and divisor).

### B.3.1.2	Uncertainty budget format

Table B.3.1.2-1: Uncertainty contributions for T-put test with defined SNR at reference point

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| During T-put measurement |  |  |
| 1 | gNB emulator SNR uncertainty | B.3.1.4.1 |
| 2 | gNB emulator DL EVM | B.3.1.4.2 |
| 3 | gNB emulator Fading model impairments | B.3.1.4.3 |
| Note 1: Handling of effects related to isolation and alignment of Horizontal / Vertical polarisation is not defined Note 2: Handling of effects related to Quality of Quiet zone is not defined |  |  |

### B.3.1.3	Uncertainty assessment

The uncertainty assessment tables are organized as follows:

-	The uncertainty assessment for T-put measurement at defined SNR, is provided in Table B.3.1.3-1

Table B.3.1.3-1: Uncertainty assessment for T-put test with defined SNR at reference point

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| During T-put measurement |  |  |  |  |  |
| 1 | gNB emulator SNR uncertainty | 0.3dB | Normal | 2.00 | 0.15 |
| 2 | gNB emulator DL EVM | - | One-sided, beneficial | - | 0 |
| 3 | gNB emulator Fading model impairments Note 3 | [0.5dB] | Norma | 2.00 | [0.25] |
| SNR Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [0.57] |
| Note 1: Handling of effects related to isolation and alignment of Horizontal / Vertical polarisation is not defined Note 2: Handling of effects related to Quality of Quiet zone is not defined Note 3: The value is same as for LTE and shall be verified for other channel models by RAN5 during detailed MU assessment |  |  |  |  |  |

### B.3.1.4	Measurement error contribution descriptions

#### B.3.1.4.1	gNB emulator SNR uncertainty

This contribution originates from setting the ratio of signal and noise in the conducted part of the test system. It is estimated to be the same as for LTE conducted testing in TS 36.521-1 Annex F, which is ±0.3dB. The default for values in 36.521-1 Annex F is 95% confidence interval, normal distribution.

#### B.3.1.4.2	gNB emulator Downlink EVM

When simulations of demodulation performance are run, the downlink signal is modelled with a defined EVM, representing imperfections in the signal transmitted by the gNB. This EVM value is agreed across companies to align simulations, and is normally lower than the gNB EVM requirement, to represent “typical” conditions. The EVM used for simulations is therefore built in to the requirement points, normally specified as the SNR required to meet a specified throughput, with a defined modulation and Reference channel, under defined propagation conditions.

For a conformance test, the EVM defined for the simulations is taken as a maximum allowed value for the test system, as a worse gNB emulator EVM would make the signal harder to demodulate, and disadvantage the UE. In a test system the EVM cannot normally be set to a specific value, but is specified to be no higher than a defined value.

Following this approach, the uncertainty from gNB emulator Downlink EVM is a one-sided distribution, with beneficial effect. Without treating the positive and negative uncertainties separately, and as it would not make the SNR worse, the effective uncertainty is 0dB.

#### B.3.1.4.3	gNB emulator fading model impairments

This contribution originates from imperfections in the gNB emulator fading model, compared to the applied fading model. It is estimated to be the same as for LTE conducted testing in TS 36.521-1 Annex F, which is ±0.5dB. The default for values in 36.521-1 Annex F is 95% confidence interval, normal distribution.

### B.3.1.5	Assessment of testable DL SNR range and accuracy for D=15cm

The signal and the noise provided by the test system are both attenuated by the over-the-air link loss. The UE noise then adds to the noise provided by the test system, hence degrading the SNR seen by the UE and potentially limiting the testable SNR range. The calculations and graphs in this subclause allow this SNR degradation to be assessed over a range of scenarios.

For conducted tests, the noise provided by the test system can be set much higher than the UE noise and the SNR degradation is negligible. However for over-the-air test systems, the power that can realistically be delivered into the test system probe antenna is limited, so the test point is likely to be closer to the UE noise and a small SNR degradation is allowable.

#### B.3.1.5.1	Method and Parameters

The method is the same as B.2.1.5.1, but some values related to the test system are different, and provided in Table B.3.1.5.1-1. The calculation of noise level is in clause 7.2.1.3.

Table B.3.1.5.1-1: Free Space path loss

|  | 43GHz |  |
| --- | --- | --- |
| @0.5m separation | -59.1 | dB |

The radiative near field measurement distance is calculated for a DUT with radiating aperture D = 15cm, using the formula in clause 7.2.1.2:

Table B.3.1.5.1-2: Radiated near-field distance for D = 0.15m

|  | 24GHz | 43GHz |  |
| --- | --- | --- | --- |
| Wavelength | 0.0125 | 0.0070 | m |
| Radiated near-field distance | 0.32 | 0.43 | m |

Table B.3.1.5.1-3: Void

Table B.3.1.5.1-4: Void

#### B.3.1.5.2	Void

#### B.3.1.5.3	Void

#### B.3.1.5.4	SNR range for SNRRP - SNRBB ≤ 1dB

Based on the method of setting the noise from the Test system to give a maximum of 1dB degradation in overall SNR between reference point and baseband, we can then work back through the signal chain to determine how high the SNR can be set. As the noise is set to a fixed level, the maximum SNR is set by the test system power amplifier and the channel bandwidth to be tested.

The SNR upper bound depends on the type of test system. For the Direct Near field (DNF) setup the diagram below illustrates the principle, and is based on the “DNF 100MHz” tab of the accompanying spreadsheet.

The process works back through the signal chain, from left to right in the diagram.

Figure B.3.1.5.4-1: Estimation of single band UE SNR range for Direct near field (DNF)

The test equipment must supply at least the wanted noise level at the reference point. If the noise was lower, the degradation in SNR would be greater than 1dB, and may cause a conformant UE to fail.

The accuracy of setting the signal and noise levels has been taken as +/-[6.0]dB. The uncertainty is subject to further analysis by RAN5.

During conformance test, the test system will need to find the UE Rx beam peak. At present an allowance of [0.5] dB has been included, but is subject to further analysis by RAN5.

Inclusion of these two contributions directly reduces the maximum SNR that can be measured by a test system for a given channel bandwidth. To find the maximum SNR that can be measured by a test system with a specific Channel BW, the baseband SNR in the spreadsheet is increased until the value “Wanted signal + headroom, dBm/Ch BW” is just below the “TE Power amplifier 1dB compression, dBm” value. The resulting values are given in Table B.3.1.5.4-1. Single band UE values are obtained by setting the UE multi-band relaxation factor to 0dB.

Table B.3.1.5.4-1: Predicted SNR upper bound values for Direct Near field (DNF)

|  | Channel Bandwidth | Maximum SNR |
| --- | --- | --- |
| Single band UE | 100MHz | [22.6dB] |
|  | 200MHz | [19.6dB] |
| Multi-band UE Note 1 | 100MHz | [20.6dB] |
|  | 200MHz | [17.6dB] |
| Note 1: For ∑MBp from TS 38.101-2 Table 6.2.1.3-4 allow up to 2dB |  |  |

Note that these are UE baseband SNR values, so the Reference point figures used in TS 38.101-4 will be 1dB higher. Values are based on UE parameters and currently foreseen test equipment limitations, and could be improved in future.

Note that for the DNF method, values may need to be adjusted to take account of near-field effects.

An example of SNR calculation for DNF method is provided in “Spreadsheet 2 - Demod SNR range calculator.xls” file attached to the TR.

## B.3.2	Direct far field (DFF) setup

The Measurement uncertainty contributions and uncertainty assessment are expected to be the same as for the Direct near field (DNF) setup in B.3.1.

### B.3.2.1	Void

### B.3.2.2	Void

### B.3.2.3	Void

### B.3.2.4	Void

### B.3.2.5	Assessment of testable SNR range

The signal and the noise provided by the test system are both attenuated by the over-the-air link loss. The UE noise then adds to the noise provided by the test system, hence degrading the SNR seen by the UE and potentially limiting the testable SNR range.

For conducted tests, the noise provided by the test system can be set much higher than the UE noise and the SNR degradation is negligible. However for over-the-air test systems, the power that can realistically be delivered into the test system probe antenna is limited, so the test point is likely to be closer to the UE noise and a small SNR degradation is allowable.

#### B.3.2.5.1	Method and Parameters

The method and parameters are the same as B.2.1.5.1. The calculation of noise level is in clause 7.2.1.3.

#### B.3.2.5.2	Void

#### B.3.2.5.3	Void

#### B.3.2.5.4	SNR range for SNRRP - SNRBB ≤ 1dB

Based on the method of setting the noise from the Test system to give a maximum of 1dB degradation in overall SNR between reference point and baseband, we can then work back through the signal chain to determine how high the SNR can be set. As the noise is set to a fixed level, the maximum SNR is set by the test system power amplifier and the channel bandwidth to be tested.

The SNR upper bound depends on the type of test system. For the Direct Far field (DFF) setup the diagram below illustrates the principle, and is based on the “DFF 100MHz” tab of the accompanying spreadsheet.

The process works back through the signal chain, from left to right in the diagram.

Figure B.3.2.5.4-1: Estimation of single band UE SNR range for Direct far field (DFF)

The test equipment must supply at least the wanted noise level at the reference point. If the noise was lower, the degradation in SNR would be greater than 1dB, and may cause a conformant UE to fail.

The accuracy of setting the signal and noise levels has been taken as +/-[6.0]dB. The uncertainty is subject to further analysis by RAN5.

During conformance test, the test system will need to find the UE Rx beam peak. At present an allowance of [0.5] dB has been included, but is subject to further analysis by RAN5.

Inclusion of these two contributions directly reduces the maximum SNR that can be measured by a test system for a given channel bandwidth. To find the maximum SNR that can be measured by a test system with a specific Channel BW, the baseband SNR in the spreadsheet is increased until the value “Wanted signal + headroom, dBm/Ch BW” is just below the “TE Power amplifier 1dB compression, dBm” value. The resulting values are given in Table B.3.2.5.4-1. Single band UE values are obtained by setting the UE multi-band relaxation factor to 0dB.

Table B.3.2.5.4-1: Predicted SNR upper bound values for Direct far field (DFF)

|  | Channel Bandwidth | Maximum SNR |
| --- | --- | --- |
| Single band UE | 100MHz | [19.4dB] |
|  | 200MHz | [16.4dB] |
| Multi-band UE Note 1 | 100MHz | [17.4dB] |
|  | 200MHz | [14.3dB] |
| Note 1: For ∑MBp from TS 38.101-2 Table 6.2.1.3-4 allow up to 2dB |  |  |

Note that these are UE baseband SNR values, so the Reference point figures used in TS 38.101-4 will be 1dB higher. Values are based on UE parameters and currently foreseen test equipment limitations, and could be improved in future.

An example of SNR calculation for DFF method is provided in “Spreadsheet 2 - Demod SNR range calculator.xls” file attached to the TR.

## B.3.3	Indirect far field (IFF) setup

### B.3.3.1	Uncertainty budget calculation principle

The uncertainty tables cover the actual measurement using the DUT receiver. If applicable, any uncertainty arising from a calibration or alignment process before the measurements should also be included.

The MU budget should comprise of a minimum 5 headings:

1)	The uncertainty source,

2)	Uncertainty value,

3)	Distribution of the probability,

4)	Divisor based on distribution shape,

5)	Calculated standard uncertainty (based on uncertainty value and divisor).

### B.3.3.2	Uncertainty budget format

Table B.3.3.2-1: Uncertainty contributions for T-put test with defined SNR at reference point

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| During T-put measurement |  |  |
| 1 | gNB emulator SNR uncertainty | B.3.3.4.1 |
| 2 | gNB emulator DL EVM | B.3.3.4.2 |
| 3 | gNB emulator fading model impairments | B.3.3.4.3 |
| Note 1: Handling of effects related to isolation and alignment of Horizontal / Vertical polarisation is not defined Note 2: Handling of effects related to Quality of Quiet zone is not defined |  |  |

### B.3.3.3	Uncertainty assessment

The uncertainty assessment tables are organized as follows:

-	The uncertainty assessment for T-put measurement at defined SNR, is provided in Table B.3.3.3-1

Table B.3.3.3-1: Uncertainty assessment for T-put test with defined SNR at reference point

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| During T-put measurement |  |  |  |  |  |
| 1 | gNB emulator SNR uncertainty | 0.3dB | Normal | 2.00 | 0.15 |
| 2 | gNB emulator DL EVM | - | One-sided, beneficial | - | 0 |
| 3 | gNB emulator fading model impairments Note 3 | [0.5dB] | Normal | 2.00 | [0.25] |
| SNR Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [0.57] |
| Note 1: Handling of effects related to isolation and alignment of Horizontal / Vertical polarisation is not defined Note 2: Handling of effects related to Quality of Quiet zone is not defined Note 3: The value is same as for LTE and shall be verified for other channel models by RAN5 during detailed MU assessment |  |  |  |  |  |

### B.3.3.4	Measurement error contribution descriptions

#### B.3.3.4.1	gNB emulator SNR uncertainty

See B.3.1.4.1

#### B.3.3.4.2	gNB emulator Downlink EVM

See B.3.1.4.2

#### B.3.3.4.3	gNB emulator fading model impairments

See B.3.1.4.3

### B.3.3.5	Assessment of testable SNR range

The Assessment of testable SNR range follows the same principle as B.3.1.5, but the values related to the test system are different and provide a different testable SNR range.

#### B.3.3.5.1	Method and Parameters

The method is the same as B.2.1.5.1, but the values related to the test system are different. The calculation of noise level is in clause 7.2.1.3.

#### B.3.3.5.2	Void

#### B.3.3.5.3	Void

#### B.3.3.5.4	SNR range for SNRRP - SNRBB ≤ 1dB

Based on the method of setting the noise from the Test system to give a maximum of 1dB degradation in overall SNR between reference point and baseband, we can then work back through the signal chain to determine how high the SNR can be set. As the noise is set to a fixed level, the maximum SNR is set by the test system power amplifier and the channel bandwidth to be tested.

The SNR upper bound depends on the type of test system. For the Indirect Far field (IFF) setup the diagram below illustrates the principle, and is based on the “IFF 100MHz” tab of the accompanying spreadsheet.

The process works back through the signal chain, from left to right in the diagram.

Figure B.3.3.5.4-1: Estimation of single band UE SNR range for Indirect far field (IFF)

The test equipment must supply at least the wanted noise level at the reference point. If the noise was lower, the degradation in SNR would be greater than 1dB, and may cause a conformant UE to fail.

The accuracy of setting the signal and noise levels has been taken as +/-[6.0]dB. The uncertainty is subject to further analysis by RAN5.

During conformance test, the test system will need to find the UE Rx beam peak. At present an allowance of [0.5] dB has been included, but is subject to further analysis by RAN5.

Inclusion of these two contributions directly reduces the maximum SNR that can be measured by a test system for a given channel bandwidth. To find the maximum SNR that can be measured by a test system with a specific Channel BW, the baseband SNR in the spreadsheet is increased until the value “Wanted signal + headroom, dBm/Ch BW” is just below the “TE Power amplifier 1dB compression, dBm” value. The resulting values are given in Table B.3.3.5.4-1. Single band UE values are obtained by setting the UE multi-band relaxation factor to 0dB.

Table B.3.3.5.4-1: Predicted SNR upper bound values for Indirect far field (IFF)

|  | Channel Bandwidth | Maximum SNR |
| --- | --- | --- |
| Single band UE | 100MHz | [19.7dB] |
|  | 200MHz | [16.7dB] |
| Multi-band UE Note 1 | 100MHz | [17.7dB] |
|  | 200MHz | [14.6dB] |
| Note 1: For ∑MBp from TS 38.101-2 Table 6.2.1.3-4 allow up to 2dB |  |  |

Note that these are UE baseband SNR values, so the Reference point figures used in TS 38.101-4 will be 1dB higher. Values are based on UE parameters and currently foreseen test equipment limitations, and could be improved in future.

An example of SNR calculation for IFF method is provided in “Spreadsheet 2 - Demod SNR range calculator.xls” file attached to the TR.

<< End of text changes >>

<< Handling of attachments >>

## Add the following attached files to TR 38.810:

## “Spreadsheet 1 - RRM SNR range calculator v2”

## “Spreadsheet 2 - Demod SNR range calculator v2”

## Please remove the existing files:

## “Spreadsheet 1 - RRM SNR range calculator”

## “Spreadsheet 2 - Demod SNR range calculator”

<< End of changes >>