3GPP TSG- RAN4 Meeting #92	R4-1907981

# Ljubljana, Slovenia, 26th - 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Absolute SS-RSRP accuracy to include antenna gain |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 38.215 clause 5.1.1 specifies the that FR2 SS-RSRP shall be measured based on the combined signal from antenna elements, but this point (inside the UE) is not accessible and cannot be used to specify absolute SS-RSRP reporting accuracy. The Absolute SS-RSRP reporting accuracy is affected by the gain of the UE antennas, but the existing accuracy values exclude antenna gain variation. A more detailed explanation is given in R4-1907974. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the FR2 SS-RSRP absolute accuarcy requirements to include UE antenna gain variation. The values are defined in the Rx beam peak direction, with a (negative-only) relaxation for speherical coverage directions. The antenna gain variation is expected to be power class dependent, so provision is made for different values in each power class. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SS-RSRP absolute accuracy would remain an unmeasurable requirement, and is difficult to apply in a network to relate for example applied power to thresholds. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.3.1.1, 10.1.5.1.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

### 10.1.3	Intra-frequency RSRP accuracy requirements for FR2

#### 10.1.3.1	Intra-frequency SS-RSRP accuracy requirements

##### 10.1.3.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.3.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.3.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR2

| Accuracy for Rx beam peak Angle of Arrival Note 5 |  |  |  |  |  |  |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE Power class |  |  |  |  |  |  |  | SSB Ês/Iot | Io Note 2 range |  |  |
| 1 |  | 2 |  | 3 |  | 4 |  |  | Minimum Io |  | Maximum Io |
| Norm | Ext | Norm | Ext | Norm | Ext | Norm | Ext |  |  |  |  |
| dB |  | dB |  | dB |  | dB |  | dB | dBm / SCSSSB Note 1 | dBm/BWChannel | dBm/BWChannel |
| [6+a1] | [ 9+a1] | [6+a2] | [9+a2] | [6+a3] | [9+a3] | [6+a4] | [9+a4] | ≥-6 | Same value as SSB_RP in Table B.2.2-2 Note 4 | N/A | -70 |
| [8+a1] | [11+a1] | [8+a2] | [11+a2] | [8+a3] | [11+a3] | [8+a4] | [11+a4] |  | N/A | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: According to UE Power class, operating band, SCS and angle of arrival. Note 5: For Angles of arrival within EIS Spherical coverage, the lower limit of reported value is decreased by (Minimum SSB_RP in Spherical coverage - Minimum SSB_RP in Rx Beam Peak), where the Minimum SSB_RP values are specified in Table B.2.2-2. |  |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table 10.1.3.1.1-1:

- The values of a1, a2, a3 and a4 for Power classes 1 to 4 respectively are FFS, where ax is the range of antenna gain variation in beam peak direction according to the UE Power class.

<< Unchanged sections skipped >>

### 10.1.5	Inter-frequency RSRP accuracy requirements for FR2

#### 10.1.5.1	Inter-frequency SS-RSRP accuracy requirements

##### 10.1.5.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR2 that is on a different frequency than the serving cell.

The accuracy requirements in Table 10.1.5.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.5.1.1-1: SS-RSRP Inter frequency absolute accuracy in FR2

| Accuracy for Rx beam peak Angle of Arrival Note 5 |  |  |  |  |  |  |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE Power class |  |  |  |  |  |  |  | SSB Ês/Iot | Io Note 2 range |  |  |
| 1 |  | 2 |  | 3 |  | 4 |  |  | Minimum Io |  | Maximum Io |
| Norm | Ext | Norm | Ext | Norm | Ext | Norm | Ext |  |  |  |  |
| dB |  | dB |  | dB |  | dB |  | dB | dBm / SCSSSB Note 1 | dBm/BWChannel | dBm/BWChannel |
| [6+a1] | [ 9+a1] | [6+a2] | [9+a2] | [6+a3] | [9+a3] | [6+a4] | [9+a4] | ≥-4 | Same value as SSB_RP in Table B.2.3-2 Note 4 | N/A | -70 |
| [8+a1] | [11+a1] | [8+a2] | [11+a2] | [8+a3] | [11+a3] | [8+a4] | [11+a4] |  | N/A | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: According to UE Power class, operating band, SCS and angle of arrival. Note 5: For Angles of arrival within EIS Spherical coverage, the lower limit of reported value is decreased by (Minimum SSB_RP in Spherical coverage - Minimum SSB_RP in Rx Beam Peak), where the Minimum SSB_RP values are specified in Table B.2.3-2. |  |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table 10.1.5.1.1-1:

- The values of a1, a2, a3 and a4 for Power classes 1 to 4 respectively are FFS, where ax is the range of antenna gain variation in beam peak direction according to the UE Power class.

<< End of changes >>