3GPP TSG-RAN WG4 Meeting #92                                                               R4-1908098

Ljubljana, SI, 26th-30th August, 2019

Source:	Samsung, Bell Mobility, TELUS, Nokia

Title:	TP for TR 37.716-11-11: DC_7_n66 and DC_7-7_n66

Agenda item:			10.3.2

Document for:	Approval

# 1. Introduction

In RAN#84 meeting, the revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band) was approved [1], in which the latest EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band) was attached. This contribution is a text proposal for TR 37.716-11-11 to include DC_7A_n66A, DC_7A-7A_n66A and DC_7C_n66A.

# 2. Background

Based on the co-existence study, the IMD4 of 2UL DC_7A_n66A will impact the B7 receiver, MSD need define for B7.

Table 1 DC_7A_n66A and DC_3A_n7A operating bands and IMD 4 frequency range

|  | UL frequency range (MHz) | DL frequency range (MHz) | IMD4 frequency range (MHz) |
| --- | --- | --- | --- |
| B7 | 2500-2570 | 2620-2690 | 2560-2840 |
| n66 | 1710-1780 | 2110-2200 |  |
| B3 | 1710-1785 | 1805-1880 | 2560-2855 |
| n7 | 2500-2570 | 2620-2690 |  |

In Table 1, it showed that the IMD4 frequency range of B7 UL and n66 UL will fully cover B7 DL frequency range, which is similar with the case of DC_3A_n7A, since B3 UL frequency range entirely contains n66 UL frequency range, the IMD4 of B7 UL and n66 UL will be fully overlap with the IMD4 frequency range of B3 UL and n7 UL. Therefore, the MSD of DC_7A_n66A due to IMD4 can reuse the MSD of DC_3A_n7A, as shown in Table 2:

Table 2 MSD for DC_7A_n66A

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n66A | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | 7 | 2535 | 10 | 50 | 2655 | 10.2 | FDD | IMD4 |

In additional, DC_7_n66 and DC_7-7_n66 could reuse the TIB,c and RIB,c values of CA_7A-66A.

# 3. Reference

- RP-191568, Revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band a+ 1 NR band), NTT DOCOMO, INC.

# 4. Text Proposal

<Start of Text Proposal>

# 1	Scope

The present document is a technical report for Dual Connectivity (EN-DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address Dual Connectivity (EN-DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL)for the Rel-16 band combinations in Table 1-1. The co-existence analysis and RF front end requirements such as Delta RIB,C and TIB,C are described based on the band combination basis since such information have no difference between the EN-DC configulations consisting with the same E-UTRA band and the same NR band.

Table 1-1: Release 16 EN-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) within FR1

| DC combination | Uplink EN-DC configuration |
| --- | --- |
| DC_1A_n3A | DC_1A_n3A |
| DC_1C_n3A | DC_1C_n3A |
| DC_1A_n5A | DC_1A_n5A |
| DC_1A_n7A | DC_1A_n7A |
| DC_1A_n41A | DC_1A_n41A |
| DC_1A_n50A | DC_1A_n50 |
| DC_2A-2A_n5A | DC_2A_n5A |
| DC_2A_n41A | DC_2A_n41A |
| DC_2C_n41A | DC_2A_n41A |
| DC_3A_n1A | DC_3A_n1A |
| DC_3A-3A_n1A | DC_3A_n1A |
| DC_3A_n3A | DC_3A_n3A |
| DC_3A_n5A | DC_3A_n5A |
| DC_3C_n5A | DC_3A_n5A DC_3C_n5A |
| DC_3A_n20A | DC_3A_n20A |
| DC_3A_n41A | DC_3A_n41A |
| DC_3A_n50A | DC_3A_n50A |
| DC_3A-3A_n77A | DC_3A_n77A |
| DC_3A-3A_n78A | DC_3A_n78A |
| DC_5A_n2A | DC_5A_n2A |
| DC_5A_n71A | DC_5A_n71A |
| DC_5A_n79A | DC_5A_n79A |
| DC_7A_n1A | DC_7A_n1A |
| DC_7A-7A_n1A | DC_7A_n1A |
| DC_7A_n5A | DC_7A_n5A |
| DC_7C_n5A | DC_7A_n5A DC_7C_n5A |
| DC_7A_n66A | DC_7A_n66A |
| DC_7A-7A_n66A | DC_7A_n66A |
| DC_7C_n66A | DC_7A_n66A |
| DC_7A_n71A | DC_7A_n71A |
| DC_7A_n77A | DC_7A_n77A |
| DC_7A-7A_n77A | DC_7A_n77A |
| DC_8A_n1A | DC_8A_n1A |
| DC_8A_n3A | DC_8A_n3A |
| DC_8A_n41A | DC_8A_n41A |
| DC_8A_n41C | DC_8A_n41A DC_8A_n41C |
| DC_8A_n41(2A) | DC_8A_n41A |
| DC_12A_n2A | DC_12A_n2A |
| DC_12A_n71A | DC_12A_n71A |
| DC_13A_n5A | DC_13A_n5A |
| DC_13A_n71A | DC_13A_n71A |
| DC_20A_n1A | DC_20A_n1A |
| DC_20A_n3A | DC_20A_n3A |
| DC_20A_n41A | DC_20A_n41A |
| DC_20A_n50A | DC_20A_n50A |
| DC_25A-25A_n41A | DC_25A_n41A |
| DC_28A_n5A1 | DC_28A_n5A |
| DC_28A_n8A | DC_28A_n8A |
| DC_28A_n41A | DC_28A_n41A |
| DC_28A_n50A | DC_28A_n50 |
| DC_30A_n2A | DC_30A_n2A |
| DC_39A_n41A | DC_39A_n41A |
| DC_40A_n41A | DC_40A_n41A |
| DC_40A_n78A | DC_40A_n78A |
| DC_40C_n78A | DC_40C_n78A |
| DC_40A_n79A | DC_40A_n79A |
| DC_66A_n2A | DC_66A_n2A |
| DC_66A-66A_n5A | DC_66A_n5A |
| DC_66A-66A-66A_n5A | DC_66A_n5A |
| DC_66A_n25A | DC_66A_n25A |
| DC_71A_n5A | DC_71A_n5A |
| NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |

<Unchanged text omitted>

## 6.1.x	DC_7_n66, DC_7-7_n66

### 6.1.x.1	Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7_n66 | 7 | n66 |  |
| DC_7-7_n66 | CA_7-7 | n66 |  |

### 6.1.x.2	Configuration for DC

Table 6.1.x.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_7A_n66A | DC_7A_n66A | 7A | n66A |
| DC_7A-7A_n66A | DC_7A_n66A | CA_7A-7A | n66A |
| DC_7C_n66A | DC_7A_n66A | CA_7C | n66A |

### 6.1.x.3	Maximum output power for DC

Table 6.1.x.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_7A_n66A | 23 | +2/-31 |
| NOTE 1: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

### 6.1.x.4	Spurious emission band UE co-existence for DC

Table 6.1.x.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7A_n66A | E-UTRA Band 2, 4, 5, 7, 12, 13, 17, 26, 28, 29, 42, 43, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

### 6.1.x.5	MSD analysis for DC

For Band 7 +Band n66 2UL DC 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis were calculated and listed in Table 6.1.x.5-1.

Table 6.1.x.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 12500 | 12850 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| Two tone 2nd order IMD products (MHz) | 720 | 860 | 4210 | 4350 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| Two-tone 3rd order IMD products (MHz) | 850 | 1060 | 3220 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| Two-tone 3rd order IMD products (MHz) | 5920 | 6130 | 6710 | 6920 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| Two-tone 4th order IMD products (MHz) | 2560 | 2840 | 5720 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| Two-tone 4th order IMD products (MHz) | 7630 | 7910 | 9210 | 9490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| Two-tone 4th order IMD products (MHz) | 1440 | 1720 | 8420 | 8700 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| Two-tone 5th order IMD products (MHz) | 8220 | 8570 | 4270 | 4620 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| Two-tone 5th order IMD products (MHz) | 11710 | 12060 | 9340 | 9690 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| Two-tone 5th order IMD products (MHz) | 3940 | 4290 | 10 | 340 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| Two-tone 5th order IMD products (MHz) | 10920 | 11270 | 10130 | 10480 |

Based on Table 6.1.x.5-1;

- There is no harmonic issue at legacy E-UTRA Band 7 and NR band n66

- 2nd order IMD may also fall into B12, B13, B14, B17, B20, B26, B27, B28, B29, B67, B68, B85 and TDD B44.

- 3rd order IMD may also fall into B5, B6, B8, B18, B19, B26, B27, B69 and TDD B38, B41, B42, B46, B47, n77, n78.

- 4th order IMD may also fall into B7, B11, B21, B24, B32, B69, B74, B75 and TDD B38, B41, B45, B46, B47, B50.

- 5th order IMD may also fall into TDD n77, n79.

The 4th IMD will be impact to the own received band 7.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.1.x.5-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.x.5-2: 2UL Band 7 +Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd order harmonic n66, IMD3, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | 3rd order harmonic n66 |
|  | 5470 | - | 5725 | Yes |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 3rd order harmonic n66, IMD4 |

### 6.1.x.6	∆TIB and ∆RIB values

For DC_7_n66 and DC_7-7_n66, the TIB,c and RIB,c values are given in the below tables:

Table 6.1.x.6-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n66 DC_7-7_n66 | 7 | 0.5 |
|  | n66 | 0.5 |

Table 6.1.x.6-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n66 DC_7-7_n66 | 7 | 0.5 |
|  | n66 | 0.5 |

### 6.1.x.7	MSD

Table 6.1.x.7-1 lists the MSD required for the dual connectivity configuration.

Table 6.1.x.7-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_7A_n66A DC_7A-7A_n66A DC_7C_n66A | 7 | 2535 | 10 | 50 | 2655 | 10.2 | FDD | 4th IMD |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |

<End of Text Proposal>