3GPP TSG-RAN WG4 Meeting #92	R4-1908214

# Ljubljana, Slovenia, 26th – 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | xxxx | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR FR1 PDSCH requirements finalization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Finilization of requirements for HST and 2nd priority TDD patterns - Correction of requirements for Tests 1-5 and 1-6 in Tables 5.2.2.2.1-3 and 5.2.3.2.1-3 due to mistake in excel spread sheet with collection of simulation results - Corection of TRS configuration for TDD HST tests |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed square brackets - Swap requiremnets points for Tests 1-5 and 1-6 in Tables 5.2.2.2.1-3 and 5.2.3.2.1-3 - Changed TRS frequency occupation from “BWP size” to “52 PRB” due to limitation on TRS configuration from TS 38.214 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-15 Normal PDSCH resuirements are not finilized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.1, 5.2.2.2.1, 5.2.3.1.1, 5.2.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

##### 5.2.2.1.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.2.1.1-3 and Table 5.2.2.1.1-4, with the addition of test parameters in Table 5.2.2.1.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.1-1.

Table 5.2.2.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 1-2, 1-3, 1-5, 2-1, 2-2 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 2 receive antenna conditions. | 1-4 |
| Verify the PDSCH mapping Type A enhanced performance requirement Type X under 2 receive antenna conditions and with 2 MIMO layers. | 3-1 |

Table 5.2.2.1.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Test 1-1 2 for other tests |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Tests 1-1, 1-5 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | Test 1-5: 10 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-5: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 8 for Test 1-4 4 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.2.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | TDLB100-400 | 2x2, ULA Low | 70 | -0.8 |
| 1-2 | R.PDSCH.1-1.2 FDD | 10 / 15 | QPSK, 0.30 | TDLC300-100 | 2x2, ULA Low | 70 | 0.2 |
| 1-3 | R.PDSCH.1-4.1 FDD | 10 / 15 | 256QAM, 0.82 | TDLA30-10 | 2x2, ULA Low | 70 | 24.6 |
| 1-4 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 30 | 1.1 |
| 1-5 | R.PDSCH.1-8.1 FDD | 10 / 15 | 16QAM, 0.48 | HST-750 | 1x2 | 70 | 6.5 |

<SKIP UNCHANGED PART>

##### 5.2.2.2.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.2.2.1-3 and Table 5.2.2.2.1-4, with the addition of test parameters in Table 5.2.2.2.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.1-1.

Table 5.2.2.2.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 1-2, 1-3, 1-5, 1-6, 1-7, 1-8, 1-9, 2-1, 2-2 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 2 receive antenna conditions. | 1-4 |
| Verify the PDSCH mapping Type A enhanced performance requirement Type X under 2 receive antenna conditions and with 2 MIMO layers. | 3-1 |

Table 5.2.2.2.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Tests 1-1, 1-8, 1-9 2 for other tests |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Tests 1-1, 1-7, 1-8, 1-9 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | Tests 1-8, 1-9: l0 = 4 for CSI-RS resource 1 and 3 l0 = 8 for CSI-RS resource 2 and 4 Other tests; Table 5.2-1. |
|  | CSI-RS periodicity | Slots | Test 1-7: 20 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-7: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
|  | Frequency Occupation |  | Test 1-7: Start PRB 0 Number of PRB = 52 Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 16 for Test 1-4 10 for Test 1-9 8 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.2.2.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-1.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-1A | TDLB100-400 | 2x2, ULA Low | 70 | -1.1 |
| 1-2 | R.PDSCH.2-1.2 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLC300-100 | 2x2, ULA Low | 70 | 0.2 |
| 1-3 | R.PDSCH.2-4.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low | 70 | 25.3 |
| 1-4 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 2x2, ULA Low | 30 | 1.6 |
| 1-5 | R.PDSCH.2-5.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-2 | TDLA30-10 | 2x2, ULA Low | 70 | -0.9 |
| 1-6 | R.PDSCH.2-6.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-3 | TDLA30-10 | 2x2, ULA Low | 70 | -0.8 |
| 1-7 | R.PDSCH.2-10.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | HST-1000 | 1x2 | 70 | 6.6 |
| 1-8 | R.PDSCH.2-11.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-5 | TDLB100-400 | 2x2, ULA Low | 70 | -1.0 |
| 1-9 | R.PDSCH.2-12.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-6 | TDLB100-400 | 2x2, ULA Low | 70 | -1.1 |

<SKIP UNCHANGED PART>

##### 5.2.3.1.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.3.1.1-3, Table 5.2.3.1.1-4, Table 5.2.3.1.1-5 and Table 5.2.3.1.1-6, with the addition of test parameters in Table 5.2.3.1.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.1-1.

Table 5.2.3.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 1-2, 1-3, 1-5, 2-1, 2-2, 3-1, 4-1 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 4 receive antenna conditions. | 1-4 |
| Verify the PDSCH mapping Type A enhanced performance requirement Type X under 4 receive antenna conditions and with 3 MIMO layers. | 5-1 |

Table 5.2.3.1.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Test 1-1 WB for Test 3-1 2 for other tests |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Test 1-1, 1-5 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | Test 1-5: 10 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-5: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 8 for Test 1-4, 2-1 4 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.3.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | TDLB100-400 | 2x4, ULA Low | 70 | -3.5 |
| 1-2 | R.PDSCH.1-1.2 FDD | 10 / 15 | QPSK, 0.30 | TDLC300-100 | 2x4, ULA Low | 70 | -2.9 |
| 1-3 | R.PDSCH.1-4.1 FDD | 10 / 15 | 256QAM, 0.82 | TDLA30-10 | 2x4, ULA Low | 70 | 21.0 |
| 1-4 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x4, ULA Low | 30 | -1.5 |
| 1-5 | R.PDSCH.1-8.1 FDD | 10 / 15 | 16QAM, 0.48 | HST-750 | 1x4 | 70 | 3.5 |

<SKIP UNCHANGED PART>

##### 5.2.3.2.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.3.2.1-3, Table 5.2.3.2.1-4, Table 5.2.3.2.1-5 and Table 5.2.3.2.1-6, with the addition of test parameters in Table 5.2.3.2.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.1-1.

Table 5.2.3.2.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under4 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 1-2, 1-3, 1-5, 1-6, 1-7, 1-8, 1-9, 2-1, 2-2, 3-1, 4-1 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 4 receive antenna conditions. | 1-4 |
| Verify the PDSCH mapping Type A enhanced performance requirement Type X under 4 receive antenna conditions and with 3 MIMO layers. | 5-1 |

Table 5.2.3.2.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Tests 1-1, 1-8, 1-9 WB for Test 3-1 2 for other tests |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Tests 1-1, 1-7, 1-8, 1-9 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | Tests 1-8, 1-9: l0 = 4 for CSI-RS resource 1 and 3 l0 = 8 for CSI-RS resource 2 and 4 Other tests; Table 5.2-1. |
|  | CSI-RS periodicity | Slots | Test 1-7: 20 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-7: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
|  | Frequency Occupation |  | Test 1-7: Start PRB 0 Number of PRB = 52 Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 16 for Test 1-4 8 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.3.2.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-1.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-1A | TDLB100-400 | 2x4, ULA Low | 70 | -4.1 |
| 1-2 | R.PDSCH.2-1.2 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLC300-100 | 2x4, ULA Low | 70 | -2.7 |
| 1-3 | R.PDSCH.2-4.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 70 | 21.6 |
| 1-4 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 2x4, ULA Low | 30 | -1.2 |
| 1-5 | R.PDSCH.2-5.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-2 | TDLA30-10 | 2x4, ULA Low | 70 | -3.8 |
| 1-6 | R.PDSCH.2-6.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-3 | TDLA30-10 | 2x4, ULA Low | 70 | -3.6 |
| 1-7 | R.PDSCH.2-10.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | HST-1000 | 1x4 | 70 | 3.6 |
| 1-8 | R.PDSCH.2-11.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-5 | TDLB100-400 | 2x4, ULA Low | 70 | -4.0 |
| 1-9 | R.PDSCH.2-12.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-6 | TDLB100-400 | 2x4, ULA Low | 70 | -4.0 |

END OF CHANGE