3GPP TSG-RAN WG4 Meeting #92	R4-1908651

# Ljubljana, Slovenia,  26th Aug, 2019 - 30th Aug, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | 0878 | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS37.141 Corrections on NBB requirement (section 7.4) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Similar as corrections as R4-1907664, the RB number of interfering signal should not exceed 99 for 20MHz, other further clarifications are also mirrored from TS 38.104 NBB requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add two notes for NBB requirement in section 7.4.5.2 which is from TS38.104 NBB requirement. - Corrections RB numbers from 100 to 99 for NBB requirement; - Other editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NBB requirement is not defined correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.4.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS37.145-1, 37.145-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of CR>

7.4.5.2	General narrowband blocking test requirement

For the narrowband blocking requirement, the interfering signal shall be an E-UTRA 1RB signal as specified in Annex A.3.

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For BS operating in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 3MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For BS capable of multi-band operation, the requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least 3MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal coupled to the Base Station antenna input, using the parameters in Table 7.4.5.2-1 the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], subclause 7.2.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.104 [3], subclause 7.2.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.105 [4], subclause 7.2.

-	For any measured GSM/EDGE carrier, the conditions are specified in TS 45.005 [6], Annex P.2.1.

-	For any measured NB-IoT carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [5], subclause 7.2.

-	For any measured NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [27], subclause 7.2.

Table 7.4.5.2-1: Narrowband blocking requirement

| Base Station Type | RAT of the carrier | Wanted signal mean power [dBm] (Note 1,2,6) | Interfering signal mean power [dBm] | Interfering RB (Note 3,7) centre frequency offset from the Base Station RF Bandwidth edge or sub-block edge inside a gap [kHz] |
| --- | --- | --- | --- | --- |
| Wide Area BS | NR, E-UTRA, NB-IoT (Note 4), UTRA and GSM/EDGE | PREFSENS + x dB (Note 2) | -49 | ±(240 +m*180), m=0, 1, 2, 3, 4, 9, 14 (Note 5) ±(557.5 +m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 (Note 6) |
| Medium Range BS |  |  | -44 |  |
| Local Area BS |  |  | -41 |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see subclause 7.2 in TS 37.104. NOTE 2: “x” is equal to 6 in case of NR, E-UTRA or UTRA wanted signals and equal to 3 in case of GSM/EDGE wanted signal. “x” is specified in Table 7.4.2-2 for NB-IoT. NOTE 3: Interfering signal (E-UTRA 3MHz) consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF Bandwidth edge. NOTE 4: For NB-IoT, the mentioned desensitized values consider only one NB-IoT PRB in the guard band, which is placed adjacent to the E-UTRA PRB edge as close as possible (i.e., away from edge of channel bandwidth). NOTE 5: Applicable for channel bandwidths equal to or below 20 MHz. NOTE 6: Applicable for channel bandwidths above 20MHz. NOTE 6: 7.5 kHz shift is not applied to the wanted signal of NR. NOTE 7: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |  |  |

Table 7.4.5.2-2: “x” for NB-IoT wanted signals

| Operation mode | LTE channel bandwidth for in-band/guard band operation | x |
| --- | --- | --- |
| Standalone | - | 12 |
| In Band | 3 MHz | 11 |
|  | 5 MHz | 9 |
|  | 10 MHz | 6 |
|  | 15 MHz | 6 |
|  | 20 MHz | 6 |
| Guard band | 5 MHz | 13 |
|  | 10 MHz | 6 |
|  | 15 MHz | 6 |
|  | 20 MHz | 6 |

< End of CR>