3GPP TSG-RAN WG4 Meeting #92	R4-1908702

# Ljubljana, SI, 26th – 30th, Aug, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on TS38.133 for TCI state switching(section 8.10) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-8-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update to TCI state switching delay requirements: - In the known condition for TCI state, the TCI state shall also remain detectable with the same Spatial Rx parameter during the TCI state switching period. - Only L1-RSRP measurement time in TL1-RSRP should be considered in unknown TCI state switching. TReport should be 0. - RAN4 should differentiate the TCI state switching requirement for QCL-TypeD and other QCL types. The TL1-RSRP-meas =0 when TCI state switching is for {QCL-TypeA, QCL-TypeB, QCL-TypeC}. - UE is expected the PDCCHs scheduling in n+X slots right after the time of finishing the TCI-state switching to UE. Otherwise, UE will start the procedure for unknown TCI state switch and to try different Rx beams. - It should clarify only one TCI state configured in TCI state list tci-StatesToAddModList by RRC reconfiguration in the RRC based TCI state switching delay. - During the RRC based TCI state switching, the UE is not required to receive PDCCH/PDSCH or transmit PUCCH/PUSCH during the RRC reconfiguration procedure and the SSB/CSI-RS occasion time for L1-RSRP measurement with the target TCI state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update definition of known TCI state. - Update T L1-RSRP with TReport = 0. - Update requirement for L1-RSRP measurement time with different QCL type. - The PDCCH/PDSCH with target TCI state will be scheduled no later than slot n+X right after receiving PDSCH carrying MAC-CE activation command at slot n. - Clarify the scenario for RRC based TCI state delay requirement - Update the schedulign restriction for RRC based TCI state delay - Update the wording |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TCI state swithing delay requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

8.10	Active TCI state switching delay

8.10.1	Introduction

The requirements in this section apply for a UE configured with one or more TCI state configurations on serving cell in MR-DC or standalone NR. UE shall complete the switch of active TCI state within the delay defined in this section.

### 8.10.2	Known conditions for TCI state

The TCI state is known if it has been meeting the following conditions:

- TCI state switch is within [X] ms of last transmission of the resource for beam reporting/ measurement for the target TCI state

- The UE has sent at least 1 measurement report for the target TCI state

- The TCI state shall remain detectable with the same spatial reception parameter during the TCI state switching period

  - SNR of the TCI state is > -3dB

Otherwise, the TCI state is unknown.

### 8.10.3	MAC-CE based TCI state switch delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ THARQ +3ms +TOk*(Tfirst-SSB + TSSB-proc). The UE can assume that PDCCH/PDSCH with target TCI state will be scheduled no later than slot n+ THARQ +3ms +TOk*(Tfirst-SSB + TSSB-proc)+ X right after receiving PDSCH carrying MAC-CE activation command at slot n. Otherwise, UE will fulfil the requirement for MAC-CE based unknown TCI state switch delay.

The UE shall be able to receive on the old TCI state until slot n+ THARQ +3ms +TOk*(Tfirst-SSB).

Where, THARQ is the timing between DL data transmission and acknowledgement as specified in [7];

Tfirst-SSB is time to first SSB transmission after TCI state command is received by the UE;

TSSB-proc =2ms;

TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise.

If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ THARQ +3ms + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) .

The UE shall be able to receive on the old TCI state until slot n+ THARQ +3ms+ TL1-RSRP +TOk*(Tfirst-SSB).

Where, T L1-RSRP is the time for L1-RSRP measurement when TCI-state switch involves qcl-Type with typeD, defined as

  - TBM_Measurement_Period_SSB for SSB in FR2 as specified in section 9.5.4.1, with the assumption of M=1

  - TBM_Measurement_Period_CSI-RS for CSI-RS  in FR2 as specified in section 9.5.4.2

    - with the assumption of M=1 for periodic CSI-RS

    - for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

  - TReport=0.

Otherwise, T L1-RSRP =0 when TCI-state switch only involves other qcl-Type.

Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE;

TOuk = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement

If the target TCI state is not measured and reported by the UE prior to TCI state switching, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ THARQ +3ms + TL1-RSRP + TOuk* (Tfirst-SSB + TSSB-proc). The UE shall be able to receive on the old TCI state until slot n+ THARQ +3ms+ TL1-RSRP +TOk*(Tfirst-SSB).

During MAC CE based TCI state switch the UE is allowed an interruption due to one shot timing adjustment on the serving or any activated serving cells as defined in section 8.2.

### 8.10.4	DCI based TCI switch delay

If the target TCI state is known, when a UE is configured with the higher layer parameter tci-PresentInDCI which is set as 'enabled' for the CORESET scheduling the PDSCH at slot n, UE shall be able to receive PDSCH or transmit PUSCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+timeDurationForQCL, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], the value of timeDurationForQCL is defined in TS 38.306[14].

The known condition for TCI state defined in section 8.10.2 is applied.

During DCI based TCI state switch the UE is allowed an interruption due to one shot timing adjustment on the serving or any activated serving cells as defined in section 8.2.

### 8.10.5	RRC based TCI state delay

If the target TCI state is known, upon receiving PDSCH carrying RRC activation command with only one TCI state in the TCI state list at slot n, UE shall be able to receive PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ TRRC_processing  +TOk*(Tfirst-SSB + TSSB-proc). Where TRRC_processing is the RRC processing delay, Tfirst-SSB, TSSB-proc and TOk are as defined in Section 8.10.3. The UE can assume that PDCCH/PDSCH with target TCI state will be scheduled no later than slot n+ TRRC_processing  +TOk*(Tfirst-SSB + TSSB-proc) +X right after receiving PDSCH carrying RRC activation command with only one TCI state in the TCI state list at slot n. Otherwise, UE will fulfil the requirement for RRC based unknown TCI state switch delay.

If the target TCI state is unknown, upon receiving PDSCH carrying RRC activation command with only one TCI state in the TCI state list at slot n, UE shall be able to receive PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs no later than at slot n+ TRRC_processing  +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc) . Where TRRC_processing is the RRC processing delay, Tfirst-SSB and TOuk are as defined in Section 8.10.3.

During the RRC based TCI state switching, the UE is not required to receive PDCCH/PDSCH or transmit PUCCH/PUSCH during the RRC reconfiguration procedure and the SSB/CSI-RS occasion time for L1-RSRP measurement QCL-ed with the target TCI state.

During RRC based TCI state switch the UE is allowed an interruption due to one shot timing adjustment on the serving or any activated serving cells as defined in section 8.2.

### 8.10.6	Active TCI state list update delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDCCH to schedule PDSCH with the new TCI state no later than slot n+ THARQ +3ms +TOk*(Tfirst-SSB + TSSB-proc). Where THARQ, Tfirst-SSB, TSSB-proc and TOk are as defined in Section 8.10.3.

End of Change