3GPP TSG-WG RAN4 Meeting #92	R4-1908979

# Ljubljana, Slovenia, 26th – 30th August, 2019

Source: 	Nokia, Nokia Shanghai Bell, [Verizon]

Title: 					TP for TR 37.716-21-11 Introduction of DC_2A-66A-n66A_BCS0

Agenda Item: 			9.4.2 [DC_R16_2BLTE_1BNR_3DL2UL-Core]

Document for:	Approval

# Introduction

This TP introduces two-band EN-DC configurations of DC_2A-66A_n66A_BCS0 to TR 37.716-21-11.

**************************** Start of changes ********************************************

## 5.1.X	DC_2A-66A_n66A

### 5.1.X.1	Operating bands for DC

Table 5.1.X.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n66 | CA_2-66 | n66 | No |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n66A | DC_2A_n66A | CA_2A-66A | n66A |

### 5.1.X.3	Coexistence studies

Based on co-existence studies of DC_2A_n66A in TR 37.863-01-01, the third order IMD may fall into band 2 and/or n66; the fifth order IMD may fall into band 66/n66.

### 5.1.X.4	∆TIB and ∆RIB values

For DC_2-66_n66, the TIB,c and RIB,c values are derived from CA_2-66 in TS 36.101.

Table 5.X.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n66 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.X.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

### 5.1.X.5	REFSENS requirements

MSD due to IMD products are already specifed for 2DL2UL DC combos (DC_2A_n66A) in TS 38.101-3, where band 2 and n66 MSD are specified. To avoid the specification complexity, band 66 MSD is not specified as the same MSD is applied if band 66 carrier frequency is replaced by band n66 carrier frequency.

**************************** End of changes ********************************************