3GPP TSG-RAN WG4 Meeting #92	R4-1909173

Ljubljana, Slovenia, 26th– 30th August, 2019

Source: 	Huawei, HiSilicon

Title:	TP on system scenarios for NB-IoT co-existence with NR

Agenda Item:	8.12.2

Document for:	Approval

1	Introduction

In last RAN plenary meeting #83, the revised WID was approved to add the description on the scope of NB-IoT coexistence with NR.

- Study aspects of FDD/TDD NB-IoT coexistence with NR [RAN4, RAN1, RAN2]

- For NB-IoT in-band, guard band and standalone operation co-existence with NR, including the case of NR configured in 15kHz SS Block SCS and the case of 30kHz SS Block SCS as specified in 38.101-1, investigate the following: [RAN4]

    - 15KHz, 30KHz, and 60KHz numerologies for NR FR1 concerned bands, with higher priority given first to 15kHz and then to 30kHz

    - Channel raster, PRB and subcarrier grid alignment between NB-IoT and NR

    - Study feasible NB-IoT carrier(s) placement allocation without RF backward compatibility impact and compatible with Rel’13 NB-IoT and Rel’15 NR, to operate simultaneously within various NR channel bandwidths

    - Study if the +6dB downlink RE power boosting can still be allowed for both in-band and guard band operation modes when co-existing with NR

    - Synchronization issue between NR and NB-IoT, including timing advance

    - Frequency band support in NB-IoT and NR

    - Testability applicable to RF

    - Coexistence between R15 NR and R13/R14/R15 NB-IoT.

Note: After RAN1 concludes the objective on R16 NB-IoT coexistence aspects, evaluate coexistence between R15 NR and R16 NB-IoT.

In this contribution we provide some discussion on the system scenarios of NB-IoT coexistence with NR.

2	Discussion

As approved in [2], on system scenarios the clarification on the current definition of NB-IoT operation modes is needed.

  - WF on system scenarios

    - To clarify the current definition of NB-IoT operation modes when NB-IoT is deployed within NR  channel bandwidth, including when spectrum is refarmed from E-UTRA to NR

        - whether NB-IoT in-band or guard band operation without a hosting E-UTRA carrier would have backward compatibility impact on Rel’13/14/15 NB-IoT UE, which may expect the presence of a hosting E-UTRA carrier, especially for NB-IoT in-band operation

For NB-IoT operating in LTE, system scenarios have been divided into 3 following operations,

- Category NB1/NB2 stand-alone operation: category NB1/NB2 is operating standalone when it utilizes its own spectrum, for example the spectrum used by GERAN systems as a replacement of one or more GSM carriers, as well as scattered spectrum for potential IoT deployment.

- Category NB1/NB2 guard band operation: category NB1/NB2 is operating in guard band when it utilizes the unused resource block(s) within a E-UTRA carrier’s guard-band.

- Category NB1/NB2 in-band operation: category NB1/NB2 is operating in-band when it utilizes the resource block(s) within a normal E-UTRA carrier.

When it comes to NB-IoT co-existence with NR, depend on the utilized spectrum for NB-IoT operations, 3 system scenarios should be considered,

- NB-IoT standalone operating: NB-IoT is operating standalone when it utilizes its own spectrum

- NB-IoT operating in NR guard band: NB-IoT is operating when it utilizes the unused resource block(s) within a NR carrier’s guard-band

- NB-IoT operating in NR in-band: NB-IoT is operating when it utilizes the resource block(s) within a normal NR carrier.

3	Conclusions

In this contribution we provide discussion on system scenarios of NB-IoT coexistence with NR. It is proposed to approve the attached TP for the TR of the WI.

4	References

[1] RP-190757, Revised WID: Additional enhancements for NB-IoT, Huawei

[2] R4-1905203, WF on coexistence of NB-IoT with NR, Huawei, HiSilicon, Ericsson, CHTTL, SGS Wireless, Nokia, ZTE, Dish, T-Mobile USA

[3] R4-1907809, “WF on coexistence of NB-IoT with NR”, Huawei, HiSilicon, Ericsson, Nokia, ZTE, DISH, T-Mobile USA, CHTTL

Text Proposal

<Start of TP>

5.1	General

### 5.1.1	system scenarios

For NB-IoT operating in LTE, system scenarios have been divided into 3 following operations,

- Category NB1/NB2 stand-alone operation: category NB1/NB2 is operating standalone when it utilizes its own spectrum, for example the spectrum used by GERAN systems as a replacement of one or more GSM carriers, as well as scattered spectrum for potential IoT deployment.

- Category NB1/NB2 guard band operation: category NB1/NB2 is operating in guard band when it utilizes the unused resource block(s) within an E-UTRA carrier’s guard-band.

- Category NB1/NB2 in-band operation: category NB1/NB2 is operating in-band when it utilizes the resource block(s) within a normal E-UTRA carrier.

When it comes to NB-IoT co-existence with NR, depend on the utilized spectrum for NB-IoT operations, 3 system scenarios should be considered,

- NB-IoT standalone operating: NB-IoT is operating standalone when it utilizes its own spectrum

- NB-IoT operating in NR guard band: NB-IoT is operating when it utilizes the unused resource block(s) within a NR carrier’s guard-band

- NB-IoT operating in NR in-band: NB-IoT is operating when it utilizes the resource block(s) within a normal NR carrier.

<End of TP>