3GPP TSG-RAN WG4 Meeting #92						R4-1909219

Ljubljana, Slovenia, 26th – 30th August 2019

Source:	Sony, Ericsson

Title:	Beam Correspondence, SNR versus RSRP

Agenda item:	9.13.2

Document for:	Approval

- Background

Beam correspondence was further discussed in RAN4 #91 in Reno and most of the specification items were agreed upon [1], [2]. The analysis supporting the specification is described in [3] where simulations, performed by several contributing companies, are based on imperfect estimation of RSRP values and  pre-coders (beam former),  agreed in RAN4 #91 BIS [4], [5]. To complete a comprehensive test specification, the side condition which includes the interplay among RSRP error, number of resource elements (REs), and SNR need to be understood.

- Problem description

The general problem of estimation of the best (or suitable) UL beam based on UE measuring on DL synchronization signals is described in [6]. The reasoning in [6] (and further elaborated in [7]) is based on an RSRP estimation error (log-normal distribution μ= 0,   = 2 dB). During the discussion in RAN4 #90 bis and RAN4 #91 the question was raised how this RSRP error correlates to (UE) SNR. Elaboration on available SNR in the test chamber was done in [8] where it was concluded that: “The SNR conditions for the definition of the beam correspondence tolerance requirement are in the range of -6.0 dB to 19.4 dB”. We will look further into the relation between RSRP error and SNR and look at the dependency of number of available DL synchronization resource elements (REs).

- Simulation set-up

In our previous paper ([6], [7]) we have made analysis of the variation of the RSRP error in the test chamber taking antenna pattern into account. Remaining simulation is to correlate the RSRP error to an SNR. We have therefore simulated a single-antenna test probe communicating with a single-antenna device-under-test (DUT) through a pure line-of-sight (LOS) channel. For this analysis the DUT antenna is ideal and isotropic. Only one polarization mode is considered at both the test probe and the DUT. During the simulations, the receiver noise power is held at a constant level and the transmitted power is adjusted to match the target SNR level. Then, the RSRP is estimated by non-coherently averaging the estimated channel energy over Nref reference REs. That is,

RSRP=1Nrefn=1Nref|​hnxn+wn2  ,

where {hn}n=1Nref are radio channel samples, {xn}n=1Nref are transmitted symbols fulfilling xn=constant for n=1,…,Nref (i.e., QPSK symbols), and {wn}n=1Nref is the receiver noise. Plots below are based on RSRP estimates in decibels:

RSRPdB=10log10(RSRP).

## Test cases

We have simulated the test cases specified in the table below. It has been assumed that one resource element (RE) is available per resource block (RB) for reference signals. Thus, Nref=NRB. Further, hn=constant for all simulations and all n, and its phase is IID across simulations.

| Environment | Channel Profile | UE testing condition |
| --- | --- | --- |
| Free space | LOS | Isotropic DUT antenna. |

Other test case parameters are as follow:

| Parameter | Value | Units |
| --- | --- | --- |
| Number of simulations | 10000 | - |
| Number of rotation steps in azimuth | 1 | - |
| Number of resource blocks | 2, 5, 10, 20, 50, 100, or 400 | - |
| Number of reference signals per resource block | 1 | - |
| Number of samples for (non-coherent) averaging | 2, 5, 10, 20, 50, 100, or 400 | - |
| Subcarrier separation | 120 | KHz |
| Bandwidth | 2.88, 7.2, 14.4 , 28.8, 72, 144, or 576 | MHz |
| SNR | -10,-9,…,19,20 | dB |
| Number of UE beams | 1 | - |
| UE beams type | Single-polarized, isotropic | - |
| Number of BS beams | 1 | - |
| UE beams type | Single-polarized, simulated | - |
| Carrier frequency | 28 | GHz |
| Normalized Path Gains | False | - |

# Simulation Results

## RSRP estimation error

Figure 1 shows the standard deviation of the non-coherent RSRP estimator for different SNR target levels, and for different number of available reference signals, Nref. It could be seen that the standard deviation of the estimator depends on the SNR level and the number of reference REs (Nref). Except for very small values of SNR and Nref, the standard deviation appears to be small. It is evident that a discussion of the required SNR for a BC test must also involve discussing the number of reference REs (Nref).

Observation 1	The RSRP estimation accuracy is impacted by both SNR and the number of resource elements.

Proposal 1	For BC test, RAN4 shall discuss the number of reference REs in conjunction with SNR.

Figure 1. Simulation on RSRP standard deviation value based on SNR.

# Conclusion

In this contribution we have discussed the relation between SNR, number of reference REs and RSRP estimation error for beam correspondence. We have made the following observations and proposals:

Observation 1	The RSRP estimation accuracy is impacted by both SNR and the number of resource elements.

Proposal 1	For BC test, RAN4 shall discuss the number of reference REs in conjunction with SNR.

# References

- R4-1907610 	“Draft CR to TR38.810 on beam correspondence”, Samsung, Apple

- R4-1907611 	“Draft CR to TS38.101-2 on beam correspondence”, Samsung, Apple, Verizon

- R4-1907613 	“CR to 38.817-01 to capture outcome of beam correspondence”, Apple Inc

- R4-1905186 	“Beam correspondence ad-hoc meeting minutes”, Samsung

- R4-1905205 	“Second ad-hoc MoM for beam correspondence”, Samsung

- R4-1904237	“Beam Correspondence, remaining X and Y”, Sony, Ericsson

- R4-1906330	“Beam Correspondence, remaining X and Y”, Sony, Ericsson

- R4-1907612	“Remaining issues with the beam correspondence requirement”, Apple Inc.