3GPP TSG-RAN WG4 Meeting # 92	R4-1909357

# Ljubljana, SI,  26th  - 30th Aug, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | <CR#> | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-3 on UE maximum output power for DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The maximum output power for inter-band EN-DC (two bands) in Table 6.2B.1.3-1 for DC_41C_n78A and DC_41C_n79A are incorrect. - Some other editorial corrections are pointed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the EN-DC configurations of DC_41C_n78A and DC_41C_n79A for maximum output power in Table 6.2B.1.3-1. - Some other typos have been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The above mentioned problems will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3.1.0, 6.2B.3.1.1, 6.2B.3.1.2.0, 6.2B.3.2.0, 6.2B.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

<< Unchanged sections omitted >>

6.2B.1	UE maximum output power for DC

6.2B.1.1	Intra-band contiguous EN-DC

The following UE Power Classes define the total maximum output power for any transmission bandwidth(s) of the CG(s) configured.

The maximum output power is measured as the total maximum output power across the UE antenna connector(s). The period of measurement shall be at least one sub frame.

Table 6.2B.1.1-1: Maximum output power for EN-DC (continuous sub-blocks)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_(n)71AA |  |  | 23 | +2/-3 |
| DC_(n)41AA | 26 | +2/-21 | 23 | +2/-21 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power Class 3 is the default power class unless otherwise stated. |  |  |  |  |

If UE supports a different power class than the default UE power class for EN-DC band combination, and the supported power class enables higher maximum output power than that of the default power class:

-	if the E-UTRA UL/DL configuration is 0 or 6; or

-	if the E-UTRA UL/DL configuration is 1 and special subframe configuration is 0 or 5; or

-	if the IE p-maxUE-FR1-r15 as defined in TS 36.331 [4] is provided and set to the maximum output power of the default power class or lower;

-	apply all requirements for the default power class, and set the configured transmitted power as specified in subclause 6.2B.4;

-	else

-	apply all requirements for the supported power class, and set the configured transmitted power class as specified in subclause 6.2B.4;

6.2B.1.2	Intra-band non-contiguous EN-DC

Table 6.2B.1.2-1: Maximum output power for EN-DC (non-continuous sub-blocks)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_3A_n3A2 |  |  | 23 | +2/-3 |
| DC_41A_n41A | 26 | +2/-21 | 23 | +2/-21 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Only single switched UL is supported in Rel.15 NOTE 3: Power Class 3 is the default power class unless otherwise stated. |  |  |  |  |

If UE supports a different power class than the default UE power class for EN-DC band combination, and the supported power class enables higher maximum output power than that of the default power class:

-	if the E-UTRA UL/DL configuration is 0 or 6; or

-	if the E-UTRA UL/DL configuration is 1 and special subframe configuration is 0 or 5; or

-	if the IE p-maxUE-FR1-r15 as defined in TS 36.331 [4] is provided and set to the maximum output power of the default power class or lower;

-	apply all requirements for the default power class, and set the configured transmitted power as specified in subclause 6.2B.4;

-	else

-	apply all requirements for the supported power class, and set the configured transmitted power class as specified in subclause 6.2B.4;

6.2B.1.3	Inter-band EN-DC within FR1

For inter-band EN-DC of E-UTRA and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_1A_n28A | 23 | +2/-3 |
| DC_1A_n40A | 23 | +2/-3 |
| DC_1A_n51A | 23 | +2/-3 |
| DC_1A_n77A | 23 | +2/-3 |
| DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A DC_1A_n84A_ULSUP-FDM_n78A | 23 | +2/-3 |
| DC_1A_n79A | 23 | +2/-3 |
| DC_2A_n5A | 23 | +2/-31 |
| DC_2A_n66A | 23 | +2/-31 |
| DC_2A_n71A | 23 | +2/-3 |
| DC_2A_n78A | 23 | +2/-3 |
| DC_3A_n7A | 23 | +2/-31 |
| DC_3A_n28A | 23 | +2/-31 |
| DC_3A_n40A | 23 | +2/-31 |
| DC_3A_n51A | 23 | +2/-31 |
| DC_3A_n77A | 23 | +2/-31 |
| DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A, DC_3A_n80A_ULSUP-FDM_n78A | 23 | +2/-31 |
| DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A, DC_3A_n80A_ULSUP-FDM_n79A | 23 | +2/-31 |
| DC_3A_n82A | 23 | +2/-31 |
| DC_5A_n40A | 23 | +2/-31 |
| DC_5A_n66A | 23 | +2/-31 |
| DC_5A_n78A | 23 | +2/-3 |
| DC_7A_n28A | 23 | +2/-31 |
| DC_7A_n51A | 23 | +2/-31 |
| DC_7A_n78A DC_7C_n78A | 23 | +2/-3 |
| DC_8A_n40A | 23 | +2/-31 |
| DC_8A_n77A | 23 | +2/-3 |
| DC_8A_n78A DC_8A_n81A_ULSUP-TDM_n78A, DC_8A_n81A_ULSUP-FDM_n78A | 23 | +2/-3 |
| DC_8A_n79A DC_8A_n81A_ULSUP-TDM_n79A, DC_8A_n81A_ULSUP-FDM_n79A | 23 | +2/-3 |
| DC_11A_n77A | 23 | +2/-3 |
| DC_11A_n78A | 23 | +2/-3 |
| DC_11A_n79A | 23 | +2/-3 |
| DC_12A_n5A | 23 | +2/-3 |
| DC_12A_n66A | 23 | +2/-3 |
| DC_18A_n77A | 23 | +2/-3 |
| DC_18A_n78A | 23 | +2/-3 |
| DC_18A_n79A | 23 | +2/-3 |
| DC_19A_n77A | 23 | +2/-3 |
| DC_19A_n78A | 23 | +2/-3 |
| DC_19A_n79A | 23 | +2/-3 |
| DC_20A_n8A | 23 | +2/-3 |
| DC_20A_n28A DC_20A_n83A | 23 | +2/-3 |
| DC_20A_n51A | 23 | +2/-3 |
| DC_20A_n77A | 23 | +2/-3 |
| DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A, DC_20A_n82A_ULSUP-FDM_n78A | 23 | +2/-3 |
| DC_21A_n77A | 23 | +2/-3 |
| DC_21A_n78A | 23 | +2/-3 |
| DC_21A_n79A | 23 | +2/-3 |
| DC_25A_n41A | 23 | +2/-3 |
| DC_26A_n41A | 23 | +2/-3 |
| DC_26A_n77A | 23 | +2/-3 |
| DC_26A_n78A | 23 | +2/-3 |
| DC_26A_n79A | 23 | +2/-3 |
| DC_28A n51A | 23 | +2/-3 |
| DC_28A_n77A | 23 | +2/-3 |
| DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A, DC_28A_n83A_ULSUP-FDM_n78A | 23 | +2/-3 |
| DC_28A_n79A | 23 | +2/-3 |
| DC_30A_n5A | 23 | +2/-3 |
| DC_30A_n66A | 23 | +2/-3 |
| DC_38A_n78A | N/A | N/A |
| DC_39A_n78A | 23 | +2/-31 |
| DC_39A_n79A | 23 | +2/-31 |
| DC_40A_n77A | N/A | N/A |
| DC_41A_n77A DC_41C_n77A | 23 | +2/-31 |
| DC_41A_n78A DC_41C_n78A | 23 | +2/-31 |
| DC_41A_n79A DC_41C_n79A | 23 | +2/-31 |
| DC_42A_n51A | 23 | +2/-3 |
| DC_42A_n77A | N/A | N/A |
| DC_42A_n78A | N/A | N/A |
| DC_42A_n79A | N/A | N/A |
| DC_66A_n5A | 23 | +2/-31 |
| DC_66A_n71A | 23 | +2/-3 |
| DC_66A_n78A, DC_66A_n86A_ULSUP-TDM_n78A, DC_66A_n86A_ULSUP-FDM_n78A | 23 | +2/-3 |
| NOTE 1: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. |  |  |

6.2B.1.3a	Inter-band NE-DC within FR1

For inter-band NE-DC of E-UTRA and NR in FR1, the following UE power classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3a-1: Maximum output power for inter-band NE-DC (two bands)

| NE-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n1A_28A | 23 | +2/-3 |

6.2B.1.4	Inter-band EN-DC including FR2

UE maximum output power requirement for E-UTRA single carrier and CA operation specified in subclauses 6.2.2 and 6.2.2A of [4] and for NR single carrier and CA operation specified in subclause 6.2.1 and 6.2A.1 of [3] apply.

6.2B.1.5	Inter-band EN-DC including both FR1 and FR2

UE maximum output power requirement for E-UTRA single carrier and CA operation specified in subclauses 6.2.2 and 6.2.2A of [4] and for NR single carrier and CA operation specified in subclause 6.2.1 of [2] and subclause 6.2.1 and 6.2A.1 of [3] apply.

==============================================================

<< End of changes >>