3GPP TSG-RAN WG4 Meeting #92	R4-1909610

# Ljubljana, Slovenia, Aug 26th - 30th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UE one-shot timing adjustment requirements (section 7.1.2.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | R15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The threshold of H used in UE one-shot timing adjustment requirements is still TBD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. To define the value of H. 2. To clairfy when UE need to perform one-shot timing adjustment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE one-shot timing adjustment requirements are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.1.2.2	One shot timing adjustment

When UE is indicated to switch active TCI state and the magnitude of the timing difference T between old TCI state and new TCI state exceeds H, then the UE shall adjust its transmission timing in one adjustment only once provided that the following conditions are met at the UE. Otherwise, when the the magnitude of the T ≤ H, then the UE shall adjust its transmission timing according to the rules defined in clause 7.1.2.1.

-	SSB_RP and SSB Ês/Iot according to Annex B.2.6.1 for a corresponding operating Band,

The UE transmit timing immediately after UE applying the new active TCI stateshall be: T2-(NTA+NTA offset)+2(T1-T2). After applying the one shot timing adjustment the UE shall adjust its transmission timing according to the rules defined in clause 7.1.2.1.

Where: T= T1-T2

-	T1 is the reception time at the UE just before active TCI state switch occurs,

-	T2 is the reception time to be used at the UE just after active TCI state switch occurs,

-	H is defined in table 7.1.2.2-1.

Table 7.1.2.2-1: The value of H

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | H |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 72*64*Tc |
|  |  | 30 | 36*64*Tc |
|  |  | 60 | 18*64*Tc |
|  | 30 | 15 | 72*64*Tc |
|  |  | 30 | 36*64*Tc |
|  |  | 60 | 18*64*Tc |
| 2 | 120 | 60 | 18*64*Tc |
|  |  | 120 | 9*64*Tc |
|  | 240 | 60 | 18*64*Tc |
|  |  | 120 | 9*64*Tc |

<End of Change 1>