3GPP TSG-RAN WG4 Meeting #92	R4-1910164

# Ljubljana, Slovenia,  26 August, 2019 - 31 August, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | 1 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of OTA parameters to SS-RSRP accuracy test in section A.5.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Perf |  |  |  |  |  | Date: |  |  | 2019/29/8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | All OTA parameters are still TBD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify PCI to be used in the test to avoid potential cross correlation issues Add OTA settings for subtests for highest level testing phase targetting Io approximately -50dBm at reference point and approximately 5dB lower in T3 (corresponding to minimum level that can be tested with AWGN considering worst case UE noise floor). Remove subtest 2 Add assumed minimum and maximum antenna gain for RSRP absolute accuracy requirement Renumber subtest 3 as subtest 2 and specify that in this subtest there is no AWGN added in the test equipment Add an editors note “Editor’s Note : Addition of a high SNR phase during the testing is under consideration. The relative accuracy of cell 2 in high SNR conditions compared with cell 2 in lower SNR conditions and the relative accuracy of cell 3 in high SNR conditions compared with cell 3 in lower SNR conditions would be verified. The angle of arrival of cell 2 and cell 3 should not change when measurements in high SNR conditions are compared with measurements in lower SNR conditions.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OTA parameters in intrafrequency RSRP accuracy tests remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

Change 1

### A.5.7.1	SS-RSRP

#### A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

##### A.5.7.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 10.1.2.1.1and 10.1.2.1.1 for intra-frequency measurements.

##### A.5.7.1.1.2	Test parameters

In this set of test cases all NR cells are on the same carrier frequency. Supported test configurations are shown in table A.5.7.1.1.2-1. Both absolute and relative accuracy of SS-RSRP intra frequency measurements are tested by using the parameters in Table A.5.7.1.1.2-2 and A.5.7.1.1.2-3. The E-UTRA PCell is configured as specified in section A.3.7.2.2. In all test cases, Cell 1 is the PCell, cell 2 is the PSCell and Cell 3 the target cell.

Table A.5.7.1.1.2-1: SS-RSRP Intra frequency SS-RSRP supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FDD LTE PCell, Cell 2&3 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | TDD LTE PCell, Cell 2&3 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

Table A.5.7.1.1.2-2: SS-RSRP Intra frequency general test parameters

| ParameterNote 5 | Unit | Test 1 |  |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 3 |  |  | Cell 2 | Cell 3 |
| Physical cell ID |  | 489 | 0 |  |  | 489 | 0 |
| SSB ARFCN |  | freq1 |  |  |  | freq1 |  |
| Duplex mode |  | TDD |  |  |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  |  |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |  |  | 100: NRB,c = 66 |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - |  |  | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - |  |  | CR.3.1 TDD | - |
| Dedicated CORESET Reference Channel |  | CCR.3.1 TDD | - |  |  | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 |  |  | OP.1 | OP.1 |
| SSB configuration |  | SSB.1 FR2 | SSB.1 FR2 |  |  | SSB.1 FR2 | SSB.1 FR2 |
| SMTC configuration |  | SMTC.1 | SMTC.1 |  |  | SMTC.1 | SMTC.1 |
| Time offset with Cell 2 | s | - | 3 |  |  | - | 3 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 |  |  | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |  |  | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
|  | dB | 2.84 | -0.35 |  |  | Note 6 | Note 6 |
| Propagation conditions |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: All parameters apply for configuration 1 and 2 Note 6: No noise is added in this phase of the test |  |  |  |  |  |  |  |

Table A.5.7.1.1.2-3: SS-RSRP  Intra frequency OTA related test parameters

| ParameterNote 6 |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 2 | Cell 3 |  |
| Angle of arrival configuration |  |  |  | According to section A.3.8.15.2.2 |  | According to section A.3.15.2.2 |  |
| Note1 |  | NR_TDD_FR2_A | dBm/15kHzNote4 | -94.03 |  | Note 7 |  |
|  |  | NR_TDD_FR2_B |  |  |  | Note 7 |  |
|  |  | NR_TDD_FR2_F |  |  |  | Note 7 |  |
|  |  | NR_TDD_FR2_G |  |  |  | Note 7 |  |
|  |  | NR_TDD_FR2_T |  |  |  | Note 7 |  |
|  |  | NR_TDD_FR2_Y |  |  |  | Note 7 |  |
| Note1 |  | NR_TDD_FR2_A | dBm/SCSNote3 | -85 |  | Note 7 |  |
|  |  | NR_TDD_FR2_B |  |  |  | Note 7 |  |
|  |  | NR_TDD_FR2_F |  |  |  | Note 7 |  |
|  |  | NR_TDD_FR2_G |  |  |  | Note 7 |  |
|  |  | NR_TDD_FR2_T |  |  |  | Note 7 |  |
|  |  | NR_TDD_FR2_Y |  |  |  | Note 7 |  |
|  | dB | 2.85 | -0.35 |  | N/A |  |  |
| Es | NR_TDD_FR2_A | dBm/SCSNote4 | N/A | (Table B.2.2-2 spherical coverage +3.1dB) | (Table B.2.2-2 spherical coverage +3.1dB) | (Table B.2.2-2 spherical coverage +3.1dB) |  |
|  | NR_TDD_FR2_B |  |  |  |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |  |  |  |
|  | NR_TDD_FR2_T |  |  |  |  |  |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  |  |
| SSB_RP | NR_TDD_FR2_A | dBm/SCS | -82.15 | -85.35 | (Table B.2.2-2 spherical coverage +3.1dB) | (Table B.2.2-2 spherical coverage +3.1dB) |  |
|  | NR_TDD_FR2_B |  |  |  |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |  |  |  |
|  | NR_TDD_FR2_T |  |  |  |  |  |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  |  |
| Note 9 |  |  | dB | 0 | -5 | -5.98 | -5.98 |  |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz Note4 | -50.16 |  | (Table B.2.2-2 spherical coverage +35.1dB) |  |  |
|  | NR_TDD_FR2_B |  |  |  |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |  |  |  |
|  | NR_TDD_FR2_T |  |  |  |  |  |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: All parameters apply for configuration 1 and 2 Note 7: No noise is added in this phase of the test and both cells are at equal power Note 8: SSB_RP is applied at a level 1dB above the minimum level specified in table Table B.2.2-2 for spherical coverage Note 9: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 2dB for UE multi-band relaxation factor ∑MBs from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

Editor’s Note : Addition of a high SNR phase during the testing is under consideration. The relative accuracy of cell 2 in high SNR conditions compared with cell 2 in lower SNR conditions and the relative accuracy of cell 3 in high SNR conditions compared with cell 3 in lower SNR conditions would be verified. The angle of arrival of cell 2 and cell 3 should not change when measurements in high SNR conditions are compared with measurements in lower SNR conditions.

##### A.5.7.1.1.3	Test Requirements

The SS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in sections 10.1.3.1.1 and relative accuracy requirements in section 10.1.3.1.2. The following requirements are to be verified

The absolute accuracy of cell 2 and cell 3 shall be verified in test 1 and test 2. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in table A.5.7.1.1.3-1.

The relative accuracy of cell 2 compared with cell 3 shall be verified in test 1 and test 2. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 10.1.3.1.2-1

Editor’s Note : Addition of a high SNR phase during the testing is under consideration. The relative accuracy of cell 2 in high SNR conditions compared with cell 2 in lower SNR conditions and the relative accuracy of cell 3 in high SNR conditions compared with cell 3 in lower SNR conditions would be verified. The angle of arrival of cell 2 and cell 3 should not change when measurements in high SNR conditions are compared with measurements in lower SNR conditions.

Table A.5.7.1.1.3-1: SS-RSRP  absolute accuracy test requirement for different UE power classes

| UE power class | Test requirement Note1.2 |
| --- | --- |
| 1 | SSB_RP-TBD-δ-≤Reported RSRP(dB)≤SSB_RP+TBD+ δ dB |
| 2 | SSB_RP-TBD-δ-≤Reported RSRP(dB)≤SSB_RP+TBD+ δ dB |
| 3 | SSB_RP-22.6-δ-≤Reported RSRP(dB)≤SSB_RP+[20]+ δ dB |
| 4 | SSB_RP-TBD-δ-≤Reported RSRP(dB)≤SSB_RP+TBD+ δ dB |
| Note 1: SSB_RP is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the cell under consideration Note 2: δ is the RSRP absolute accuracy requirement from table 10.1.3.1.1-1, e.g. if the requirement corresponding to the Io used in the test is ±6dB, δ=6 and if the requirement corresponding to the Io used in the test is ±8dB, δ=8 |  |